3GPP TSG-SA3 Meeting #104-e 	S3-212946

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	Lenovo, Motorola Mobility

- Title:	Solution to address KI#1

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This pCR provides a new solution to address Key Issue #1 in TR 33.864.

# 2	References

[1]	3GPP TR 33.864 Study on the security of Access and Mobility Management Function (AMF) re-allocation; Rel-17.

# 3	Rationale

This pCR proposes a new solution to address Key Issue #1.

# 4	Detailed proposal

SA3 is kindly requested to agree the pCR below to TR 33.864.

*****Start of Change*****

## 6.X	Solution #X: AMF re-allocation and secured reroute via RAN enabled by AUSF

### 6.X.1	Introduction

This solution addresses Key Issue #1: "Security of AMF re-allocation procedures".

In this solution the 5G NAS security context is protected before being re-routed via RAN together with the Registration Request (RR) message using a reroute security context provided by the AUSF. The provisioning of the reroute security context to the reallocated AMF (to enable decryption of protected reroute NAS message and handling of registration request) is carried out with a sufficient home control. The protection utilizes the AUSF as the trusted NF by both the initial and the target AMF.

### 6.X.2	Solution details

The initial AMF protects the security context or the potentially horizontally derived security context with a reroute security context (i.e., interity key and an encryption key) generated and provided by the AUSF. Then the initial AMF sends the Registration Request (RR), a protected 5G NAS security context container (which includes the security context along with the keyAmfHDerivationInd indicator if needed and potentially UL/DL NAS COUNTs) and set of clear text information (i.e., target AMF authentication information and other information) to the target AMF via RAN.

Figure 6.X.2-1: AMF re-allocation with NAS message and 5G NAS security context re-route via RAN

Figure 6.X.2-1 shows the solution steps:

1.	The UE sends a Registration Request message including a SUCI or 5G-GUTI. If the UE has a 5G NAS security context (Registration with 5G-GUTI) it includes a protected NAS container in the Registration Request message.

2.	Steps 2-6b of TS 23.502 [2], clause 4.2.2.2.3 are followed.

3.	The initial AMF based on local policy and subscription information, decides to reroute the RR message to a target AMF via RAN.The initial AMF/SEAF sends a Security context request message to request reroute security context from the AUSF for the purpose of AMF re-allocation by sending a related indication, target AMF information (i.e., AMF set ID or AMF address(es)), SUPI and privacy protected UE ID (i.e., SUCI/5G-GUTI) as received in the registration request.

NOTE 1:	The details of reroute security context generation at the AUSF can be specified in the normative phase.

4.	The AUSF generates the reroute security context (i.e., an integrity key and encryption key) from the AUSF key and using one or more of the inputs provided in Step 3. Further AUSF generates an authentication parameter (NAS_Sec_ID to enable authentication of the target AMF) using hash of the reroute security context, target AMF information and SUPI received in step 3. The AUSF stores the reroute security context, target AMF information, UE ID and authentication parameter along with the SUPI. The authentication parameter NAS_Sec_ID can also be used to identify the reroute security context generated and stored at the AUSF.

5.	The AUSF responds with the reroute security context and an authentictaion parameter NAS_Sec ID.

NOTE 2:	The NAS security context generation (e.g., horizontal key derivation) at the initial AMF can follow the method described in Solution 9 and 10 (to be shared in a protected NAS container to the Target AMF via RAN). Similary Steps 12-16 can be followed similar to Solution 9 and 10.

6 .	The initial AMF optionally performs horizontal Kamf derivation of Kamf-0 to generate a new Kamf-1. This step would ensure that the target AMF has no access to the Kamf-0 key used by the initial AMF. If the Initial AMF performs horizontal Kamf derivation, then the initial AMF resets the corresponding uplink and downlink NAS COUNTs.

7.	The initial AMF encrypts the security context (including Kamf-0 or Kamf-1), the keyAmfHDerivationInd indicator (if performed) and potentially other parameters (e.g., UL/DL NAS COUNTs if horizontal key derivation was performed in Step 6) with the reroute security context (i.e., encryption key) and creates a protected 5G NAS security context container. The initial AMF further applies integrity protection (i.e., using the reoute integrity key) over the encrypted NAS container, complete registration request and other clear text information (i.e., Authentication parameter, Target AMF information, NAS Security Algorithm IDs, routing information (which contains for e.g., address/FQDN/AUSF identification information). The Initial AMF indicates the algorithm information in the Steps 8-9 used for the protection of 5G security context container.

8.	The initial AMF forwards the complete Registration Request message, the protected 5G NAS security context container, the clear text information (i.e., such as authentication parameter NAS_Sec_ID, routing information, NAS security algorithm IDs, and Target AMF Information) to the RAN.

9.	The RAN forwards the complete Registration Request message, the protected 5G NAS security context container, the clear text information to the target AMF.

10a-c.	The target AMF/SEAF sends a Key request message to the AUSF by providing its own AMF information (AMF Set ID and address), Reroute key indication, the authentication parameter NAS_Sec ID and UE ID (i.e., 5G-GUTI/SUCI as received in the registration request). The AUSF verifies authentication parameter NAS_Sec ID, the AMF information (with the locally stored information) and other information. If the verification is successful, the AUSF returns SUPI and the reroute security context to the target AMF and deletes the locally stored reroute security context and associated information.

11.	 The target AMF verifies the integrity and confidentiality of the protected NAS container.

12.		After decrypting the security context,

if SUCI is included in the Registration Request, the target AMF skips step 12 (as no additional information about established PDU sessions etc. is stored in the old AMF).

If a 5G-GUTI is included in the Registration Request and the target AMF has received a 5G NAS security context and potentially a keyAmfHDerivationInd indicator, then:

-	If there is no connectivity between the target AMF and old AMF (cases 2.a.ii and 2.b.ii in clause 4.3), the target AMF skips step 12 (as any additional information about established PDU sessions etc. stored in the old AMF cannot be retrieved by the target AMF).

- 	If there is connectivity between the target AMF and the old AMF (cases 2.a.i and 2.b.i in clause 4.3), the target AMF can fetch any additional information about established PDU sessions etc. stored in the old AMF.

13.	If the target AMF has received the keyAmfHDerivationInd indicator, then the target AMF shall run a NAS SMC procedure with the UE, to take the new Kamf-1 key into use with the UE.

14.	The target AMF needs to initiate a new primary authentication with the UE to generate a new Kamf-2. The new primary authentication procedure is protected by the Kamf-1. This step would ensure that the initial AMF has no access to the new Kamf-2 key generated between target AMF and the UE.

The target AMF determines that a NAS re-route via RAN has taken place and the target AMF shall use the Kamf-1 only for the purpose of sending protected NAS Security Mode Command and Authentication Challenge/Request to the UE, and for receiving protected NAS Security Mode Complete and Authentication Response from the UE.

15-16.	The target AMF needs to run a new NAS SMC procedure with the UE to take the new Kamf-2 into use with the UE. The target AMF needs to include the request to the UE to include the complete Registration Request message in the NAS Security Mode Complete message by setting the flag "request initial NAS flag" in the NAS Security Mode Command message.  The UE includes the complete Registration Request message (sent in step 1) in the NAS Security Mode Complete message to the target AMF. This means that the target AMF can take the Registration Request message received in NAS Security Mode Complete message into use and drop the Registration Request message rerouted via RAN.

NOTE 3:	If the operators consider to reroute protected NAS security context via RAN and further if operators prefer to use NSSF as a common NF to perform the new security functionality for controlling the reroute security context provisioning, then the features described in this solution for AUSF can be applied to the NSSF with two minor adaptations (i.e., (i) the SUPI of the UE need to be re-routed from the initial AMF to the target AMF via protected NAS container through RAN. (ii) The NSSF need to be preprovisioned with a root key in an implementation specific way to support reroute security context generation.). But the usage of NSSF for reroute security context provisioning will not ensure sufficient home control as designed in the current 5G system while providing serving network security keys. Further if it is required to reroute the Registration request received in the NAS SM complete message from the initial AMF to Target AMF via RAN, then it can also be protected similar to the Reroted NAS security context.

### 6.X.3	Evaluation

In this solution the 5G NAS security context is protected before being re-routed via RAN together with the Registration Request (RR) message. The protection utilizes the AUSF as a trusted NF by both the initial and the target AMF/SEAF. The AUSF involvement ensures the home control in validating the reallocated AMF and provisioning of reroute security context to handle the rerouted registration request. The initial AMF sends the security context to the target AMF encrypted so that that the AMF key is not exposed to the RAN node. Although that RAN node has access to limited parameters to attempt recreation of the decryption key of the protected 5G NAS security context container, the RAN node does not have access to the AUSF key which is the actual root of the reroute security context. Further, the RAN node cannot directly request the decryption key since it does not have a direct SBA interface to the AUSF. The RAN node is allowed to connect to the core network only via the NGAP protocol specified in TS 38.413 [5]. This solution needs home control for Registration with AMF re-allocation’

In this solution there is one more optional NAS SMC performed by the target AMF after the Registration Request message and the protected 5G NAS security context container including the AMF key (Kamf-0 or Kamf-1) has been re-routed via RAN. This optional NAS SMC is performed in order to take the new Kamf-1 key into use by the UE and target AMF (after the optional horizontal Kamf derivation of Kamf-0 to generate a new Kamf-1 in the initial AMF).  The optional horizontal Kamf derivation in the initial AMF provides backward security so the target AMF has no access to the Kamf-0 and its corresponding NAS key used between the UE and the initial AMF. The initial AMF has access of the new Kamf-1 key re-routed via RAN to the target AMF, but after the target AMF has taken the new Kamf-1 key into use with the UE by running a NAS SMC, the target AMF can initiate a new protected Authentication procedure with the UE in order to generate a new Kamf-2 key shared with the UE, which the initial AMF has no access to. By running a new NAS SMC procedure between target AMF and UE to take the new Kamf-2 key into use, forward security is provided.

This solution has the following impact:

UE: 	No UE impact.

AMF:	This initial AMF may need to perform horizontal Kamf derivation of the Kamf before forwarding the protected 5G NAS security context container together with the complete Registration Request message on the N2 interface to the RAN.

-	The initial AMF needs to request for reroute security context from the AUSF and process the AUSF response.

-	The initial AMF needs to perform confidentiality and integrity protection of the 5G NAS security context and  potentially other information.

-	The target AMF needs to verify the integrity and decrypt the protected 5G NAS security context container and other information.

-	The target AMF may need to perform a NAS SMC procedure to take any potentially horizontally derived Kamf key into use before initiating a primary authentication

-	The target AMF also needs to perform an authentication request in order to produce its own security context.

AUSF:		The AUSF needs a new service to generate rereoute security keys and authentication paramater (NAS_Sec_ID) for the protection of the reroute 5G NAS security container of the initial AMF and provide the keys to the target AMF.

RAN: 	This solution has impact on RAN and N2 interface. The REROUTE NAS REQUEST message is defined in TS 38.413 [4] and the initial AMF includes the INITIAL UE MESSAGE into the REROUTE NAS REQUEST message to RAN. The REROUTE NAS REQUEST message needs to be updated to include the protected 5G NAS security context container and potentially other parameters. Also the RAN needs to forward the protected 5G NAS security context container and potentially other parameters to the target AMF together with the INITIAL UE MESSAGE.

*****End of Change*****