3GPP TSG-SA3 Meeting #104-e 	draft_S3-212983-r1

- e-meeting, 16 - 27 August 2021											Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Evaluation to the Solution 10

- Document for:	Approval, Information, Discussion

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to approve this pCR to introduce evaluation to the solution 10

# 2	References

[1]	33.866

# 3	Rationale

Evaluation added to the solution which provides Authorization of NF Service Consumers for data access via DCCF.

# 4	Detailed proposal

************** START OF CHANGES

### 6.10.3	Evaluation

The solution proposes authorization mechanisms in order to authorize a data consumer to request a service of data producer, and to authorize DCCF and MFAF to request service on behalf of the data consumer.

The solution reuses existing mechanisms of CCA and OAuth 2.0 as defined in TS 33.501, with  the enhancements in the access token request sent by the NF Service Consumer and the claims generated by the NRF, and provides protection against the following threats :-

- Based on a request from a DCCF, the Messaging Framework may provide data from a producer to a requesting data consumer, even though the consumer is not authorized to receive this data.

- A DCCF could subscribe for data from the data source on behalf of the data consumer without the data consumer authorizing DCCF to do so.

- The data producer may be unable to correctly verify the identity of the data consumer since the data request is coming from DCCF on behalf of the consumer.

The solution fulfills the following requirements of the KI 1.3

- The data consumer shall be authorized to access the data from the data source.

- Authorization of the DCCF shall be supported to access a service of a data source on behalf of a data consumer.

************** END OF CHANGES