3GPP TSG-SA3 Meeting #104-e  	draft_S3-212985-r1

- e-meeting, 16 – 27 August 2021													Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Enhancement to the solution on Detection of anomalous NF behaviour by NWDAF

- Document for:	Approval, Information, Discussion

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to approve this pCR to enhance the solution on Detection of anomalous NF behaviour by NWDAF

# 2	References

[1] 	3GPP TR 33.866: “Study on security aspects of enablers for Network Automation (eNA) for the 5G system (5GS) Phase 2”

# 3	Rationale

This document provides an enhacement to the solution on Detection of anomalous NF behaviour by NWDAF addressing key issue 2.2 to propose an alternate procedure to provide security specific NF logs to NWDAF by OAM for analysis.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.866.

********** START OF CHANGES

## 6.7	Solution#7: Detection of anomalous NF behaviour by NWDAF

### 6.7.1	Introduction

This solution addresses key issue #2.2.

The 5GC supports various NF deployments in order to ensure that the NF can provide services from multiple locations. These cloud platforms may be owned and operated by the PLMN operator himself or they may be run on commercial public cloud platforms. When NFs are distributed across multiple cloud infrastructures, it is possible that the NFs behave in an anomalous manner. Such anomalous behaviour can arise either due to configuration errors or internal data corruption, or due to an attack.

To allow an operator to manage its NF cloud deployment efficiently, additional security relevant log data from the NFs can be provided to the NWDAF, which can then be used to analyse and detect the root cause for an anomaly.

### 6.7.2	Solution details

The OAM or some operator defined AF may subscribe/request to the NWDAF to notify/respond when it detects an abnormal behaviour for a particular NF instance (for example excessive resource usage or consistent failure to provide its service). There can be various causes of abnormal behaviour such as an erroneous operation at a NF, an error at the side of the cloud provider where the NF is deployed, or an impending attack at a particular NF by a malicious entity in the system.

In order to analyse the root cause of the anomaly with certain degree of confidence, more information or related data can help. Thus, NWDAF requests inputs from different entities in the system (such as OAM, NRF and NFs) for detailed analysis. Security related data for analytics can also be collected by NWDAF from the NFs (or via OAM). The final output analytics is then sent to the OAM or the AF for adequate measures.

In order to facilitate the security related log data collection from NFs two different methods are proposed

NOTE: Defining the protocols used for log collection is not in the scope of this document

#### 6.7.2.1	Collection of security related log data of NFs via NFs EventExposure APIs

The procedure depicted in Figure 6.7.2-1 allows a consumer to request analytics from NWDAF for anomalous NF behaviour and its root cause.

Figure 6.7.2-1: NF anomalous behaviour analytics provided by NWDAF using security logs obtained from NFs EventExposure APIs

1.	The OAM or operator's AF sends a request/subscription to the NWDAF for NF anomalous behaviour analytics using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve resource usage and NF resources configuration of all targeted NF instances, following the procedure specified in clause 6.2.3.2 in TS 23.288 [1].

3a.	The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, using Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

3b.	NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

4a.	The NWDAF subscribes or requests the additional security specific log info (as specified in the table 6.7.2-1) for a particular NF by invoking the Nnf_EventExposure_Subscribe service operation.

4b.	The NF then notifies the NWDAF (e.g. with the complete log report) by invoking Nnf_EventExposure_Notify service operation.

5.	The NWDAF derives the relevant analytics using the inputs provided by the OAM, NRF, and the NF (as specified in the table 6.7.2-2)

6.	The NWDAF provides requested NF anomalous information along with the corresponding root cause (for instance a malicious NF trying to attack other NF for denial of service, or an erroneous NF unable to provide service to other NFs) using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

#### 6.7.2.2	Collection of security related log data of NFs via OAM

The procedure depicted in Figure 6.7.2-2 allows a consumer to request analytics from NWDAF for anomalous NF behaviour and its root cause using the security specific NF logs obtained from OAM

Figure 6.7.2-2: NF anomalous behaviour analytics provided by NWDAF using security logs obtained from OAM

1.	The OAM or operator's AF sends a request/subscription to the NWDAF for NF anomalous behaviour analytics using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve resource usage and NF resources configuration of all targeted NF instances, following the procedure specified in clause 6.2.3.2 in TS 23.288 [1]. The NWDAF further sends the request to the OAM to provide the security specific log into (as specified in the table 6.7.2-1).

3. OAM then collects all the requested information along with the security logs from the NFs and provide it to NWDAF.

4a.	The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, using Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

4b.	NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

5.	The NWDAF derives the relevant analytics using the inputs provided by the OAM and NRF (as specified in the table 6.7.2-2)

6.	The NWDAF provides requested NF anomalous information along with the corresponding root cause (for instance a malicious NF trying to attack other NF for denial of service, or an erroneous NF unable to provide service to other NFs) using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

Table 6.7.2-1 Security specific parameters provided by NFs (NF/NRF) to assist in detection

| Information | Description |
| --- | --- |
| Timestamp | A time stamp associated with the service request which was sent by the NF Service Consumer to a NF Service Producer. |
| NF Service Consumer / SCP Identifier | The consumer instance or the SCP which sends the service request along with the access token to the NF Service Producer. |
| NF Service Producer Identifier | The producer instance which receives the requests and which verifies the access token received along with the requests |
| Authorization status of NF Service Consumer | Indicated if a given NF Service Consumer is authorized to receive an access token or not, as provided by NRF. |
| Access Token Authenticity | Information such as, if access token provided is for the service request it is sent, and if it is generated for the NF Service Consumer which is requesting the service. |
| Access Token Validity | Verification result, i.e. whether the access token is valid or invalid. |
| Number of requests to access a service | Number of simultaneous requests received at the NF Service Producer for a particular time window. |
| Requested Service Name | Name of the service for which the requests had been received. |
| Service Response Confirmation | Confirmation whether the NF Service Producer was able to fulfil the service requests or not. |

Table 6.7.2-2 Inputs provided to NWDAF in assisting the detection of anomalous NF

| Information | Source | Description |
| --- | --- | --- |
| Security Log Data | OAM/NF | Additional security relevant log info as described in table 6.7.2-1. |
| NF Load | NRF | The load of specific NF instance(s) recorded in their NF profile as defined per TS 29.510 [2]. |
| NF resource usage | OAM | The usage of assigned virtual resources for specific NF instance(s) (e.g., mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [3] clause 5.7. |

Table 6.7.2-3 and Table 6.7.2.-4 specifies the output analytics from NWDAF

Table 6.7.2-3: Anomalous NF behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception category | Indication if the anomalous behaviour is an attack or geniune error |
| > Exception level | Scalar value indicating the severity of the abnormal behaviour |
| > List of target NF(s) | One or more NFs which are affected due to the anomoulous NF in the system |
| > List of anomolous NF(s) | One or more NFs which are the probable cause of the anomalous activity in the system (either because they are malicious or due to internal errors) |

Table 6.7.2-4: Anomalous NF behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception category | Indication if the anomalous behaviour is an attack or geniune error |
| > Exception level | Scalar value indicating the severity of the abnormal behaviour |
| > List of target NF(s) | One or more NFs which are affected due to the anomoulous NF in the system |
| > List of anomolous NF(s) | One or more NFs which are the probable cause of the anomalous activity in the system (either because they are malicious or due to internal errors) |
| > Confidence | Confidence of this prediction |

Based on the input, an analytics function can monitor and find abnormalities in NF load or NF resource usage, which can result in an alert. Specific security log data can help to understand if this is normal behaviour or could be resulting from a NF that is behaving anomalous.

Thus, when NFs send service requests to other NFs, input data such as the number of service requests sent, the percentage of successful service requests, the percentage of successful access token verification and the serving NF load and resource usage can be used to provide training data for normal service requests reception and load.

Input data such as NF Service Consumer / SCP identifier, the NF Service Producer Identifier and the Requested Service Name can be used to identity the anomalous/erroneous NF in the system.

NOTE: The derivation of output from input depends on the algorithms used or the policy present. How to derive the output from input is up to implementation logic, which is out of scope of 3GPP.

********** END OF CHANGES