3GPP TSG-SA3 Meeting #104-e 	draft_S3-213007-r2

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	Thales

- Title:	pCR to TR 33.846: Assessment of attack risk

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

This contribution proposes some text for assessment of attack risk table

# 2	References

[1]	3GPP TR 33.846 v0.12.0

[2] 	Ravishankar Borgaonkar (published online: July 2019), "New Privacy Threat on 3G, 4G, and Upcoming 5G AKA Protocols", https://eprint.iacr.org/2018/1175.pdf

# 3	Rationale

This contribution proposes some text for the assessment of attack risk table in clause 7.0.1 of 3GPP TR 33.846 [1] in order to identity the Key Issue to address first due to high level of severity.

Key Issues #4.1 relates to the re-use of keystream in AKA protocol enabling an attacker to estimate the least significant bits of SQN, which leads to privacy leakage of the subscribers. Such an attack is already published, e.g. [2].

We consider that the level of severity is high since this published attack [2] may impact the trust in AKA protocol security level in general.

# 4	Detailed proposal

It is proposed that SA3 review and approve the following pseudo-CR to 3GPP TR 33.839 [1].

### 7.0.1	Assessment of attack risk

Editor's Note: This clause should provide for all KIs addressed a clear statement (in a condensed way) on the security threat, i.e. the likelihood of the attack to happen under which conditions, level of severity.

| Key Issues | Security threats | Comments (e.g. likelihood, level of severity) |
| --- | --- | --- |
| Key Issue #2.1: Linkability by distinguishing MAC failure and synchronization failure | Traceability of the user/victim IMSI-probing is when an attacker tries to find out whether the subscriber with this identity is present in a given area. |  |
| Key Issue #2.2: Linkability by SUCI replay | Traceability of the user/victim IMSI-probing is when an attacker tries to find out whether the subscriber with this identity is present in a given area. | 1. The likelihood of this issue is extremely low. 2. The actual identity of the user cannot be revealed by a SUCI so there is no threat to the user as the user can not be identified. |
| Key Issue #2.2: Linkability by generation of different SUCIs | Traceability of the user/victim IMSI-probing is when an attacker tries to find out whether the subscriber with this identity is present in a given area. | This attack only applies to scenarios where the SUCI calculation is to be performed by the ME. |
| Key Issue #2.2: DoS attack | DoS attack on UDM |  |
| Key Issue #3.1: Attack due to expired authentication result in the UDM | N/A |  |
| Key Issue #3.2: SUPI guessing attacks | Privacy threat | This attack only applies to scenarios where the SUCI calculation is to be performed by the ME. |
| Key Issue #4.1: Protection of SQN during AKA re-synchronisations | Privacy leakage of subscribers Keystream re-use in the AKA protocol. Leakage of information on the sequence number in AKA re-synchronisations. Leaked sequence number information possibly provides information on the number of authentications performed. | This attack is based on the re-use of key stream during AKA re-synchronization. |