3GPP TSG-SA3 Meeting #105-e 	draft_S3-213882-r4

- e-meeting, 8 - 19 November 2021

- Source:	KPN, Vodafone

- Title:	pCR to Living document for BEST_5G: resolution of Editor’s notes

- Document for:	Approval

Agenda Item:	4.11

# 1	Decision/action requested

It is requested to approve this update of the living document for TS 33.163 resolving editor’s notes

# 2	References

[1]	S3-213881, Living document for TS 33.163

# 3	Rationale

The editor’s notes in the living document are resolved in the following way.

-	Clause 4.2.3: text is added to explain how the HSE is related to GBA, 5G GBA and AKMA architectures.

-	Clause 4.6.1.1, Step 2 of 5GS: the step is changed so that no longer UE ID is needed to identify whether keys are needed. Instead, the presence of a valid Session ID (part of the EMSDP message structure) is used.

-	Clause 4.6.1.1, Step 4: the procedure no longer assumes that 5G AKA is used. Also, EAP-AKA’ is allowed. The corresponding key hierarchy in Clause 4.6.2.2 (Figure 4.6.2.2-3) has been changed accordingly.

-	Clause 4.6.1.1, After Step 12: the editor’s note is removed, since other parts of the living document (e.g., Clause 4.2.3, Clause 4.6.2.2, Clause 5.1.1) are updated to show which keys are used.

-	Clause 4.6.2.2: In addition to the change of key hierarchy for 5GS (in order to include EAP-AKA’), the single key hierarchy diagram for external key agreement (GBA, 5G GBA, AKMA, proprietary) has been replaced by three separate key hierarchy diagrams, showing which keys are used as the basis for each of the key hierarchies.

-	Clause 5.1.1: For each of the external key agreements (GBA, 5G GBA, AKMA, proprietary) the keys used as input key are explicitly mentioned.

-	Clause 6.2.6.1.1: The editor’s note on ‘requesting BEST confidentiality service’ has been resolved by changing the text from ‘is requesting’ to ‘supports’. In fact, it is not the UE that is requesting this service, but it is the HSE that chooses to apply encryption or not. The UE can only indicate its support for encryption. The presence of Serving network information was not the intended indicator for ‘requesting’ this service. Its presence was only required, because encryption was not always allowed depending on the serving network, so the HSE had to know the serving network in case it wanted to apply encryption.

-	Clause 6.2.6.1.2: Table 6.2.6.1.2-3, Key Agreement TLV. This TLV has been extended to include B-TID (for GBA and 5G GBA) and A-KID (for AKMA). The length of B-TID, A-KID and Pre-Agreed Identifier are out-of-scope for TS 33.163.

-	Clause 6.2.6.1.2: The editor’s notes on the relationship between key identifiers and keys for GBA, 5G GBA, and AKMA are resolved by replacing the generic Pre-Agreed Key Identifier by B-TID and A-KID, respectively.

# 4	Detailed proposal

START OF CHANGE 0a

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.863: "Study on battery efficient security for very low throughput Machine Type Communication (MTC) devices".

[3]	3GPP TS 33.102: "3G security; Security architecture".

[4]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[5]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[6]	3GPP TS 55.241: "Specification of the GIA4 integrity algorithm for GPRS; GIA4 specification"

[7]	3GPP TS 55.251: "Specification of the GEA5 encryption and GIA5 integrity algorithms for GPRS; GEA5 and GIA5 algorithm specification"

[8]	3GPP TS 35.201: " Specification of the 3GPP confidentiality and integrity algorithms; Document 1: f8 and f9 specification".

[9]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[10]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[11]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[12]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[13]	3GPP TS 33.220: " Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[14]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[15]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[xx]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[yy]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)"

[zz]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3"

END OF CHANGE 0a

START OF CHANGE 1

### 4.2.3	Architecture when using GBA, 5G GBA, AKMA or proprietary key agreement

When using GBA, according to TS 33.220 [13], 5G GBA, according to TS 33.220 [13], Annex X, AKMA, according to TS 33.535 [yy], or proprietary key agreement, the architecture may be either as for EPS architecture or 5GS architecture as required. In this case, the HSE shall be connected to the EPS or 5GS architecture according to the GBA, 5G GBA, AKMA or proprietary service instead. For GBA and 5G GBA, the HSE shall act as NAF. For AKMA, the HSE shall act as AF.

END OF CHANGE 1

START OF CHANGE 2

#### 4.6.1.1	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement can be based on the 3GPP AKA mechanism detailed in TS 33.102 [3], TS 33.401 [12], or TS 33.501 [xx], respectively, or based on one of the following external key agreement methods: GBA [13], 5G GBA [13], Annex X, AKMA [yy], or proprietary key agreement.  For 3GPP AKA based key agreement, the AKA challenge is then transported between the HSE and the UE as part of the BEST service detailed in clause 4.3.2. The key hierarchy is shown in clause 4.6.2.2.

The EMDSP protocol has 7 Key IDs for each session ID.  Each Key ID has a separate keyset consisting of an integrity Key (KE2Mint), an encryption key (KE2Menc), optionally an Intermediate Key (KIntermediate) and optionally an EAS PSK (KEAS_PSK). The Key IDs shall be set during the derivations of the keys as specified in clause 5.1.

The Intermediate Key (KIntermediate) is used together with a separate enterprise server identifier (EAS Id) to calculate the EAS PSK (KEAS_PSK).  There can be many KEAS_PSK derived from one KIntermediate.

The EAS PSK (KEAS_PSK) is used together with the Enterprise Key to calculate KE2Eint and KE2Eenc when BEST User plane security services are used in UE-to-EAS mode.

Figure 4.6.1.1-1 shows the generic key agreement process for EPS networks:

UE

HSE

1. EMSDP Session Request

EMKS

HSS

2. Keys required?

3. Authentication-Information-Request

4. Authentication-Information-Answer

d. Authentication-Information-Answer

c. Authentication-Information-Answer (to deliver authentication vectors – if required)

a. Authentication-Information-Request

b. Authentication-Information-Request (to request authentication vectors - if required)

Authentication vectors needed?

8. EMSDP Session Start

5. Calculate HSE Keys

EAS

6. EAS Session Request

7. EAS Session Start

12. Calculate E2E Keys

9. EMSDP Session Start Confirmation (conditional).

11. Calculate UE Keys

10. EAS Session Start Confirmation

Without EMKS

With EMKS

Figure 4.6.1.1-1: Generic BEST key agreement process for EPS networks

The Key agreement steps are:

1.	EMSDP Session Request (UE ID, BEST capabilities, Enterprise information, serving network (cond)). The UE shall send the EMSDP Session Request (UE ID, BEST capabilities, Enterprise information, serving network (cond)) to set up a new BEST session. For UEs supporting the Rel.17 release of BEST, the BEST capabilities shall indicate that 4G key agreement is supported.

2.	Keys required? - the HSE shall check to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the HSE has a valid set of keys for the indicated session and the UE ID is valid for that session then the HSE may start the BEST session without re-negotiating the keys (step 8).

-	If the UE ID is valid for that HSE and the HSE does not have a valid set of keys for the indicated session or the HSE wishes to update the keys, then it shall first renegotiate the keys (steps 2 to 7) and then start the BEST session (step 8).

-	If the UE ID is not valid for that session ID or the UE does not support the level of service required by the HSE or the enterprise information is not valid for the HSE, then the HSE may reject the command.

3. Authentication-Information-Request over S6a interface – The HSE shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI. For UEs supporting the Rel.14 release of BEST, the request shall indicate that UMTS authentication vectors are requested. For UEs supporting the Rel.17 release of BEST, the request shall indicate that EPS authentication vectors are requested.

4. Authentication-Information-Answer over S6a interface – The HSS uses the S6a interface to the HSE to return the requested authentication vectors.

a. Authentication-Information-Request over the S6a interface between HSE and EMKS – Where an EMKS is used, the HSE shall use the S6a interface to the EMKS to request an authentication vector using the UE IMSI.

b. Authentication-Information-Request over the S6a interface between EMKS and HSE – The EMKS shall use the S6a interface to the HSS to request one or more authentication vectors using the UE IMSI.

c. Authentication-Information-Answer over the S6a interface between EMKS and HSE – The HSS shall use the S6a interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the S6a interface between HSE and EMKS – The EMKS uses the S6a interface to the HSE to return the requested authentication vector.

5. Calculate UE-to-HSE Keys -  See key derivation details in clause 5.

6. The HSE may optionally send  "EAS Session Request" to the EAS– In case BEST UP service is used in UE-to-EAS mode, the HSE shall use the HSE interface to the EAS to inform the EAS of the new UE-to-EAS session request and shall forward the  EAS PSK (KEAS_PSK)  to the EAS.  In case the BEST key agreement service is used, the HSE shall forward to the EAS, the EAS PSK (KEAS_PSK) and the key identifier for the Intermediate Pre Shared Key (KIntermediate).

7. The Enterprise Server sends a "EAS Session Start" to the HSE – The Enterprise Server shall respond by sending the "UE-to-EAS Session Start" message. In case BEST UP service is used, this message may contain an EAS container that includes an identifier for the Enterprise Key.

8. EMSDP Session Start message -  The HSE shall send a EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID(opt) , EAS container (opt)). The Session Parameters shall contain RAND and AUTN from the received authentication vectors. As described in TS 33.401 [12], clause 6.1.2, the "separation bit" in the AMF field of AUTN shall be set to 0 if UMTS authentication vectors have been received, and the "separation bit" in the AMF field of AUTN shall be set to 1 if EPS authentication vectors have been received. For UEs supporting the Rel.17 release of BEST, the Session Parameters shall indicate the selected key agreement (i.e. 4G key agreement).

9. EMSDP Session Start Confirmation - UE optionally, if requested in the Session Start Confirmation, responds with an EMSDP Session Start Confirmation message.

10.  EAS Session Start Confirmation - The HSE may optionally send EAS Session Start Confirmation.

11. Calculate UE Keys – See key derivation details in clause 5.

12. Calculate UE-to-EAS Keys – In case of the UE-to-EAS BEST UP service, the Enterprise server generates UE-to-EAS keys as per the key derivation details in clause 5.

Figure 4.6.1.1-2 shows the generic key agreement process for 5GS networks:

Figure 4.6.1.1-2: Generic BEST key agreement process for 5GS networks

The Key agreement steps are:

1.	EMSDP Session Request (UE ID, BEST capabilities, Enterprise information, serving network). The UE shall send the EMSDP Session Request (UE ID, BEST capabilities, Enterprise information, serving network) to set up a new BEST session. The BEST capabilities shall indicate that 5G key agreement is supported.

2.	Keys required? - the HSE shall check to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the EMSDP Session Request is sent with a valid Session ID (i.e., not 0) and there is a valid key set for that session, then the HSE may start the BEST session without re-negotiating the keys (step 8). In this case the received UE ID may be ignored.

-	If the EMSDP Session Request is sent without a valid Session ID or using a single byte Session ID set to 0 or the HSE wishes to update the keys, then it shall first renegotiate the keys (steps 2 to 7) and then start the BEST session (step 8).

-	If the UE does not support the level of service required by the HSE or the enterprise information is not valid for the HSE, then the HSE may reject the command.

3.	Nudm_UEAuthentication_GetBESTav Request over SBA interface – The HSE shall use the SBA interface to the UDM/ARPF to request an authentication vector using the UE SUCI. The service operation Nudm_UEAuthentication_GetBESTav Request has the same inputs as the service operation Nudm_UEAuthentication_Get Request.

4.	Nudm_UEAuthentication_GetBESTav Response over SBA interface – The UDM/ARPF uses the SBA interface to the HSE to return a BEST-adapted 5G HE authentication vector (if 5G AKA is the selected authentication method) or a transformed authentication vector (if EAP-AKA' is the selected authentication method).The service operation Nudm_UEAuthentication_GetBESTav Response has the same outputs as the service operation Nudm_UEAuthentication_Get Response, except that in the BEST-adapted 5G HE authentication vector the parameter KAUSF is replaced by KHSE (see clause 5.1.0a).

5.	Calculate UE-to-HSE Keys -  See key derivation details in clause 5.

6.	The HSE may optionally send  "EAS Session Request" to the EAS– In case BEST UP service is used in UE-to-EAS mode, the HSE shall use the HSE interface to the EAS to inform the EAS of the new UE-to-EAS session request and shall forward the  EAS PSK (KEAS_PSK)  to the EAS.  In case the BEST key agreement service is used, the HSE shall forward to the EAS, the EAS PSK (KEAS_PSK) and the key identifier for the Intermediate Pre Shared Key (KIntermediate).

7.	The Enterprise Server sends a "EAS Session Start" to the HSE – The Enterprise Server shall respond by sending the "UE-to-EAS Session Start" message. In case BEST UP service is used, this message may contain an EAS container that includes an identifier for the Enterprise Key.

8.	EMSDP Session Start message -  The HSE shall send a EMSDP Session Start (Key Agreement, Session Parameters, Request Validation, HSE ID (opt), EAS container (opt)). The Session Parameters shall contain RAND and AUTN from the received authentication vectors, and the authentication method selected by the UDM in step 4. The "separation bit" in the AMF field of AUTN shall be set to 1. The Session Parameters shall indicate that 5G key agreement has been selected.

9.	EMSDP Session Start Confirmation - UE optionally, if requested in the Session Start Confirmation, responds with an EMSDP Session Start Confirmation message.

10.	EAS Session Start Confirmation - The HSE may optionally send EAS Session Start Confirmation.

11.	Calculate UE Keys – See key derivation details in clause 5.

12.	Calculate UE-to-EAS Keys – In case of the UE-to-EAS BEST UP service, the Enterprise server generates UE-to-EAS keys as per the key derivation details in clause 5.

END OF CHANGE 2

START OF CHANGE 3

#### 4.6.2.2	BEST Key Hierarchy for Separate BEST Domain

For EPS networks, and UEs supporting the Rel.14 release of BEST, the BEST key hierarchy is as depicted in Figure 4.6.2.2-1.

Figure 4.6.2.2-1: BEST Key Hierarchy for EPS networks and Rel.14 BEST UEs

For EPS networks, and UEs supporting the Rel.17 release of BEST, the BEST key hierarchy is as depicted in Figure 4.6.2.2-2.

Figure 4.6.2.2-2: BEST Key Hierarchy for EPS networks and Rel.17 BEST UEs

For 5GS networks the BEST key hierarchy is as depicted in Figure 4.6.2.2-3.

Figure 4.6.2.2-3: BEST Key Hierarchy for 5GS networks

The derivation of the key KHSE is described in clause 5.1.0a.

For GBA and 5G GBA based key agreement, the BEST key hierarchy is as depicted in Figure 4.6.2.2-4.

Figure 4.6.2.2-4: BEST Key Hierarchy for GBA and 5G GBA based key agreement

For AKMA based key agreement, the BEST key hierarchy is as depicted in Figure 4.6.2.2-5.

Figure 4.6.2.2-5: BEST Key Hierarchy for AKMA based key agreement

For proprietary key agreement, the BEST key hierarchy is as depicted in Figure 4.6.2.2-6.

Figure 4.6.2.2-6: BEST Key Hierarchy for proprietary key agreement

The KIntermediate, KEAS_PSK and all of the keys derived from them are generated when indicated to do so in the BEST CP messaging.

END OF CHANGE 3

START OF CHANGE 3a

### 5.1.0a	Derivation of KHSE

When deriving a KHSE from CK, IK and the serving network name when producing authentication vectors, and when the UE computes KHSE according to 5G AKA, the following parameters shall be used to form the input S to the KDF:

-	FC =  0x63;

-	P0 = serving network name;

-	L0 = length of the serving network name (variable length as specified in TS 24.501 [zz]);

-	P1 = SQN   AK,

-	L1 = length of SQN  AK (i.e. 0x00 0x06).

The XOR of the Sequence Number (SQN) and the Anonymity Key (AK) is sent to the UE as a part of the Authentication Token (AUTN), see TS 33.102 [3]. If AK is not used, AK shall be treated in accordance with TS 33.102 [3], i.e. as 000…0.

The serving network name shall be constructed as specified in clause 6.1.1.4 of TS 33.501 [xx].

The input key KEY shall be equal to the concatenation CK || IK of CK and IK.

When deriving a KHSE from CK', IK' and the serving network name when producing authentication vectors, and when the UE computes KHSE according to EAP-AKA', the following parameters shall be used to form the input S to the KDF:

-	FC =  0x64;

-	P0 = serving network name;

-	L0 = length of the serving network name (variable length as specified in TS 24.501 [zz]);

-	P1 = SQN   AK,

-	L1 = length of SQN  AK (i.e. 0x00 0x06).

The XOR of the Sequence Number (SQN) and the Anonymity Key (AK) is sent to the UE as a part of the Authentication Token (AUTN), see TS 33.102 [3]. If AK is not used, AK shall be treated in accordance with TS 33.102 [3], i.e. as 000…0.

The serving network name shall be constructed as specified in clause 6.1.1.4 of TS 33.501 [xx].

The input key KEY shall be equal to the concatenation CK' || IK' of CK' and IK'.

END OF CHANGE 3a

START OF CHANGE 4

### 5.1.1	Derivation of UE-to-HSE keys and Intermediate Key

The HSE and UE shall derive the BEST UE-to-HSE keys and the Intermediate key which are derived from CK and IK, KASME, KAUSF, or GBA/ 5G GBA/ AKMA/ proprietary agreed key depending on the key agreement version selected. The following input string shall be used when the UE and the HSE derive the BEST UE-to-HSE user plane service keys KE2Menc and/or KE2Mint or the Intermediate BEST key for usage in further key derivations for the UE-to-EAS user plane services or the key agreement services:

-	FC = 0x60,

-	P0 = HSE id if supplied else NULL,

-	L0 = length of HSE id (i.e. 0x00 0x03 if HSE id supplied or 0x00 0x00 if not),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm type distinguisher

-	L2 = length of algorithm type distinguisher (i.e. 0x00 0x01)

Table 5.1.1-1: Algorithm type distinguishers

| Algorithm type distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |
| BEST Intermediate Key (KIntermediate) | 0x03 |

For 3G key agreement, the input key shall be equal to the concatenation CK || IK of CK and IK.

For 4G key agreement, the input key shall be equal to KASME.

For 5G key agreement, the input key shall be equal to KHSE (see clause 5.1.0a)

For GBA and 5G GBA, the input key shall be equal to Ks_(int/ext)_NAF.

For AKMA, the input key shall be equal to KAF.

For proprietary key agreement, the proprietary key shall be used as the input key.

The Intermediate Key ID shall be set equal to SQN  AK.

END OF CHANGE 4

START OF CHANGE 4a

### 6.2.2	EMSDP general structure

This clause details a type 01 control plane EMSDP message and a type 01 user plane EMSDP message.

Figure 6.2.2-1 shows the general structure of the EMSDP type 01 message:

| Control Plane type 01 message: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UP / CP Flag | RFU | Key ID | CP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |  |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |  |
| User Plane type 01 message: |  |  |  |  |  |  |  |  |
| UP / CP Flag | RFU | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |  |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |  |

Note 1: The length of these fields is indicated in the BEST HSE configuration TLV from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure 6.2.2-1: data stack for the EMSDP transfers

UP / CP Flag: 	This is a 1 bit field that when set to ‘1’ means that the data packet is a User Plane message and when set to ‘0’ means that the message is a control plane message.

RFU:	This is a 1 bit field that is reserved for future use. Set to ‘0’.

Key ID:	This is a 3 bit field that indicates the key being used for encryption and Integrity protection.  If no keys have been agreed between the HSE and the UE then this shall be ‘000’.  The Key ID is associated to a specific Session ID.

CP COUNTER:	This is a counter, used for control plane data messages, that is incremented every control plane message.  It is used to protect control plane data messages against replay attacks and its length is set based on the counter scheme indicated in the BEST HSE configuration TLV (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.  This counter value is associated to a specific Session ID.  There are two independent CP counter values, one for messages from the UE and one for messages to the UE.  The HSE shall not check the CP counter value if the incoming message is "EMSDP Session Request " and the Session ID is 0.

UP COUNTER:	This is a counter, used for user plain data messages, that is incremented every user plain data message.  It is used to protect user plain data message against replay and its length is set based on the counter scheme indicated in the BEST HSE configuration TLV (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE. This counter value is associated to a specific Session ID.  There are two independent UP counter values, one for messages from the UE and one for messages to the UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.    Its length is determined according to the Session ID scheme that is agreed. For signalling messages sent from the UE, outside of a BEST session, the Session ID length shall be 1 octet and the Session ID value shall be all 0’s. Two simultaneously existing sessions shall not have the same Session ID unless they correspond to the same UE.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST HSE configuration TLV.  For an EMSDP session request command the MAC shall not be present.

Data length:	This holds the length of the following data in this data packet. Its length is set in the EMSDP Session Request message.  This is not present if the data length is set to 0 in the BEST HSE configuration TLV.

Data:	This is the data being transferred.

Note: The content and processing of the BEST UP EMSDP data payload is out of scope.

END OF CHANGE 4a

START OF CHANGE 5

##### 6.2.6.1.1	EMSDP Session Request

The EMSDP Session Request command shall be used by the UE to trigger a new BEST session from the HSE.  This message shall include an identification of the UE, an indication of its BEST support. Optionally, the EMSDP Session Request command may include information on the end enterprise service that this data is a part of. For UEs not supporting 5G authentication, if the UE supports a 'BEST user plane confidential service' (as indicated by the support of any encryption algorithm), then the Serving network information shall be present, else this information is optional. For UEs supporting 5G authentication, the Serving network information shall be present.

This message may be sent after a PDP context has been setup.

Note: 	the content of this message is used in the following EMSDP start session.

The cmd options for the EMSDP session request command are as follows:

Table 6.2.6.1.1-1: EMSDP session request command options

| Name | M / C / O |
| --- | --- |
| IMSI TLV or SUCI TLV | M |
| BEST UE configuration TLV | M |
| Enterprise Setup Information Element TLV | M |
| Serving Network TLV | C |

IMSI TLV: This is a TLV that contains the IMSI as follows:

Table 6.2.6.1.1-2: IMSI TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| IMSI TLV Tag | 1 byte | M | 01 |
| Length | 1 byte | M | Length of IMSI value (X) |
| IMSI value | X bytes | M | according to TS 31.102 [15] clause 4.2.2 bytes 2 to 9.. |

SUCI TLV: This is a TLV that contains the 5G SUCI as follows:

Table 6.2.6.1.1-x: SUCI TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| SUCI TLV Tag | 1 byte | M | 0x |
| Length | 1 byte | M | Length of SUCI value (X) |
| SUCI value | X bytes | M | according to TS 33.501 [xx]. |

The SUCI TLV shall be used if SUCI is supported by the UE, otherwise the IMSI TLV shall be used.

BEST configuration TLV: This is a TLV that contains the BEST configuration details for the UE as follows:

Table 6.2.6.1.1-3: BEST UE configuration TLV

| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BEST UE configuration TLV Tag = '02' |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |
| BEST release supported by the UE |  |  |  | BEST counter schemes supported by the UE | Reserved for future use (set to 000) |  |  |
| BEST protocols supported for control plane messages | Reserved for future use (set to 000) |  |  | BEST protocols supported for user plane messages | Reserved for future use (set to 000) |  |  |
| User data push to UE Supported | BEST encryption algorithm GEA4 supported | BEST encryption algorithm GEA5 supported | BEST encryption algorithm UEA1 supported | BEST encryption algorithm UEA2 supported | BEST encryption algorithm EEA0 supported | BEST encryption algorithm 128-EEA1 supported | BEST encryption algorithm 128-EEA2 supported |
| BEST encryption algorithm 128-EEA3 supported | BEST integrity algorithm GIA4 supported | BEST integrity algorithm GIA5 supported | BEST integrity algorithm UIA1 supported | BEST integrity algorithm UIA2 supported | BEST integrity algorithm 128-EIA1 supported | BEST integrity algorithm 128-EIA2 supported | BEST integrity algorithm 128-EIA3 supported |
| Reserved for future use (set to 00) |  | BEST encryption algorithm 128-NEA1 supported | BEST encryption algorithm 128-NEA2 supported | BEST encryption algorithm 128-NEA3 supported | BEST integrity algorithm 128-NIA1 supported | BEST integrity algorithm 128-NIA2 supported | BEST integrity algorithm 128-NIA3 supported |
| BEST key agreement - 4G authentication supported | BEST key agreement - 5G authentication supported | BEST key agreement - GBA supported | BEST key agreement - 5G GBA supported | BEST key agreement - AKMA supported | BEST key agreement - Proprietary key agreement supported | Reserved for future use (set to 00) |  |

Where:

-	BEST release supported by the UE – an indicator the release of the BEST solution that the UE has been designed to.  If no release is indicated this means that the BEST service is not supported.

-	Value: This shall be a 4 bit field where "0000" = Rel.14, "0001" = Release 15, "0010" = Release 16, "0011" = Release 17 and "0100" to "1111" are RFU,

-	BEST counter schemes supported by the UE – a flag for each supported scheme. At least one scheme must be supported and indicated.

-	Value: '1' – Optimised EMSDP counter scheme.

-	BEST protocols supported for control plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

-	Value: '1' – EMSDP.

-	BEST protocols supported for user plane messages – a flag for each BEST control plane protocol that is supported by the UE. At least one scheme must be supported and indicated.

-	Value: '1' – EMSDP.

-	This flag is set to '0' if only the BEST Key agreement service is used

-	User data push to UE Supported – a flag to indicate whether the UE supports user data pushed to the UE without a request for user data from the UE.

-	Values:

'1' = User data push to UE supported,

'0' = User data push to UE not supported.

-	BEST encryption algorithms supported by the UE – a flag for each supported algorithm: GEA4, GEA5, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2, 128-EEA3, 128-NEA1, 128-NEA2 and 128-NEA3. EEA0 shall always be supported and means no encryption.

-	Values:

'1' = Algorithm supported,

'0' = Algorithm not supported.

-	BEST integrity algorithms supported by the UE – a flag for each supported algorithm: GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2,128-EIA3, 128-NIA1, 128-NIA2 and 128-NIA3.

-	Values:

'1' = Algorithm supported,

'0' = Algorithm not supported.

-	BEST Key agreement mechanisms supported by the UE: a flag for each supported key agreement method: 4G authentication, 5G authentication, GBA, 5G GBA, AKMA and proprietary key agreement.

-	Values:

'1' = Algorithm supported,

'0' = Algorithm not supported.

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection as follows:

Table 6.2.6.1.1-4: Enterprise Setup Information Element TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Enterprise Setup Information Element TLV Tag | 1 byte | M | 03 |
| Length | 1 byte | M | Length of Enterprise URL (X+1) |
| UE-to-EAS flag | 1 byte | M |  |
| Enterprise Id | X bytes | O |  |

UE-to-EAS flag: If set to 0 it indicates that the UE requests a UE-to-HSE BEST secure session. If set to 1, it means that the UE requests a UE-to-EAS BEST secure session. This flag is not used if BEST User plane security services are not used.

Enterprise Id: The enterprise Id is used by the HSE to identify the enterprise and the service that the data belongs to.  These services are out of scope of this specification. As an example, a URL may be used to identify the enterprise.

Serving network TLV: This is a TLV that contains information on the serving network.

Table 6.2.6.1.1-5: Serving Network TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Serving Network Element TLV Tag | 1 byte | M | 0B |
| Length | 1 byte | M | Length of MCC/MNC value (3) |
| MCC/MNC | 3 bytes | M | MCC/MNC as defined in 3GPP TS 24.008 [5] |

Response:

If the HSE agrees to setup the session, it shall respond with an EMSDP Session Start command.

If the HSE does not agree to setup a BEST session, it may respond with a EMSDP Message Reject command.  This command may include the reason that the request has been rejected.

END OF CHANGE 5

START OF CHANGE 6

##### 6.2.6.1.2	EMSDP Session Start

The EMSDP Session Start command is used by the HSE to setup a new BEST session.  This message shall contain information on the BEST service setup, key agreement details, a hash of the information sent by the UE in the prior EMSDP Session Request command and optionally, the HSE identity.

On receipt of this command the UE shall:

-	If no indication is given, perform a 3G security context authentication with the USIM using the RAND and AUTN combination from the Key Agreement TLV. If the USIM returns IK and CK values, the UE uses these keys and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.  If the USIM determines re-synchronisation is required and returns an AUTS then the UE sends a EMSDP Message  Reject command containing the AUTS to the HSE.

-	If indicated, perform a 4G security context authentication with the USIM using the RAND and AUTN combination from the Key Agreement TLV. If the USIM returns IK and CK values, the UE uses these keys to derive KASME and subsequently use KASME and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.  If the USIM determines re-synchronisation is required and returns an AUTS then the UE sends a EMSDP Message  Reject command containing the AUTS to the HSE.

-	If indicated, perform a 5G security context authentication with the USIM using the RAND and AUTN combination from the Key Agreement TLV. If the USIM returns IK and CK values, the UECKKAUSFalseiese k t derive KAUSFHSE (deing on the authentication method selected) and subsequently use KAUSFHSEs ane HE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5. The derived on and value ofdentical to clause 5.1.0a. If the USIM determines re-synchronisation is required and returns an AUTS then the UE sends a EMSDP Message  Reject command containing the AUTS to the HSE.

-	If indicated, perform a GBA based authentication. After establishing the agreed key (see clause 5.1.1), the UE uses this key and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.

-	If indicated, perform a 5G GBA based authentication. After establishing the agreed key (see clause 5.1.1), the UE uses this key and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.

-	If indicated, perform a AKMA based authentication. After establishing the agreed key (see clause 5.1.1), the UE uses this key and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.

-	If indicated, perform a proprietry based authentication. After establishing the agreed key (see clause 5.1.1), the UE uses this key and the HSE identity supplied (if the HSE Identity TLV is present) to generate the session keys for the EMSDP messages as detailed in clause 5.

-	Verify that the UE supports the BEST service indicated in the BEST Service configuration TLV.

-	Verify the received message format, the CP COUNTER value and the message MAC value.

-	Verify that the MAC supplied in the MAC TLV matches the MAC that would be produced for the previous EMSDP Session Request message if the BEST configuration in the BEST Service configuration TLV had been applied using the integrity key calculated from the Key agreement TLV.

The EMSDP Session Start command has the following cmd options:

Table 6.2.6.1.2-1: EMSDP Session Start command options

| Name | M / C / O |
| --- | --- |
| BEST Service configuration TLV | M |
| Key agreement TLV | M |
| EMSDP session request MAC TLV | C |
| HSE Identity TLV | O |
| EAS Container | O |

BEST Service configuration TLV: The BEST Service configuration TLV sets the BEST service parameters to be used in this session as follows:

Table 6.2.6.1.2-2: BEST Service configuration TLV

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BEST HSE configuration TLV Tag = '04' |  |  |  |  |  |  |  |  |
| Length of Best protocol ID contents = x bytes |  |  |  |  |  |  |  |  |
| BEST Service Activated | BEST encryption algorithm GEA4 to be used | BEST encryption algorithm GEA5 to be used | BEST encryption algorithm UEA1 to be used |  | BEST encryption algorithm UEA2 to be used | BEST encryption algorithm EEA0 to be used | BEST encryption algorithm 128-EEA1 to be used | BEST encryption algorithm 128-EEA2 to be used |
| BEST signalling plane protocol identifier |  |  |  |  |  |  |  |  |
| BEST user plane protocol identifier |  |  |  |  |  |  |  |  |
| BEST encryption algorithm 128-EEA3 to be used | BEST integrity algorithm GIA4 to be used | BEST integrity algorithm GIA5 to be used | BEST integrity algorithm UIA1 to be used |  | BEST integrity algorithm UIA2 to be used | BEST integrity algorithm 128-EIA1 to be used | BEST integrity algorithm 128-EIA2 to be used | BEST integrity algorithm 128-EIA3 to be used |
| Reserved for future use (set to 00) |  | UE triggered key refresh supported | Local BEST configuration management allowed |  | Reserved for future use (set to 0000) |  |  |  |
| New Session Required | Use EAS UP Keys | EMSDP MAC length |  |  | Size of EMSDP Data Length |  |  |  |
| Reserved for future use (set to 00) | Authentication method | BEST encryption algorithm 128-NEA1 to be used |  | BEST encryption algorithm 128-NEA2 to be used | BEST encryption algorithm 128-NEA3 to be used | BEST integrity algorithm 128-NIA1 to be used | BEST integrity algorithm 128-NIA2 to be used | BEST integrity algorithm 128-NIA3 to be used |
| BEST key agreement - 4G authentication required | BEST key agreement - 5G authentication required | BEST key agreement - GBA required |  | BEST key agreement - 5G GBA required | BEST key agreement - AKMA required | BEST key agreement - Proprietary key agreement required | Reserved for future use (set to 00) |  |

-	BEST Service Activated – a bit flag that when set instructs the UE to use the BEST service and when clear instructs the UE not to use the BEST service,

-	BEST signalling plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 signalling plane EMSDP message. All other values are reserved for future use.

-	BEST user plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 user plane EMSDP message. All other values are reserved for future use.

-	BEST encryption algorithm to be used – 1 octet that is used to define which of the following algorithms to use for encryption: GEA0, GEA4, GEA5, UEA0, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2, 128-EEA3, 128-NEA1, 128-NEA2, and 128-NEA3. Only one algorithm from this list shall be indicated.  If the Visited network indicated that BEST encryption is restricted, then the HSE shall indicate EEA0.

-	BEST integrity algorithm to be used – 1 octet that is used to define which one of the following algorithms to use for integrity: GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2, 128-EIA3, 128-NIA1, 128-NIA2, and 128-NIA3. Only one algorithm from this list shall be indicated.

-	Local BEST configuration management allowed – a flag to indicate that the software connected to the UE is allowed to manage the BEST service.

- 	UE triggered key refresh supported – The HSE indicating to the UE whether key refresh requests will be ignored or responded.

-	New Session Required – 1 bit that indicates if a new session is required.  If this bit is set to 0 then the details agreed for the last session can be used and a new session is not required to be setup.

-	Use EAS UP keys – If set to 0 it indicates that the UE should not derive the UE-to-EAS keys. If set to 1 it means that the UE shall derive UE-to-EAS keys to be used in a UE-to-EAS BEST secure session.

-	EMSDP MAC length – 2 bits that indicates how many octets in the EMSDP data packet the integrity checksum (MAC) will be on, as follows: "00"=4 octets, "01"=8 octets, "10"=12 octets and "11"= 16 octets.  This value shall not be set to a size that is greater than MAC size produced by the chosen algorithm.

-	Size of EMSDP Data Length – 4 bits that indicate how many octets are used for the EMSDP Data Length. "0000" is reserved for future use.

-	Authentication method – 1 bit that indicates which authentication method was selected by the UDM.

-	Values:

'0' = 5G AKA was selected,

'1' = EAP-AKA' was selected.

-	BEST Key Agreement Method - 6 bits that indicate which one of the key agreement methods to use, the bit set indicates the relevant method from the following methods:  4G authentication, 5G authentication, GBA, 5G GBA, AKMA and proprietary key agreement. At most one of the BEST Key Agreement Method values shall be set to 1 and the BEST Key Argeement Method set shall be one of the supported BEST Key Agreement Methods indicated in the preceeding Sesssion Request.

Any remaining bits are reserved for future use and are set to "0..0".

Key agreement TLV:

Table 6.2.6.1.2-3: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | 05 |
| Length | 1 byte | M | 1 or 36 or X+1 or Y+1 or Z+1 |
| Additional Information | 1 Byte | M | Additional Keys to be generated |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [5] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [5] |
| B-TID | X bytes | C | See 4.4.7 in 3GPP TS 33.220 [13] |
| A-KID | Y bytes | C | See 4.4.2 in 3GPP TS 33.535 [yy] |
| Pre-Agreed Key Identifier | Z bytes | C | Alphanumeric string |
| NOTE 1: For 3G/4G/5G key agreement, RAND IE and AUTN IE shall be present, B-TID, A-KID, and Pre-Agreed Key Identifier shall not be present. NOTE 2: For GBA and 5G GBA, B-TID shall be present, RAND IE, AUTN IE, A-KID, and Pre-Agreed Key Identifier shall not be present. NOTE 3: For AKMA, A-KID shall be present, RAND IE, AUTN IE, B-TID, and Pre-Agreed Key Identifier shall not be present. NOTE 4: For proprietary key agreement, Pre-Agreed Key Identifier shall be present, RAND IE, AUTN IE, B-TID, and A-KID shall not be present. |  |  |  |

If the 'Length of the Key agreement' is set to 1 then this means use current keyset agreed for this KEY ID.  In this case the RAND IE and AUTN IE shall not be present.

If BEST key agreement - 4G authentication required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then RAND IE and AUTN IE shall be present in the Key Agreement TLV. In this case, B-TID, A-KID, and Pre-Agreed Key Identifier shall not be present in the Key Agreement TLV.

If BEST key agreement - 5G authentication required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then RAND IE and AUTN IE shall be present in the Key Agreement TLV. In this case, B-TID, A-KID, and Pre-Agreed Key Identifier shall not be present in the Key Agreement TLV.

If BEST key agreement - GBA required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then B-TID shall be present in the Key Agreement TLV set to a value that identifies the key generated by the GBA key agreement used for this purpose. In this case, RAND IE, AUTN IE, A-KID, and Pre-Agreed Key Identifier shall not be present in the Key Agreement TLV.

If BEST key agreement - 5G GBA required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then B-TID shall be present in the Key Agreement TLV set to a value that identifies the key generated by the 5G GBA key agreement used for this purpose. In this case, RAND IE, AUTN IE, A-KID, and Pre-Agreed Key Identifier shall not be present in the Key Agreement TLV.

If BEST key agreement - AKMA required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then A-KID shall be present in the Key Agreement TLV set to a value that identifies the key generated by the AKMA key agreement used for this purpose. In this case, RAND IE, AUTN IE, B-TID, and Pre-Agreed Key Identifier shall not be present in the Key Agreement TLV.

If BEST key agreement - Proprietary Key agreement required is indicated in the BEST HSE configuration TLV and if the Length is not set to 1 then Pre-Agreed Key Identifier shall be present in the Key Agreement TLV set to a value that identifies the Proprietary key to be used. In this case, RAND IE, AUTN IE, B-TID, and A-KID shall not be present in the Key Agreement TLV.

The Additional information are as follows:

Table 6.2.6.1.2-4: Additional information

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Confirm Authentication flag | RFU | RFU | RFU | RFU | Key ID |  |  |

b8: 	Confirm Authentication flag:

1 = Confirmation message required.

0 = Confirmation message not required.

b7 to b4: RFU (set to 0)

b3 to b1: Key ID

Key ID to be used for this keyset.

EMSDP Session Request MAC TLV:  The EMSDP session request MAC TLV shall be present if the previous command was an EMSDP session request message. Its contents are as follows:

Table 6.2.6.1.2-5: EMSDP session request MAC TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| EMSDP session request MAC Tag | 1 byte | M | 06 |
| Length | 1 byte | M | Length of MAC (X) |
| EMSDP session request MAC | X bytes | M | Result of MAC calculation on previous EMSDP session request message using current keys and BEST configuration in this message. |

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network. It is formatted as follows:

Table 6.2.6.1.2-6: HSE Identity TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| HSE Identity Tag | 1 byte | M | 07 |
| Length | 1 byte | M | Length of HSE Identity (04) |
| HSE Identity | 4 bytes | M | 4 octet numeric identifier for the HSE |

The EAS Container TLV: this contains a 4 octet numeric identifier for the Enterprise Key ID.

Table X.2.6.1.2-7: EAS Container TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| EAS Container Tag | 1 byte | M | 08 |
| Length | 1 byte | M | Length of Enterprise Key ID |
| Enterprise Key ID | 4 bytes | M | numeric identifier for the Enterprise Key |

Response:

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE shall send an EMSDP start session confirmation message.

If the Confirmation message required flag in the Key agreement TLV is set and the message verifies, then the UE may send an EMSDP start session confirmation message.

If the message does not verify, then the UE shall respond with a Request Rejected command.  This command may include the reason that the request has been rejected.

If the USIM returns a AUTS as a result of the authentication, the UE shall respond with a Request Rejected command with the reason "Authentication ReSync required" and including the AUTS.

END OF CHANGE 6