3GPP TSG-SA3 Meeting #105-e 	S3-214107-r1

- e-meeting, 8 - 19 November 2021											                Revision of S3-21xxxx

- Source:	China Mobile

- Title:	Add evaluation and system impact to solution #17

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

This contribution proposes evaluation and system impact to solution #17 in TR 33.857.

# 2	References

[1]	3GPP TR 33.857 v0.8.0

# 3	Rationale

This contribution proposes to add evaluation and system impact to solution #17 in TR 33.857 [1].

# 4	Detailed proposal

*** START of CHANGE ***

## 6.17	Solution #17: Solution to Provisioning of PNI-NPN Credentials

### 6.17.1	Introduction

This solution aims at addressing Key Issue #2 about provisioning of PNI-NPN credentials.

### 6.17.2	Solution details

The architectural assumption of this solution is as below. The PNI-NPN provisioning server is connected to NEF if the PNI-NPN provisioning server is outside the operator domain, in order to acquire derived keys using AKMA to protect the communication between the UE and the PNI-NPN provisioning server. Once the secure tunnel is established between the UE and the PNI-NPN provisioning server, the PNI-NPN credentials could be provisioned to the UE from the PNI-NPN provisioning server via the protected tunnel.

The procedures are as follows:

The pre-requisite is as what defined in TS 33.535[X], the UE has completes a successful primary authentication and thus results in KAKMA and A-KID generated and stored at AAnF.

- UE sends the Access Request to the PNI-NPN provisioning server, carrying the generated A-KID.

- While receving the Acess Request from the UE, the PNI-NPN provisioning server requests the derived keys used for protection the communication between the UE and the provisioning server from AAnF, carrying A-KID and PNI-NPN provisioning server ID.

- AAnF generates KPNINPN from KAKMA.

- AAnF sends the generated KPNINPN and its lifetime to PNI-NPN provisioning server.

- While receiving the KPNINPN, the PNI-NPN provisioning server sneds the access response to the UE and indicates the UE to generate KPNINPN accordingly.

- UE and the PNI-NPN provisioning server establishes a secure tunnel between them based on KPNINPN, in order to allow PNI-NPN provisioning server sends the PNI-NPn credentials in a secure manner.

### 6.17.3	System Impact

This solution has no additional system impact.

### 6.17.4	Evaluation

This solution fulfils the requirements of key issue #2 on PNI-NPN credential provisioning. This solution requires the UE and the operator of the PNI-NPN to support AKMA. The credentials provisioned by the PNI-NPN PS are known to the onboarding network.

*** END of CHANGE ***