3GPP TSG-SA WG3 Meeting #106-e	S3-220068

e-meeting, 14 – 25 February 2022                                                           revision of S3-20xabc

Source:	InterDigital

Title:	TR 33.870 – Informative Annex Y

Document for:	Approval

Agenda Item:	5.14 - Study of privacy of identifiers over radio access

Work Item / Release:	n/a

This contribution proposes Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access.

# 1	Decision/action requested

It is requested to approve this proposed content for Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access

# 2	References

[1]	SP-211364	New WID for Study of privacy of identifiers over radio access

# 3	Rationale

SP-211364 [1] in section 3, Justification, necessitates providing, “…examples of identifiers that require a comprehensive privacy evaluation” and lists SUPI, SUCI, S-NSSAI, 5G-GUTI as examples. To systematically study the potential privacy impacts/risks of 3GPP identifiers it is proposed to list them.

This contribution proposes Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access.

# 4	Detailed proposal

START OF CHANGE 1

Annex <Y> (informative):
List of 3GPP identifiers with potential privacy sensitivity.

The following table lists 3GPP identifiers and parameters transmitted over the air.

| No | Name of 3GPP Identifier | Description | Specified in 3GPP document | Summary |
| --- | --- | --- | --- | --- |
| 1 | SUPI (in NAI format) | SUbscription Permanent Identifier may exist in IMSI or in NAI formats | TS 23.003 [7], TS 23.501 [2] |  |
| 2 | SUCI (concealing SUPI in NAI format) | SUbscription Concealed Identifier | TS 23.003 [7], TS 23.501 [2] |  |
| 3 | MCC/MNC | Mobile Country Code (MCC) Mobile Network Code (MNC). Two separate parts of SUPI in IMSI format. | TS 23.003 [7] |  |
| 4 | S-NSSAI | Single Network Slice Selection Assistance Information | TS 23.003 [7], TS 23.501 [2] |  |
| 5 | 5G-GUTI | 5G Globally Unique Temporary Identifier | TS 23.003 [7] | 5G-GUTI provides an unambiguous identification of the UE that does not reveal the UE or the user's permanent identity. 5G-GUTI has two main components: - one that identifies the AMF(s) which allocated the 5G-GUTI; and - one that uniquely identifies the UE within the AMF(s). |
| 6 | CAG Identifier | A Closed Access Group (CAG) within a PLMN is uniquely identified by a CAG-Identifier | TS 23.003 [7], TS 23.501 [2] |  |
| 7 | C-RNTI | Cell Radio Network Temporary Identifier | TS 38.300 [8], TS 38.321 [9] | C-RNTI is a unique identifier dedicated to a particular UE and used for identifying RRC Connection and scheduling. C-RNTI can be reallocated when a UE accesses a new cell with the cell update procedure. |

END OF CHANGE 1