- 3GPP TSG-SA3 Meeting #107-e 	draft_S3-220774-r1

- e-meeting, 16 – 20 May 2022 	revision of S3-22xxxx

- Source:	China Mobile, China Telecommunications, Nokia, Nokia Shanghai Bell

- Title:	KI on Protection of data and analytics exchange in roaming case

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

This contribution proposes to add a new KI on Protection of data and analytics exchange in roaming case

# 2	References

[1]

# 3	Rationale

This contribution proposes to add a new KI on Protection of data and analytics exchange in roaming case

# 4	Detailed proposal

It is proposed to approve the following changes in TR 33.738.

***	START OF  CHANGE	***

## 5.X	Key Issue #X: Protection of data and analytics exchange in roaming case

### 5.X.1	Key issue details

As per Key Issue #3 in TR 23.700-81 [yy], the HPLMN/VPLMN may need to collect data or consume analytics from the VPLMN/HPLMN in roaming scenario. The data or analytics may relate to particular UEs or contain information about all UEs or groups of UEs. Both PLMNs (VPLMN, HPLMN) need the ability to control the amount of data exposed and to abstract or hide network-internal aspects based on user consent, operator policy, regulatory constraints and/or roaming agreements.

As there might be possible architecture enhancements to support this exchange in roaming scenarios and of any necessary enhancements to related NFs in HPLMN and VPLMN. This key issue studies the security aspects of data and analytics exchange in roaming case should be considered in SA3

### 5.X.2	Security threats

If the communication between PLMNs is not confidentiality protected, then sensitive information may be leaked to unauthorized entities.

If the integrity of the data exchanged between PLMNs is not protected, the data may be modified.

If the PLMNs don’t have the ability to control the amount of data exposed and to abstract or hide network-internal aspects, the sensitive data may be leaked to other entities.

Without considering the regulatory aspects related to data protection and privacy, operators exchanging data and analytics between NFs and analytics functions may easily incur in a severe breach of law in their jurisdictions. This breach will likely entail penalties, as well as significant brand damages.

If exchanged sensitive and privacy data has not been adequately protected and restricted before it is shared between PLMNs, it will be subject to be leaked and abused through a compromised NF or NWDAF controlled by an attacker (e.g., malicious insider). This may be an attack vector for more attacks targeting the operator subscribers, such as location tracking (e.g., using TAI information), fraud, etc.

### 5.X.3	Potential security requirements

Confidentiality,integrity and replay protection shall be supported between PLMNs.

Abilities to control the amount of data exposed and to abstract or hide network-internal aspects shall be supported by PLMNs

To ensure a secure and legal data and analytics exchange between operators, data protection policies need to be considered and enforced.

Based on the sensitivity of the data and applicable regulatory rules, certain data may be subject to confidentiality protection and restricted access policies before it is in transit, i.e., before it is transferred to the other operator.

***	END OF CHANGE	***