|  | 5GMRR Doc 27_06r3 LS to 3GPP CT4 on Identification of source PLMN-ID in SBA |
| --- | --- |
| Meeting Information |  |
| Meeting Name and Number | 5GMRR #25 |
| Meeting Date | 2021-11-29 |
| Meeting Location | Virtual meeting |
| Document Information |  |
| Document Author | Mihaela Ambrozie (Vodafone) |
| Additional Contributors | Marc Balon (Orange), Gery Verwimp (BICS), Edward Yau (Syniverse), Diego Tafuto (TISparkle), Kathleen Leach (T-Mobile) |
| Document Creation Date | 2021-11-26 |
| Document Status | Approval |
| Security Classification | Non-confidential |
| Document Summary |  |
| This LS to 3GPP CT4 working group is to ask for action on Identification of source PLMN-ID in SBA in roaming scenario |  |

| Version History |  |  |
| --- | --- | --- |
| Date | Version | Author / Comments |
| 2021-11-26 | 1.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-01 | 2.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-02 | 3.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-06 | 4.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-30 | 19.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-10 | 23.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-12 | 26.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-19 | 29.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-21 | 31.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-27 | final | Pieter Veenstra (NetNumber) – Chair 5GMRR |

© GSMA © 2022. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

3GPP TSG-SA3 Meeting #107-e 	S3-220619

# e-meeting, 16 - 20 May 2022

# Source:	3GPP CT4

# Title:	LS on Clarification on MBS Security Context (MSK/MTK) Definitions

# Document for:	Information, Discussion

Agenda Item:4.10

3GPP TSG-CT WG4 Meeting #109-e	C4-222303

# E-Meeting, 6th – 12th Apr 2022

Title:	LS on Clarification on MBS Security Context (MSK/MTK) Definitions

Response to:

Release:	Rel-17

# Work Item:	     5MBS

Source:	CT4

To:	SA3

cc:	CT1, CT3, SA4

Contact Person:	Varini Gupta

E-mail Address:	Varini.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment:		None

1. Overall Description:

CT4 is developing support of MBS Keys distribution via control place procedure as specified in Annex W of 3GPP TS 33.501, and would like to request following clarifications from SA3:

Observation 1: It is CT4's understanding that MBS Key Material sent by MBSF towards MB-SMF (and then SMF and UEs) consists of a list of {Key Domain ID, MSK ID, MSK, MTK ID, MTK} per MBS Session. This is based on following information in Clause 6.3.2.1A:

…

The UE shall receive the following information via the User Service Discovery / Announcement procedures if protection of the MBMS User Service is applied:

…

…

-	Identifiers of the MSKs needed for the User Service.

For each MSK, the identifiers that shall be included are Key Domain ID and MSK ID. The Key Number part of each MSK ID shall be set to 0x0 to denote the current MSK. The Key Number values in the Service Announcement shall be ignored by the UE, since they may change over time and Key Group part of MSK ID is sufficient to identify the MSKs, see clause 6.3.2.1.

-	Mapping information how the MSKs are used to protect the different RTP sessions or FLUTE channels.

…

Hence, the information needs to contain a plural number of the above MBS Keys Sets each containing {Key Domain ID, MSK ID, MSK, MTK ID, MTK} per MBS Session.

Question 1: CT4 kindly requests SA3 to confirm if above understanding is correct?

------------

Observation 2: 3GPP TS 33.246 Clause 6.3.2.1 specifies that MSK-ID is encoded as:

Every MSK is uniquely identifiable by its Key Domain ID and MSK ID

where

Key Domain ID = MCC || MNC and is 3 bytes long.

NOTE 1:	When MCC || MNC is used as key identifier, the UE should not try to use it in another context, e.g. the UE should not compare the received MCC || MNC to parameters in radio level.

MSK ID is 4 bytes long and with byte 0 and 1 containing the Key Group part, and byte 2 and 3 containing the Key Number part…

…

Question 2: How is Key Domain ID encoded in case of SNPN?

Question 3: Use of "when" in above text may give an impression that it is optional to use MCC/MNC when encoding Key Domain ID. Is it mandatory to encode Key Domain ID using MCC/MNC only or any set of characters is allowed?

Question 4: Can SA3 confirm that the Key Domain ID shall be sent together with the MBS keys from the MBSF towards the MB-SMF (and then SMF and UEs).

------------

2. Actions:

To SA3:

ACTION: 	CT4 kindly requests SA3 to provide responses to above questions.

# 3. Dates of next CT4 meetings

The upcoming CT4 meetings can be found in the CT4 Meetings calendar