|  | 5GMRR Doc 27_06r3 LS to 3GPP CT4 on Identification of source PLMN-ID in SBA |
| --- | --- |
| Meeting Information |  |
| Meeting Name and Number | 5GMRR #25 |
| Meeting Date | 2021-11-29 |
| Meeting Location | Virtual meeting |
| Document Information |  |
| Document Author | Mihaela Ambrozie (Vodafone) |
| Additional Contributors | Marc Balon (Orange), Gery Verwimp (BICS), Edward Yau (Syniverse), Diego Tafuto (TISparkle), Kathleen Leach (T-Mobile) |
| Document Creation Date | 2021-11-26 |
| Document Status | Approval |
| Security Classification | Non-confidential |
| Document Summary |  |
| This LS to 3GPP CT4 working group is to ask for action on Identification of source PLMN-ID in SBA in roaming scenario |  |

| Version History |  |  |
| --- | --- | --- |
| Date | Version | Author / Comments |
| 2021-11-26 | 1.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-01 | 2.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-02 | 3.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-06 | 4.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-30 | 19.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-10 | 23.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-12 | 26.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-19 | 29.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-21 | 31.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-27 | final | Pieter Veenstra (NetNumber) – Chair 5GMRR |

© GSMA © 2022. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

3GPP TSG-SA3 Meeting #107-e 	S3-220633

# e-meeting, 16 - 20 May 2022

# Source:	3GPP SA2

# Title:	LS Response to LS on UE providing Location Information for NB-IoT

# Document for:	Information, Discussion

Agenda Item:3

3GPP TSG SA WG2 Meeting #149E (e-meeting)		S2-2201333

February 14-25 2022

Title:	LS Response to LS on UE providing Location Information for NB-IoT

Response to:	S2-2200412 / R2-2201957

Release:	Rel-17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2

Cc:	RAN3, CT1, SA3, SA3-LI

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA2 thanks RAN2 for the LS in S2-2200412 / R2-2201957 on a UE providing Location Information for NB-IoT.

SA2 copies the pertinent portion of this LS below with labels for each question.

For NB-IoT UEs AS security cannot be assumed to be supported. This has lead RAN2 to assume that sending a detailed location over AS cannot be supported and RAN2 has thus discussed whether to send coarse location. However, in latest received LS by SA3 in R2-2200149/S3-213360 regarding NR NTN location reporting, it is stated that even sending coarse location unprotected should be avoided, so RAN2 has assumed that not even coarse location may be reported over AS for NB-IoT. RAN2 also discussed the possibility UE providing location to the MME via NAS procedures as an alternative to deliver the location information via AS. RAN2 have the following questions to RAN3/SA2 and CT1: - To RAN3/SA2, we ask whether there are any issues from a RAN3 and SA2 point of view that a NB-IoT UE does not provide any location information over AS to eNB. - RAN2 assumes that not providing location over AS may lead to impact on E-UTRAN selecting correct core network, NNSF and constructing the mapped cell ID and any potential ULI. - To RAN3/SA2/CT1, if location is not provided over AS and the location is needed for network operation is it feasible from a RAN3 and SA2 point of view, which is assumed to be protected. - If providing location over NAS is a viable solution that is acceptable for network operation, then is it feasible that this is introduced in Rel-17.

SA2 ANSWER TO Q1

If an NB-IoT UE does not provide any location information over AS to an eNB, then SA2 assumes that the E-UTRAN would provide its best estimate of the UE’s location as a TAI+CGI to the MME when the NB-IOT UE is entering or in the ECM CONNECTED state. This should not impact the ability of an MME to assign a suitable Registration Area as the RAN seems unable to provide paging areas smaller than the coverage area of one of its satellite beams. The MME would be able to use the TAC or list of multiple TACs broadcast in an NB-IoT cell, which SA2 assumes could always be provided by the E-UTRAN, to support a less optimized Registration Area for a UE and verify restrictions for Forbidden Areas. Similarly, an MME would be able to verify that a UE is located in a country served by the EPC by invoking location using an E-SMLC and would thus not critically depend on location verification by the E-UTRAN. Hence SA2 concludes that there are no serious issues from an SA2 perspective if an NB-IoT UE does not provide any location information over AS to an eNB.

SA2 ANSWER TO Q2

SA2 has not considered or studied the UE provision of its location to the MME via NAS in Release 17. There might be a need for conversion (e.g. in E-UTRAN) into a CGI and TAI in which the UE is geographically located, however, this may be unnecessary as the PLMN is more normally trying to convert CGI into geographic location coordinates! There would also be issues concerning the reliability of the UE provided location.

# 2	Actions

To RAN2

ACTION: 	SA2 asks RAN2 to take the above answers into account in completing support for NTN access for NB-IoT UEs in Release 17.

# 3	Dates of next TSG SA WG2 meetings

SA2#150E			TBD		Electronic Meeting

SA2#151E			TBD		Electronic Meeting