3GPP TSG-SA3 Meeting #107-e 	S3-220645

# e-meeting, 16 - 20 May 2022

# Source:	Deutsche Telekom

# Title:	Post-Quantum Secure Subscription Concealed Identifier

# Document for:	Discussion

Agenda Item:	5.6

# 1	Decision/action requested

Based on the observation made in this document, it is proposed to agree the new KI proposed in S3-220644.

# 2	References

[1]	Jean-Pierre Seifert et al. WiSec ’22, A Post-Quantum Secure Subscription Concealed Identifier for 6G https://doi.org/10.1145/3507657.3528540

[2]	3GPP. 2022. Security architecture and procedures for 5G System. Technical Specification (TS) 33.501. 3rd Generation Partnership Project (3GPP). http://www.3gpp.org/DynaReport/33501.htm Version 17.4.2.

[3]	NIST. 2021. Post-Quantum Cryptography - CSRC, NIST. https://csrc.nist.gov/projects/post-quantum-cryptography

[4]	3GPP TR33.870, Study on privacy of identifiers over radio access

# 3	Rationale

5G saw the introduction of an encrypted user identifier, the Subscriber Concealed Identifier (SUCI), to provide confidentiality of the subscriber’s whereabouts and identities. The SUCI protects the new generation of cellular networks against tracking devices, so-called IMSI-catchers, which have undermined users’ confidentiality ever since the inception of cellular networks.

However, the potential advent of large-scale quantum computers in the near future threatens to compromise the confidentiality provided by the SUCI yet again. The security of the public-key cryptography that underpins the SUCI relies on the hardness of the discrete logarithm problem. Using Shor’s algorithm, a quantum adversary could break the SUCI’s cryptography and once more gain the capability to track and identify users. Advancements in quantum computing are unpredictable, and a breakthrough might be only a decade away. Given the lifetimes of cellular network deployments, it is thus necessary to already integrate now quantum-resistant cryptography into the current generation of cellular networks.

# 4	Detailed proposal

## 4.1	Post-Quantum Security of the SUCI Calculation

The paper [1] elaborates on the details of the issue and the need for implementation of quantum-resistant cryptography into the current and next-generation cellular networks. To enhance the ongoing SA3 Study on privacy of identifiers over radio access, following are the observations:

- To mitigate the threat to the subscriber’s privacy, 5G introduced the Subscription Permanent Identifier (SUPI) as a superset of the IMSI and also an encrypted subscriber identifier, the Subscription Concealed Identifier (SUCI), calculated by so-called protection schemes.

- To calculate the SUCI, public-key cryptography was integrated into the SIM card: The subscriber’s device encrypts the SUPI using a hybrid encryption scheme, based on an elliptic curve Diffie-Hellman key-exchange [2]. The complete calculation itself can be done directly in the SIM card or within the phone, using the network public key that is stored in the SIM card.

- The security of currently used asymmetric cryptography rests on the assumptions that (elliptic curve) discrete logarithm problems are practically infeasible. When leveraging quantum computers, however, this assumption no longer holds.

- The National Institute of Standards and Technology (NIST) stated that "Regardless of whether we can estimate the exact time of the arrival of the quantum computing era, we must begin now to prepare our information security systems to be able to resist quantum computing.” [3].

- The quantum threat is further aggravated by the design of the SUPI protection schemes: The current SUPI protection schemes do not provide forward secrecy. As a result, breaking an operator’s public key is sufficient to de-anonymize all SUPIs encrypted with that public key, without needing to solve additional discrete logarithm problems. Consequently, an adversary can invest large resources into recording SUCIs now and breaking just a small number of operator public keys (and thus de-anonymize all collected SUCIs) later on.

Detailed proposal:

Proposal: In the merit of privacy of identifiers over radio access it shall be studied if a modification of the existing SUPI protection schemes is needed to secure cellular networks against quantum adversaries. The related KI is proposed in S3-220644

# 5	Conclusion

The following main observations can be derived from previous section/discussion:

- The current SUCI protection schemes do not withstand attacks with quantum computers. Shor’s algorithm renders “all” cryptography based on the hardness of (elliptic curve) discrete logarithm problems, such as Diffie-Hellman, insecure.

- The current protection schemes derive the shared secret 𝑘 using a long-term public key instead of using an ephemeral public key also on the operator’s side. As a result, the protocol does not provide forward secrecy, and one invocation of Shor’s algorithm is sufficient to de-anonymize many recorded SUCIs (scenario: "record now, decrypt later").

Detailed conclusion:

Conclusion: There is need to add a new KI on “Post-Quantum Security of the SUCI Calculation” to the Rel-18 “Study on privacy of identifiers over radio access” [4]. Such proposal is given in the document S3-220644.