3GPP TSG-SA3 Meeting #107-e 	S3-220688

# e-meeting, 16-20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1360 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to secondary authentication during UE onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) Annex I.9.2.4.1 is for secondary authentication with DCS uses Default UE credentials. Default Credentials are the only credentials that are configured within UE. 2) Annex I.9.2.4.2 currently states that, during onboarding, the ON-SNPN may trigger a secondary authentication procedure with a DN-AAA server. In this case, Secondary authentication using DN-AAA use-case uses two different sets of credentials. However, the UE will not be configured with a DN and related credentials to be used with the onboarding network during onboarding. Specifically, the UE will not know which identifier and/or credentials to use during secondary authentication with a DN-AAA server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)In I.9.2.4.1, a note is added to state that there are only a single set of default credentials. 2) In I 9.2.4.2, a note is added that two sets of credentials may be needed in case of DCS is not involved with Secondary authentication using DN-AAA during onboarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Secondary authentication during UE onboarding is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.9.2.4.1, Annex I.9.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### I.9.2.4	Secondary authentication

#### I.9.2.4.1	Secondary authentication using DCS

After successful primary authentication as described in I.9.2.2 (i.e. primary authentication without using DCS), upon the establishment of the Onboarding PDU Session, the ON-SNPN may trigger secondary authentication procedure with the DCS using Default UE credentials as described in clause 11.1.

NOTE XX: It is assumed that UE is configured with only Default UE credentials.

#### I.9.2.4.2	Secondary authentication using DN-AAA

After successful primary authentication as described in I.9.2.2 or I.9.2.3, upon the establishment of the Onboarding PDU Session, the ON-SNPN may trigger secondary authentication procedure with a DN-AAA server as described in clause 11.1.

NOTE YY: It is assumed that UE is configured with default PDU session authentication and authorization credentials in addition to the Default UE credentials.

*** END CHANGES ***