3GPP TSG-SA3 Meeting #107-e  	S3-220751

# e-meeting, 16 – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.216 | CR | 0024 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the test case in TS 33.216 clause 4.2.2.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_eNB |  |  |  |  |  | Date: |  |  | 2022-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in TS 33.401 clause 7.2.4.2.2, in the path-switch message, the target eNB shall send the UE EPS security capabilities received from the source eNB to the MME. The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The bidding down prevention can not be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

4.2.2.1.10	Bidding down prevention in X2-handovers

Requirement Reference: TS 33.401 [3], clause 7.2.4.2.2

Requirement Description: "In the path-switch message, the target eNB shall send the UE EPS security capabilities received from the source eNB to the MME.The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored" as specified in TS 33.401 [3], clause 7.2.4.2.2."

Threat References: TBA

Test Case:

Purpose:

Verify that bidding down is prevented in X2-handovers.

Pre-Conditions:

Test environment with source eNB and target eNB, and the source eNB may be simulated.

Execution Steps:

1a). The target eNB sends the path-switch message to the MME with not tampered UE's security capabilities.

1b).The target eNB sends the path-switch message to the MME with tampered UE's security capabilities.

- The MME verify that the UE's security capabilities received from the target eNB are the same as the UE's security capabilities that the MME has locally stored.

2b). If there is a mismatch, the MME send its locally stored 5G security capabilities of the UE to the target eNB in the Path-Switch Acknowledge message.

3b). The target eNB receives UE's security capabilities from the MME in the Path-Switch Acknowledge message, the eNB verify that the UE's security capabilities received from the MME are the same as the UE's security capabilities from the source eNB. If there is a mismatch, the target eNB update the AS security context of the UE with security capabilities of the UE received from the MME.

Expected Results:

- The UE EPS security capabilities are in the path-switch message.

- The MME sends the UE's security capabilities that the MME has locally stored in the Path-Switch Acknowledge message. And the target eNB update the AS security context of the UE with security capabilities of the UE received from the MME.

Expected format of evidence:

Snapshots containing the result

*** END OF CHANGES ***