3GPP TSG-SA3 Meeting #107-e 	S3-220822

# e-meeting, 16 - 20 May 2022

# Source:	LG Electronics

# Title:	A Key issue in UPU protection service suspension

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

It is propsed to approve the key issue descrbied in this document.

2	References

[1]	3GPP TR 33.741 "Study on home network triggered primary authentication (HONTRA)".

[2]	S3-220821, 5.9 – New Study of Security aspect of home network triggered primary authentication, 3GPP SA3#107-e

3	Rationale

This proposal introduces a key issue on UPU protection service.

4	Detailed proposal

It is proposed to include the key issue in [1].

This key issue proposal has a relation with the use case proposal [2].

*** CHANGE start ***

5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

5.X	Key Issue #X: Suspension in UPU protection service due to UPU Counter wrap around

5.X.1	Key issue details

Suspension of UPU protection service due to the CounterUPU about to wrap around will block that UDM sends updated UE parameters to UE until next primary authentication is performed. It will lead the UE to have wrong UE parameters.

In order to avoid suspention of the service, home network triggered primary authentication is required to reset the CounterUPU by refreshing the KAUSF during the primary authentication.

5.X.2	Security threats

TBD

5.X.3	Potential security requirements

3GPP system shall provide means to primary authentication triggered by home network when the CounterUPU is about to wrap up.

*** CHANGE end ***