3GPP TSG-SA3 Meeting #107-e 	S3-220861

# e-meeting, 16 – 20 May 2022                                              			Revision of S3-22xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0705 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with RAN2 for LTE UP IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN2 has sent LS to the SA3 in R2-2203663: “UPIP for the EPC connected architectures uses NR PDCP and is configured in following way: ‐ (as is done for legacy LTE UE) an LTE algorithm code point is configured in field integrityProtectionAlgorithm in IE SecurityAlgorithmConfig in the TS 36.331 SecurityModeCommand message, and this is used to derive KUPint (and also to derive KUPEnc, as for legacy LTE UE). ‐ The NR algorithm code point (corresponding to the LTE algorithm code point used in the SecurityModeCommand) indicated by the integrityProtAlgorithm included in the securityConfig in the TS 38.331 RadioBearerConfig is used to configure the UP IP algorithm applied by NR PDCP to perform integrity protection. ‐ The integrityProtection indicated in pdcp-Config in the DRB-ToAddMod(list) in the TS 38.331 RadioBearerConfig is used to activate the UP IP for a DRB using the configured algorithm, which can be done only at DRB setup. Consequently, UP IP activation/deactivation for a DRB can be changed only by DRB-release-and-add.” Thus, it is proposed to make alignment with RAN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment with RAN2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.2, 5.1.4.2, 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

#### 5.1.3.2	Algorithm Identifier Values

All algorithms specified in this subclause are algorithms with a 128-bit input key except Null ciphering algorithm.

NOTE:	Deviations from the above requirement have to be indicated explicitly in the algorithm identifier list below.

Each EPS Encryption Algorithm (EEA) will be assigned a 4-bit identifier. Currently, the following values have been defined for NAS, RRC and UP ciphering:

"00002"      EEA0      Null ciphering algorithm

"00012"      128-EEA1      SNOW 3G based algorithm

"00102"      128-EEA2      AES based algorithm

"00112"      128-EEA3      ZUC based algorithm

The remaining values have been reserved for future use.

UEs and eNBs shall implement EEA0, 128-EEA1 and 128-EEA2 for both RRC signalling ciphering and UP ciphering. UEs and eNBs may implement 128-EEA3 for both RRC signalling ciphering and UP ciphering.

UEs and MMEs shall implement EEA0, 128-EEA1 and 128-EEA2 for NAS signalling ciphering. UEs and MMEs may implement 128-EEA3 for NAS signalling ciphering.

For ciphering using NR PDCP, UEs and eNBs shall implement 128-NEA1 and 128-NEA2 both for RRC signaling encryption and user plane encryption. UEs and eNBs may implement 128-NEA3 both for RRC signaling encryption and user plane encryption. The 128-NEA1, 128-NEA2 and 128-NEA3 are specified in 3GPP TS 33.501 [43].

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

5.1.4.2	Algorithm Identifier Values

All algorithms specified in this subclause are algorithms with a 128-bit input key.

NOTE:	Deviations from the above requirement have to be indicated explicitly in the algorithm identifier list below.

Each EPS Integrity Algorithm (EIA) will be assigned a 4-bit identifier. Currently, the following values have been defined:

"00002"      EIA0      Null Integrity Protection algorithm

"00012"      128-EIA1      SNOW 3G based algorithm

"00102"      128-EIA2 	 AES based algorithm

"00112"      128-EIA3 	 ZUC based algorithm

The remaining values have been reserved for future use.

UEs and eNBs shall implement 128-EIA1 and 128-EIA2 for RRC signalling integrity protection. UEs and eNBs may implement 128-EIA3 for RRC signalling integrity protection.

UEs shall and eNBs may implement 128-EIA1 and 128-EIA2 for the user plane integrity protection. UEs and eNBs may implement 128-EIA3 for the user plane integrity protection.

UEs and MMEs shall implement 128-EIA1 and 128-EIA2 for NAS signalling integrity protection. UEs and MMEs may implement 128-EIA3 for NAS signalling integrity protection.

For integrity protection using NR PDCP, UEs and eNBs shall implement 128-NIA1 and 128-NIA2 both for RRC signaling integrity protection and user plane integrity protection. UEs and eNBs may implement 128-NIA3 both for RRC signaling integrity protection and user plane integrity protection. The 128-NIA1, 128-NIA2 and 128-NIA3 are specified in 3GPP TS 33.501 [43].

UEs shall implement EIA0 for integrity protection of NAS and RRC signalling. As specified in clause 5.1.4.1 of this specification, EIA0 is only allowed for unauthenticated emergency calls. EIA0 shall not be used for integrity protection between RN and DeNB.

Implementation of EIA0 in MMEs, RNs and eNBs is optional, EIA0, if implemented, shall be disabled in MMEs, RNs and eNBs in the deployments where support of unauthenticated emergency calling is not a regulatory requirement.

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

7.3.4	UP integrity protection activation mechanism

AS UP integrity protection activation shall be done as part of the DRB addition procedure using RRC Connection Reconfiguration procedure as described in this clause, see Figure 7.3.4 -1.

As defined in Clause 7.3.3, the MME may send the UP integrity protection policy to the eNB. If the MME does not send the UP integrity protection policy, the eNB may use locally configured UP integrity protection policy.

Figure 7.3.4-1: User plane (UP) integrity protection activation mechanism

1a.	This RRC Connection Reconfiguration procedure which is used to add DRBs shall be performed only after RRC security and UP ciphering have been activated as part of the AS security mode command procedure defined in Clause 7.2.4.5 and the UE indicates that it supports use of user plane integrity protection with EPC.

1b.	The eNB shall send the RRC Connection Reconfiguration message to the UE for UP security activation containing indication for the activation of UP integrity protection for each DRB according to the security policy and the selected security algorithm. The eNB shall select the security algorithm using received or mapped UE NR security capabilities as described in Annex E.3.4.3.

1c.	If UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the eNB does not have KUPint, the eNB shall generate KUPint and UP integrity protection for such DRBs shall start at the eNB.

2a.	UE shall verify the RRC Connection Reconfiguration message. If successful, if UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the UE does not have KUPint, the UE shall generate KUPint. The UE shall use the selected security algorithm to activate UP integrity protection for such DRBs.

2b.	If the UE successfully verifies integrity of the RRC Connection Reconfiguration message, the UE shall send the RRC Connection Reconfiguration Complete message to the eNB.

If UP integrity protection is not activated for DRBs, the eNB and the UE shall not integrity protect the traffic of such DRB and shall not put MAC-I into PDCP packet.

*************** End of 4th Change ****************