3GPP TSG-SA3 Meeting #107-e	S3-220899

# e-meeting, 16 - 20 May 2022

# Source:	OPPO

# Title:	New solution: Security procedure of KAF refresh

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This CR proposes to solve the companion Key Issue on AKMA KAF refresh

# 2	References

[1] 	3GPP TS 33.535 "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[2] 	3GPP TS 33.737 " Study on Authentication and Key Management for Applications (AKMA) phase 2".

# 3	Rationale

This contribution addresses the security requirements in the companion contribution S3-220897.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TS 33.737[2].

**** START OF CHANGE ****

## 6.X	Solution #X: Security procedures of KAF refresh

### 6.X.1	Introduction

This solution addresses the new KAF generation after the lifetime of KAF expires, while the UE has not run a new primary authentication, which also avoid the AF inappropriate reject UE’s access after the KAF expires. Specifically, it is proposed to use Nonce chosen by AAnF to derive a new KAF* from the expired KAF, the AAnF also generates a MACAAnF to protect the Nonce transmission from AF to UE. And if the AF is located inside the operator's network, AF can request AAnF to refresh KAF, if the AF is located outside the operator's network, AF can request AAnF to refresh KAF through the NEF.

### 6.X.2	Solution details

#### 6.X.2.1	Procedure

Figure 6.X.1: Security procedure of KAF refresh

1. UE initiates communication with the specific AF, and include the A-KID in the application session establish request.

2. AF retrieves the AKMA context using the A-KID, and find out the lifetime of the KAF associated with the A-KID has expired. Then the AF shall send a Naanf_AKMA_ApplicationKey_Get request to AAnF with KAFRefreshInd to indicate the KAF refresh, which also includes A-KID and AF_ID in the request.

3. AAnF generates a Nonce and uses the generated Nonce to derive KAF* and MACAAnF according to the 6.X.2.2 and 6.X.2.3.

4. AAnF sends the Naanf_AKMA_ApplicationKey_Get Response to the AF with KAF*, KAF* expiration time, SUPI (if needed), Nonce, and MACAAnF.

5. AF sends the Application Session Establishment response to UE, with Nonce, MACAAnF, and KAFRefreshInd indicating the KAF refresh.

6. After UE receives the Application Session Establishment response with KAFRefreshInd. UE shall use Nonce and KAF to derive the KAF* according to 6.X.2.2, and then calculate a MACAAnF* according to 6.X.2.3. UE compares the calculated MACAAnF* with the MACAAnF received in step 4. If they coincide, UE shall consider the KAF* refresh successful, otherwise, the KAF* refresh fails and the UE shall reject the AF.

#### 6.X.2.2	Derivation of KAF*

When deriving a KAF* from KAF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXX;

-  P0 = AF-ID;

-  L0 = length of AF-ID;

-  P1 = Nonce;

-  L1 = length of Nonce;

The input KEY shall be KAF.

#### 6.X.2.3	Generation of MACAAnF

When deriving a MACAAnF from KAF*, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXY;

-  P0 = Nonce;

-  L0 = length of Nonce;

-  P1 = A-KID;

-  L1 = length of A-KID.

The input KEY shall be KAF*.

### 6.X.3	Solution Evaluation

This solution addresses KAF refresh without having to run a new primary authentication between UE and the network, also without any new message generated.

AAnF impact: AAnF needs to generate a Nonce to derive new KAF and MAC when it receives the Naanf_AKMA_ApplicationKey_Get Request with KAFRefreshInd from AF.

AF impact: AF needs to forward the Nonce and MAC to UE in the Application Session Establishment response message.

UE impact: UE needs to compute new KAF when it receives the KAF Key_Generation request message from AF.

**** END OF CHANGE ****