3GPP TSG-SA3 Meeting #107-e	S3-220906

# e-meeting, 16 - 20 May 2022

# Source:	OPPO

# Title:	New solution: Security procedure of KAF refresh

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This pCR proposes to solve the companion Key Issue on AKMA KAF refresh

# 2	References

[1] 	3GPP TS 33.535 "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[2] 	3GPP TS 33.737 " Study on Authentication and Key Management for Applications (AKMA) phase 2".

# 3	Rationale

This contribution addresses the security requirements in the companion contribution S3-220897.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.737 [2].

**** START OF CHANGE ****

## 6.X	Solution #X: Security procedures of KAF refresh

### 6.X.1	Introduction

This solution addresses the new KAF generation after the lifetime of KAF expires, while the UE has not run a new primary authentication, which also avoid the AF inappropriate reject UE’s access after the KAF expires. Specifically, it is proposed to use Nonce chosen by AAnF to derive a new KAF* from the expired KAF.

### 6.X.2	Solution details

#### 6.X.2.1	Procedure

Figure 6.X.1: Security procedure of KAF refresh

1. When the UE initiates communication with the AF, the UE sends Application Session Establishment Request message including the latest A-KID.

2a. AF retrieves the AKMA context using the A-KID, and find out the lifetime of the KAF associated with the A-KID has expired.

2b. AF then send Naanf_AKMA_ApplicationKey_Get Request to the AAnF, which contains A-KID, AF_ID, and KAFRefreshInd indicating the KAF refresh.

3a. The AAnF retrieves the UE AKMA context according to the received A-KID. The AAnF generates a Nonce and uses the generated Nonce to derive KAF* according to clause 6.X.2.2.

3b. The AAnF sends the KAF Key_Generation request to UE through AUSF, which contains A-KID, Nonce.

3c. The AAnF sends Naanf_AKMA_ApplicationKey_Get response to AF, which contains KAF*, KAF* expiration time, and SUPI (if needed).

4. The UE derives KAF* according to clause 6.X.2.2.

5. The AF sends Application Session Establishment Response to the UE.

#### 6.X.2.2	Derivation of KAF*

When deriving a KAF* from KAF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXX;

-  P0 = AF-ID;

-  L0 = length of AF-ID;

-  P1 = Nonce;

-  L1 = length of Nonce.

The input key shall be KAF.

### X.X.3	Evaluation

This solution addresses KAF refresh without having to run a new primary authentication between UE and the network.

AAnF impact: AAnF needs to generate a Nonce to derive new KAF when it receives the Naanf_AKMA_ApplicationKey_Get Request with KAFRefreshInd from AF. AAnF needs to support an additional message sent to UE (via AUSF) for KAF Key_Generation request.

UE impact: UE needs to compute new KAF when it receives the KAF Key_Generation request message from AAnF via AUSF.

**** END OF CHANGE ****