3GPP TSG-SA3 Meeting #107-e	S3-220908

# e-meeting, 16 - 20 May 2022

# Source:	OPPO

# Title:	New solution: Authentication algorithm selection between EEC and ECS, EEC and EES

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This pCR proposes to solve the companion key issue on selection between options on Edge

# 2	References

[1] 	3GPP TS 33.535 "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[2] 	3GPP TS 33.222 “Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)”

[3] 	3GPP TR 33.739 “Study on security enhancement of support for edge computing phase 2”

# 3	Rationale

This contribution addresses the security requirements in the companion contribution S3-220907.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.739[3].

*********** First Change***********

## 6.X	Solution #X: Authentication algorithm selection procedure between EEC and ECS

### 6.X.1	Solution overview

This solution addresses security requirement for authentication algorithm selection between EEC and ECS in key issue #X.

### 6.X.2	Solution details

To support authentication between the EEC and ECS, the EEC shall optionally use/implement AKMA as specified in TS 33.535[1], GBA as specified in TS 33.222[2] or other TLS authentication methods (e.g., client certificate, which out of 3GPP) to generate the key materials.

The EEC shall set the EEC authentication algorithm capability according to the local configuration. And the ECS can be configured via network management with algorithms which are allowed for usage, or before the authentication algorithm selection procedure, the ECS can be configured with the algorithm profile supported by HPLMN.

Before the authentication algorithm selection procedure between EEC and ECS, the EEC shall be pre-configured with or have discovered the address (e.g. URI) of the ECS. The authentication algorithm selection between EEC and ECS can be implemented in the TLS based authentication procedure, e.g., the shared key-based UE authentication with certificate-based AF authentication or shared key-based mutual authentication between UE and AF, given in Annex B of TS 33535[1], clause 5.3 and 5.4 of TS 33.222[2].

### 6.X.3	Solution evaluation

This solution partially addresses KI#X "Authentication algorithm selection between EEC and ECS, EEC and EES " in the companion contribution S3-220907. This solution addresses authentication algorithm selection between EEC and ECS.

This solution based on TLS authentication protocols introduces no extra impact to network entities and existing procedures.

***********End of First Change***********

*********** Second Change***********

## 6.Y	Solution #Y: Authentication algorithm selection procedure between EEC and EES

### 6.X.1	Solution overview

This solution addresses security requirement for authentication algorithm selection between EEC and EES in key issue #X.

### 6.X.2	Solution details

To support authentication between the EEC and EES, the EEC shall optionally use/implement AKMA as specified in TS 33.535[1], GBA as specified in TS 33.222[2], or other TLS authentication methods (e.g., client certificate) to generate the key materials.

The EEC shall set the EEC authentication algorithm capability according to the local configuration. The EES can be configured via network management with lists of algorithms which are allowed for usage, or the EES can be configured with the algorithm profile of HPLMN via the ECS, before algorithm selection with EEC.

Before the authentication algorithm selection procedure between EEC and EES, the EEC shall be configured with the address (e.g. URI) of the EES by the ECS. The authentication algorithm selection between EEC and EES can be implemented in the TLS based authentication procedure, e.g., the shared key-based UE authentication with certificate-based AF authentication or shared key-based mutual authentication between UE and AF, given in Annex B of TS 33535[1], clause 5.3 and 5.4 of TS 33.222[2].

### 6.X.3	Solution evaluation

This solution partially addresses KI#X "Authentication algorithm selection between EEC and ECS, EEC and EES " in the companion contribution S3-220907. This solution addresses authentication algorithm selection between EEC and EES.

This solution based on TLS authentication protocols introduces no extra impact to network entities and existing procedures.

***********End of Second Change***********