3GPP TSG-SA3 Meeting #107-e 	S3-220939

# e-meeting, 16-20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1388 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarifications to secondary authentication during UE onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) Annex I.9.2.4.1 currently states that secondary authentication with DCS uses Default UE credentials. However, default UE credentials are credentials for primary authentication, not for secondary authentication. 2) Annex I.9.2.4.2 currently states that, during onboarding, the ON-SNPN may trigger secondary authentication procedure with a DN-AAA server. However, during onboarding, the UE will not be configured with a DN and related credentials to be used with the onboarding network. Specifically, the UE will not know which identifier and/or credentials to use during secondary authentication with a DN-AAA server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) For secondary authentication with DCS, "default PDU session authentication and authorization credentials" are used instead of default UE credentials. 2) Secondary authentication using DN-AAA during onboarding is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Secondary authentication during UE onboarding not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.9.2.4.1, Annex I.9.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### I.9.2.4	Secondary authentication

#### I.9.2.4.1	Secondary authentication using DCS

After successful primary authentication as described in I.9.2.2 (i.e. primary authentication without using DCS), upon the establishment of the Onboarding PDU Session, the ON-SNPN may trigger secondary authentication procedure with the DCS using default PDU session authentication and authorization credentials, as described in clause 11.1.

#### I.9.2.4.2	Void

*** END CHANGES ***