3GPP TSG-SA3 Meeting #107-e 	S3-220970

# e-meeting, 16 - 20 May 2022											Revision of S3-20xxxx

# Source:	Ericsson

# Title:	Remote UE Report in CP based solution

# Document for:	Approval

Agenda Item:	4.7

# 1	Decision/action requested

Remote UE Report in CP based solution

# 2	References

[1]	3GPP TS 33.503 "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)"

[2]	3GPP TS 23.304 "Proximity based Services (ProSe) in the 5G System (5GS)"

[3]	S3-220971   "Discussion on UE ID privacy for Remote UE Report "

# 3	Rationale

According to TS 23.304 [2], in step 7, the 5G ProSe Layer-3 UE-to-Network Relay shall send a Remote UE Report (Remote User ID, Remote UE info) message to the SMF for the PDU Session associated with the relay. The Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE user that was successfully connected in step 4. The Remote UE info is used to assist identifying the 5G ProSe Layer-3 Remote UE in the 5GC. It mentions, the privacy protection for Remote User ID depends on SA WG3 design.

S3-22xxxx [3] discusses how to support privacy protection of Remote User ID. In this release of the specifications, the 5G ProSe UE-to-Network Relay is seen as a trusted entity. PRUK ID is used as a Remote User ID. Whether or not a new privacy protected Remote User ID is needed could be considered in later releases.

This paper proposes that

- Remote User ID is provided by the PAnF of the Remote UE and stored in the 5G ProSe Relay UE for Remote UE Report, i.e. updates in step 13, 14 and 18.

- The 5G ProSe UE-to-network relay included the Remote User ID to the SMF of the relay UE in Remote UE report. The SMF of the relay UE contacts the PAnF of the remote UE and the PAnF of the remote UE performs the mapping or decryption of the Remote User ID to the SUPI, i.e. updates in step 19, 20.

- The PAnF services to support resolving Remote User ID, i.e. updates in clause 7.X.3.

# 4	Detailed proposal

It is proposed that SA3 approve the below draft CR to TS 33.503 [1] which introduces the security aspect for Remote UE Report.

**** FIRST CHANGE ****

#### 6.3.3.3	Security procedure over Control Plane

##### 6.3.3.3.1	General

This subclause describes the security mechanisms for the 5G ProSe Layer-3 UE-to-Network Relay authentication, authorization and key management using the 5G ProSe Remote UE specific authentication for PC5 keys establishment. Network entities AMF, PAnF, AUSF and UDM are involved for key derivation and distribution of keys used for 5G ProSe UE-to-Network Relay communication. The UE shall be provisioned with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [7] clause 4.2.2. PCF shall provision the authorization policy and parameters for 5G ProSe UE-to-Network Relay discovery and communication as specified in 5.1.4 in TS 23.304 [2].

##### 6.3.3.3.2	Connection with 5G ProSe UE-to-Network Relay connection with setup of network Prose security context during PC5 link establishment

This subclause describes a procedure for a 5G ProSe Remote UE to establish a PC5 link between a 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE is authenticated by AUSF via 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-Network Relay's AMF during 5G ProSe PC5 establishment. The mechanism can be used by a 5G ProSe Remote UE while out of coverage.

Figure 6.3.3.3.2-1: UE-to-Network Relay security procedure with setup of network Prose security context during PC5 link establishment

0.	The 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay shall be registered with the network. The 5G ProSe UE-to-Network Relay shall be authenticated and authorized by the network to provide UE-to-Network relay service. The 5G ProSe Remote UE shall be authenticated and authorized by the network to receive UE-to-Network relay service. PC5 security policies are provisioned to the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay respectively during this authorization and information provisioning procedure.

1.	The 5G ProSe Remote UE shall initiate discovery procedure using any of Model A or Model B method as specified in clause 6.3.1.2 or 6.3.1.3 of TS 23.304 [2] respectively.

2.	After the discovery of the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE shall send a Direct Communication Request to the 5G ProSe UE-to-Network Relay for establishing secure PC5 unicast link. The 5G ProSe Remote UE shall include its security capabilities and PC5 security signalling policy in the DCR message as specified in TS 33.536 [6]. The message shall also include Relay Service Code and Nonce_1.

If the 5G ProSe Remote UE does not have a valid 5GPRUK, the 5G ProSe Remote UE shall include SUCI in the DCR to trigger 5G ProSe Remote UE specific authentication and establish a 5GPRUK.

If the 5G ProSe Remote UE already has a valid 5GPRUK, the 5G ProSe Remote UE shall include the 5GPRUK ID in the DCR to indicate that the 5G ProSe Remote UE wants to get relay connectivity using the 5GPRUK.

3. Upon receiving the DCR message, the 5G ProSe UE-to-Network Relay shall send the Relay Key Request to the AMF of the 5G ProSe UE-to-Network Relay, including SUCI or 5GPRUK ID, RSC and Nonce_1 received in the DCR message. The 5G ProSe UE-to-Network Relay shall also include in the message a transaction identifier that identifies the 5G ProSe Remote UE for the subsequent messages over 5G ProSe UE-to-Network Relay's NAS messages and PC5 messages.

4. The AMF of the 5G ProSe UE-to-Network Relay shall verify whether the 5G ProSe UE-to-Network Relay is authorized to provide the UE-to-Network relay service.

If 5GPRUK ID is received from the 5G ProSe UE-to-Network Relay, the AMF of the 5G ProSe UE-to-Network Relay selects the PAnF of the 5G ProSe Remote UE based on 5GPRUK ID and skips the steps 5-10.

5. The AMF of the 5G ProSe UE-to-Network Relay shall select an AUSF based on SUCI and forward the parameters received in Relay Key Request to the AUSF in Nausf_UEAuthentication_ProseAuthenticate Request message which shall contain the 5G ProSe Remote UE’s SUCI, Relay Service Code.

6. The AUSF shall initiate a 5G ProSe Remote UE specific authentication using the ProSe specific parameters received (i.e., RSC, etc.). The serving network name handling is same as defined in TS 33.501 [3]. The security policy negotiation and protection of messages hereafter shall follow the one-to-one security establishment described in clause 6.2.3 of the present document.

The AUSF of the 5G ProSe Remote UE shall retrieve the Authentication Vectors from the UDM via Nudm_UEAuthentication_GetProseAv Request message and trigger authentication of the 5G ProSe Remote UE. This authentication is performed between the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE via the AMF of the 5G ProSe UE-to-Network Relay and the 5G ProSe UE-to-Network Relay. Based on SUPI, the UDM shall choose the authentication method.

7a. If EAP-AKA' is selected by UDM, the AUSF of the 5G ProSe Remote UE shall trigger authentication of the 5G ProSe Remote UE based on EAP-AKA'. The AUSF of the 5G ProSe Remote UE generates the EAP-Request/AKA'-Challenge message defined in clause 6.1.3.1 of TS 33.501 and send EAP-Request/AKA'-Challenge message to the AMF of the 5G ProSe UE-to-Network Relay in a Nausf_UEAuthentication_ProseAuthenticate Response message.

7b. The AMF of the 5G ProSe UE-to-Network Relay shall forward the Relay Authentication Request (including the EAP-Request/AKA'-Challenge) to the 5G ProSe UE-to-Network Relay over NAS message, including transaction identifier of the 5G ProSe Remote UE in the message. The NAS message is protected using the NAS security context created for the 5G ProSe UE-to-Network Relay.

7c. Based on the transaction identifier, the 5G ProSe UE-to-Network Relay shall forwards the EAP-Request/AKA'-Challenge to the 5G ProSe Remote UE over PC5 messages.

The USIM in the 5G ProSe Remote UE verifies the freshness of the received values by checking whether AUTN can be accepted as described in TS 33.102 [11].

For EAP-AKA', the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. The ME shall derive CK' and IK' according to Annex A.3 in TS 33.501.

7d. The 5G ProSe Remote UE shall return EAP-Response/AKA'-Challenge to the 5G ProSe UE-to-Network Relay over PC5 messages.

7e. The 5G ProSe UE-to-Network Relay forwards the EAP-Response/AKA'-Challenge together with the transaction identifier of the 5G ProSe Remote UE to the AMF of the 5G ProSe UE-to-Network Relay in a NAS message Relay Authentication Response.

7f. The AMF of the 5G ProSe UE-to-Network Relay forwards EAP-Response/AKA'-Challenge to the AUSF of the 5G ProSe Remote UE via Nausf_UEAuthentication_ProseAuthenticate Request.

The AUSF of the 5G ProSe Remote UE performs the UE authentication by verifying the received information as described in TS33.501.

For EAP-AKA’, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE may exchange EAP-Request/AKA’-Notification and EAP-Response /AKA’-Notification messages via the AMF of the 5G ProSe UE-to-Network Relay. After the exchanges, the AUSF of the 5G ProSe Remote UE derives KAUSF_P without calculating the KSEAF.

The AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE shall derive a new KAUSF_P (different from KAUSF). NAS SMC procedure is not performed between 5G ProSe Remote UE and AMF of the 5G ProSe UE-to-Network Relay.

8.	On successful authentication, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE shall generate 5GPRUK as specified in Annex A.2 and 5GPRUK ID as specified in Annex A.3 using the newly derived KAUSF_P.

9a.	 The AUSF of the 5G ProSe Remote UE shall select the PAnF and send the SUPI, 5GPRUK, 5GPRUK ID, and RSC in Npanf_ProseKey_Register Request message to the PAnF.

9b.	The PAnF shall store the Prose context info (i.e., SUPI, 5GPRUK, 5GPRUK ID, RSC) for the 5G ProSe Remote UE and send Npanf_ProseKey_Register Response message to the AUSF.

10.	The AUSF of the 5G ProSe Remote UE shall send the Authentication result, 5GPRUK ID if the authentication is successful in Nausf_UEAuthentication_ProseAuthenticate Response message to the AMF of the 5G ProSe UE-to-Network Relay.

11. The AMF of the 5G ProSe UE-to-Network Relay shall select the PAnF of the 5G ProSe Remote UE, based on 5GPRUK ID and forwards the 5GPRUK ID, Nonce_1 and RSC in the Relay Key Request to the PAnF of the 5G ProSe Remote UE in Npanf_ProseKey_Get Request message.

12. When 5GPRUK ID, Nonce_1 and RSC are received from the AMF of the 5G ProSe UE-to-Network Relay, the PAnF of the 5G ProSe Remote UE discovers the 5G PRUK stored locally for the 5G ProSe Remote UE. The PAnF of the 5G ProSe Remote UE shall generate Nonce_2 and the KNR_ProSe key as defined in Annex A.4.

13. The PAnF of the 5G ProSe Remote UE shall send the KNR_ProSe key and Nonce_2 in Npanf_ProseKey_Get Response message to the AMF of the 5G ProSe UE-to-Network Relay.

The PAnF shall also provide in the response message to the AMF of the 5G ProSe UE-to-Network Relay a Remote User ID of the 5G ProSe Remote UE, which is used in Remote UE Report,

NOTE x: 	In general, SUPI should not be sent outside of the operator network to the 5G ProSe UE-to-Network Relay in order to protect user privacy. The 5G ProSe UE-to-Network Relay cannot be regarded as a network entity in the traditional sense e.g. as an gNB. The PAnF shall send a privacy protected Remote User ID to the 5G ProSe UE-to-Network Relay.

In this release of specification, the 5G ProSe UE-to-Network Relay is seen as a trusted entity. 5GPRUK ID is used as Remote User ID.

14. When receiving a KNR_ProSe from the PAnF of the 5G ProSe Remote UE, the AMF of the 5G ProSe UE-to-Network Relay shall not attempt to trigger NAS SMC procedure with the 5G ProSe Remote UE. The AMF of the 5G ProSe UE-to-Network Relay shall send KNR_ProSe , Nonce_2 and Remote User ID in Relay Key Response to the 5G ProSe UE-to-Network Relay.

15. The 5G ProSe UE-to-Network Relay derives PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe, as defined in clause 6.3.3.3.3 of this document. KNR_ProSe ID and Krelay-sess ID are established in the same way as KNRP ID and KNRP-sess ID in TS 33.536 [6].

The 5G ProSe UE-to-Network Relay shall send the received Nonce_2 to the 5G ProSe Remote UE in Direct Security mode command message, which is protected using Krelay-int or/and Krelay-enc derived from Krelay-sess according to the negotiated PC5 signalling policies between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

16.	The 5G ProSe Remote UE shall generate the KNR_ProSe key to be used for Remote access via the 5G ProSe UE-to-Network Relay in the same way as defined in step 12. The 5G ProSe Remote UE shall derive PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe the same way as defined in step 15.

17. The 5G ProSe Remote UE shall send the Direct Security Mode Complete message containing its PC5 user plane security policies to the 5G ProSe UE-to-Network Relay, which is protected by Krelay-int or/and Krelay-enc derived from Krelay-sess according to the negotiated PC5 signalling policies between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

18. After the successful verification of the Direct Security Mode complete message, the 5G ProSe UE-to-Network Relay responds a Direct Communication Accept message to the 5G ProSe Remote UE to finish the PC5 connection establishment procedures. The 5G ProSe UE-to-Network Relay UE shall store the Remote User ID received in step14.

19. When the conditions to send a Remote UE Report reach as specified in TS 23.304 [2], the 5G ProSe Layer-3 UE-to-Network Relay shall send a Remote UE Report (Remote User ID, Remote UE info) message to the SMF of the 5G ProSe UE-to-Network Relay.

20. If the mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI is not available in the SMF of the 5G ProSe UE-to-Network Relay, the SMF of the 5G ProSe UE-to-Network Relay shall discover the PAnF of the 5G ProSe Remote UE based on the Remote User ID and sends a Resolve Remote User ID request towards the PAnF in Npanf_ResolveRemoteUserId_Get Request message, including the Remote User ID of the 5G ProSe Remote UE in the message.

The PAnF of the 5G ProSe Remote UE shall verify the Remote User ID is valid and the SMF of the 5G ProSe UE-to-Network Relay is authorized to get the SUPI of the 5G ProSe Remote UE.

The PAnF of the 5G ProSe Remote UE shall send a Resolve Remote User ID response to the SMF of the 5G ProSe UE-to-Network Relay in Npanf_ResolveRemoteUserId_Get Response message, including the SUPI of the 5G ProSe Remote UE in the message.

The SMF of the 5G ProSe UE-to-Network Relay shall store the Remote User ID, the SUPI of the 5G ProSe Remote UE and the Remote UE info in the 5G ProSe Layer-3 UE-to-Network Relay's SM context for this PDU Session associated with the Relay.

Further communication between the 5G ProSe Remote UE and the Network takes place securely via the 5G ProSe UE-to-Network Relay.

**** NEXT OF CHANGE ****

## 7.X.	Prose Anchor Function Services

### 7.X.1	General

The Prose Anchor Function (PAnF) supports providing storage for the Prose context info (i.e., SUPI, 5GPRUK, 5GPRUK ID, RSC) for a 5G ProSe Remote UE and generation of KNR_ProSe used by 5G ProSe UE-to-Network Relay for the security establishment of the PC5 link with 5G ProSe Remote UE. The PAnF also provides Remote User ID of a 5G ProSe Remote UE to be used in Remote UE Report and supports resovling Remote User ID to SUPI.

The following table shows the PAnF Services and the PAnF Service Operations.

Table 7.X.1-1: List of PAnF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Npanf_ProseKey | Npanf_ProseKey_Register | Request/Response | AUSF |
|  | Npanf_ProseKey_Get | Request/Response | AMF |
| Npanf_ResolveRemoteUserId | Npanf_ResolveRemoteUserId_Get | Request/Response | SMF |

### 7.X.2	Npanf_ProseKey service

##### 7.X.2.1 	Npanf_ProseKey_Register service operation

Service operation name: Npanf_ProseKey_Register

Description: The NF consumer requests the PAnF to store the Prose context info (i.e., SUPI, 5GPRUK, 5GPRUK ID, RSC).

Input, Required: SUPI, 5G PRUK ID, 5GPRUK, Relay Service Code.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

##### 7.X.2.2 	Npanf_ProseKey_Get service operation

Service operation name: Npanf_ProseKey_Get

Description: The NF consumer requests Prose Key from the PAnF.

Input, Required: 5GPRUK ID, Relay Service Code, Nonce_1.

Input, Optional: None.

Output, Required: KNR_ProSe and Nonce_2.

Output, Optional: None.

### 7.X.3	Npanf_ResolveRemoteUserId service

##### 7.X.3.1 	Npanf_ResolveRemoteUserId_Get service operation

Service operation name: Npanf_ResolveRemoteUserId_Get

Description: The NF consumer requests the PAnF to resolve the Remote User ID.

Input, Required: Remote User ID.

Input, Optional: None.

Output, Required: SUPI.

Output, Optional: None.

**** END OF CHANGE ****