3GPP TSG-SA3 Meeting #107-e 	S3-220993

# e-meeting, 16 - 20 May 2022																revision of S3-220319

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.220 | CR | 0215 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding a new Ua security protocol identifier for TLS 1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCryptPr |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Unlike PSK TLS 1.2 (where the Ua security protocol identifier is depenedent on the chosen ciphersuite), TLS 1.3 requires the use of the TLS master key in the ClientHello message. Hence a Ua security protocol identifier that is fixed and not dependent on the ciphersuite is needed as ciphersuite is not known when the ClientHello is sent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add such a different Ua security protocol |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TLS 1.3 with GBA keys will not work |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 has specific clause number added to better link the Ua security protocol identifier with the description of the Ua security protocol |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

# H.3	Ua security protocol identifiers for 3GPP specified protocols

The following Ua security protocol identifiers are specified by 3GPP:

( 0x01,0x00,0x00,0x00,0x00 )		Ua security protocol according to TS 33.221 [5].

( 0x01,0x00,0x00,0x00,0x01 )		Ua security protocols according to TS 33.246 [26].

NOTE 1:	TS 33.246 [26] provides key separation between the keys that are used within HTTP digest and MIKEY protocols.

( 0x01,0x00,0x00,0x00,0x02) 	Ua security protocol HTTP digest authentication according to TS 24.109 [29], unless HTTP digest authentication is used in the context of another Ua security protocol, which is already covered elsewhere in this Annex.

( 0x01,0x00,0x00,0x00,0x03 )	Ua security protocols used with HTTP-based security procedures for MBMS user services according to TS 26.237 [38].

( 0x01,0x00,0x00,0x00,0x04 )	Ua security protocols used with SIP-based security procedures for MBMS user services according to TS 26.237 [38].

( 0x01,0x00,0x00,0x00,0x05 )	Ua security protocols used with Generic Push Layer according to TS 33.224 [39], unless Generic Push Layer is used in the context of another Ua security protocol, which is already covered elsewhere in this Annex.

( 0x01,0x00,0x00,0x00,0x06 )	Ua security protocol for IMS UE to KMS  http based message exchanges according  to "IMS media plane security", TS 33.328 [40]

( 0x01,0x00,0x00,0x00,0x07 )	Ua security protocol for shared key TLS 1.3 given in clause 5.4.0.2 of TS 33.222 [25].

( 0x01,0x00,0x00, 0x01,0x00 ) 		Generation of TMPI according to Annex B.4.

NOTE 2:	This protocol identifier is not strictly a Ua protocol identifier, but its use in key derivation function is exactly equal.to a Ua protocol identifier.

( 0x01,0x00,0x01,yy,zz )	Ua security protocol for "Shared key-based UE authentication with certificate-based NAF authentication", according to TS 33.222 [25] section 5.3, or "Shared key-based mutual authentication between UE and NAF" for TLS 1.2 (see above for Ua security protocol identifier for TLS 1.3 with shared keys), according to TS 33.222 [25] section 5.4.0.1. Here, "yy,zz" is the protection mechanism CipherSuite code according to the defined values for TLS CipherSuites in the IANA TLS Cipher Suite Registry which is referenced in RFC 8446 [59].

NOTE 3: 	The "Certificate based mutual authentication between UE and NAF” according to TS 33.222 [25] section 5.5 does not require a Ua protocol identifier.

NOTE 4:	As an example: The TLS 1.2 CipherSuite TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 has code = { 0xC0,0x2B  }, thus the according protocol identifier shall be ( 0x01,0x00,0x01, 0xC0,0x2B  ).

( 0x01,0x00,0x02,yy,zz ) 	Ua security protocol for "Shared key-based UE authentication with certificate-based NAF authentication", according to TS 33.222 [25] Annex D. Here, "yy,zz" is the protection mechanism CipherSuite code according to the defined values for TLS CipherSuites in the IANA TLS Cipher Suite Registry which is referenced in RFC 8446 [59]. This Ua security protocol identifier is used for the case outlined in TS 33.222 [5] Annex D, where e.g.  HTML FORM based authentication is used within a TLS tunnel.

NOTE 4: 	The third octet (0x02) distinguish this case from other protocols tunneled inside the TLS tunnel.

**** END OF CHANGES ****