# 3GPP TSG-SA3 Meeting #107-e 																	S3-221018

# e-meeting, 16 - 20 May 2022

# Source:	Ericsson, Deutsche Telekom, Vodafone

# Title:	NSWO security revisited

# Document for:	Endorsement

Agenda Item:	4.12

# 1	Decision/action requested

The NSWO authentication mechanism introduces an unnecessary increased risk of attacks and should be mitigated.  It is proposed to send an LS to CT4 and ask CT4 separate the NSWO and primary authentication procedures on resource URI level and apply corresponding resource level granularity in OAuth authorization.

# 2	References

[1]	3GPP TS 33.501, Security architecture and procedures for 5G system

# 3	Background

## 3.1	Introduction

SA3 has decided on stage 2 level procedures for NSWO authentication, i.e., to re-use the Nausf_UEAuthentication_Authenticate service operation for both primary authentication and NSWO authentication.

This contribution describes how a compromised NSWOF could still launch primary authentication with the AUSF and use this for further attacks towards victim UEs.

This contribution also proposes mitigation for this situation so that the AUSF can prevent NSWOF to trigger primary authentication procedures based on the use of independent resources for NSWO authentication which access could be controlled by the OAuth Authorization framework.

## 3.2	Current NSWO authentication procedure

The NSWO authentication procedure in TS 33.501 [1] Annex S is shown below amended with information from clause 13.4 “Authorization of NF service access” in steps marked with A, B and C and with italics. The amended parts show the authorization related activities between AUSF, NRF and NSWOF.

Authentication procedure for NSWO in 5GS

A1.	TLS connection is established.

A2-4. The AUSF registers with the NRF and provides its profile. The AUSF, as part of its NF profile, may include "scope" or "additional scope" information to enable the NRF to determine whether a given "scope" claim shall be granted to an NF Service Consumer that requested an Oauth2 access token. E.g., a "scope" can be allowed services (Nausf_UE_AU) per NF type (AMF, NSWOF); An "additional scope" can be allowed service operations and resources per NF type.

1. The UE establishes a WLAN connection between the UE and the WLAN Access Network (AN), using procedures specified in IEEE 802.11[80].

2. The WLAN AN sends an EAP Identity/Request to the UE.

3. The UE sends an EAP Response/Identity message.  The UE shall use the SUCI in NAI format (i.e., username@realm format as specified in clause 28.7.3 of TS 23.003[19]) as its identity irrespective of whether SUPI Type configured on the USIM is IMSI or NAI. If the SUPI Type configured on the USIM is IMSI, the UE shall construct the SUCI in NAI format with username containing the encrypted MSIN and the realm part containing the MCC/MNC.

4. The EAP Response/Identity message shall be routed over the SWa interface towards the NSWO NF based on the realm part of the SUCI.

NOTE 1: NSWO NF acts as SBI/AAA proxy between the AUSF and the WLAN Access Network.

B1.	TLS connection is established.

B2. The NSWOF requests an access token from the NRF. The message includes

-	the NF Instance Id(s) of the NF Service Consumer,

-	the requested "scope" including the expected NF Service name (Nausf_UE_AU),

-	NF type of the expected NF Service Producer instance (AUSF)

-	and NF type of the NF Service Consumer (NSWOF).

B3. The NRF may verify that the input parameters (e.g., NF type) in the access token request match with the corresponding ones in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

B4.		If the authorization is successful, the NRF shall send access token to the NF Service Consumer in the Nnrf_AccessToken_Get response.

The claims in the token shall include

-	the NF Instance Id of NRF (issuer: UUID),

-	NF Instance Id of the NF Service Consumer (subject: UUID),

-	NF type of the NF Service Producer (audience: AUSF),

-	expected service name(s) (scope: Nausf_UE_AU),

-	expiration time (expiration)

-	and optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources). Mind however that the Nausf_UEAuthentication Service API as currently defined in TS 29.509 does not define any scopes for Oauth2 authorization; it defines a single scope consisting on the name of the service (i.e., "nausf-auth"), and it does not define any additional scopes at resource or operation level.

C1. TLS connection is established and the NF Service Consumer and NF Service Producer shall authenticate each other following clause 13.3.

5. The NSWO NF shall send the message Nausf_UEAuthentication_Authenticate Request with SUCI, Access Network Identity and NSWO indicator towards the AUSF. NSWO_indicator is used to indicate to the AUSF that the authentication request is for Non-seamless WLAN offload purposes. The NSWO NF shall set the Access Network Identity to "5G:NSWO".

C2.	The NF Service Producer shall verify the token as follows:

-	The NF Service Producer ensures the integrity of the token by verifying the signature using NRF’s public key or checking the MAC value using the shared secret. If integrity check is successful, the NF Service Producer shall verify the claims in the token as follows:

-	It checks that the audience claim in the access token matches its own identity or the type of NF Service Producer (AUSF).

-	If an NF Set ID present, the NF Service Producer shall check the NF Set ID in the claim matches its own NF Set ID.

-	If scope is present, it checks that the scope matches the requested service operation (Nausf_UEAU vs Nausf_UEAU_Authenticate).

- 	If the access token contains "additional scope" information (i.e. allowed resources and allowed actions (service operations) on the resources), it checks that the additional scope matches the requested service operation (Not used in AUSF).

NOTE: In this case, since the Nausf_UEAuthentication Service API as currently defined in TS 29.509 does not define a single scope consisting on the name of the service (i.e., "nausf-auth"), the AUSF is only able to  authorize that the NSWOF can use the Nausf_UEAU service; i.e. any service operation within Nausf_UEAU service and with whaterver combination of possible information elements within the service operations (e.g. NSWO_Indicator).

-	It checks that the access token has not expired by verifying the expiration time in the access token against the current data/time.

-	If the CCA is present in the service request, it may verify the CCA as specified in clause 13.3.8.3 and that the subject claim (i.e., the NF Instance Id of the NF Service Consumer) in the access token matches the subject claim in the CCA.

If the verification is successful, the NF Service Producer shall execute the requested service and responds back to the NF Service Consumer. Otherwise it shall reply based on Oauth 2.0 error response defined in RFC 6749 [43].

6. The AUSF (acting as the EAP authentication server) shall send a Nudm_UEAuthentication_Get Request to the UDM, including SUCI and the Access Network Identity and NSWO indicator.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM shall invoke SIDF. SIDF shall de-conceal SUCI to gain SUPI before UDM can process the request. Based on the NSWO indicator, the UDM/ARPF shall select the EAP-AKA´ authentication method and generate an authentication vector using the Access Network Identity as the KDF input parameter. The UDM shall include the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´) and may include SUPI to AUSF in a Nudm_UEAuthentication_Get Response message.

8. The AUSF shall store XRES for future verification. The AUSF shall send the EAP-Request/AKA'-Challenge message to the NSWO NF in a Nausf_UEAuthentication_Authenticate Response message.

NOTE:	The Access Network Identity is carried in the AT_KDF_INPUT attribute in EAP-AKA' as defined in RFC 5448 [12].

9. The NSWO NF shall send the EAP-Request/AKA'-Challenge message to the WLAN AN over the SWa interface.

10. The WLAN AN forwards the EAP-Request/AKA'-Challenge message to the UE.

11. At receipt of the RAND and AUTN in the EAP-Request/AKA'-Challenge message, the ME shall obtain the Access Network Identity from the EAP signalling and the USIM in the UE shall verify the freshness of the AV' by checking whether AUTN can be accepted as described in TS 33.102 [40]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. The ME shall derive CK' and IK' using the Access Network Identity as the KDF input parameter. If the verification of the AUTN fails on the USIM, then the USIM and ME shall proceed as described in sub-clause 6.1.3.3. The UE may derive MSK from CK’ and IK’ as per Annex F and as described in RFC 5448[12]. When the UE is performing NSWO authentication, the KAUSF shall not be generated by the UE.

12. The UE shall send the EAP-Response/AKA'-Challenge message to the WLAN AN.

13. The WLAN AN forwards the EAP-Response/AKA'-Challenge message over the SWa interface to the NSWO NF.

14. The NSWO NF shall send the Nausf_UEAuthentication_Authenticate Request with EAP-Response/AKA'-Challenge message to AUSF.

15. The AUSF shall verify if the received response RES matches the stored and expected response XRES. If the AUSF has successfully verified, it continues as follows to step 16, otherwise it returns an error to the NSWO NF. The AUSF shall derive the required MSK key from CK’ and IK’ as per Annex F and as described in RFC 5448[12], based on the NSWO indicator received in step 5. The AUSF shall not generate the KAUSF.

16. The AUSF shall send Nausf_UEAuthentication_Authenticate Response message with EAP-Success and MSK key to NSWO NF. The AUSF may optionally provide the SUPI to NSWO NF. The AUSF/UDM shall not perform the linking increased home control to subsequent procedures (as stated in present document clause 6.1.4).

17. The NSWO NF shall send the EAP-success and MSK to WLAN AN over the SWa interface. The EAP-Success message is forwarded from WLAN AN to the UE.

18. Upon receiving the EAP-Success message, the UE derives the MSK as specified in step 11, if it has not derived the MSK earlier. The UE uses MSK to perform 4-way handshake to establish a secure connection with the WLAN AN.

# 4	Problem

## 4.1	The attack

In the following we will describe how a compromised NSWOF can start primary authentication with the AUSF for a victim UE. This is possible since the same AUSF service operation and exactly the same authorization token can be used for both primary authentication and NSWO authentication. In particular, all the information which tells the AUSF to perform NSWO authentication instead of primary authentication is outside of the authorization token and can be left out or modified by the NSWOF without AUSF noticing.

With compromised NSWOF we mean an NSWOF which is a “valid” NF in the mobile network but has become compromised e.g., after its instantiation in the network.

As in clause 3 above, amended information from clause 13.4 “Authorization of NF service access” in steps A, B and C is marked with italics.

Attack on the authentication procedure for NSWO in 5GS

A1.	TLS connection is established. This step is the same as the corresponding step in clause 3 above.

A2-4. The AUSF registers with the NRF and provides its profile. This step is the same as the corresponding step in clause 3 above.

1 - 2. The UE establishes a connection with an AN node. The AN node is assumed co-operating with a compromined NSWOF and lure the victim UE to connect and send its SUCI.

NOTE: The type of the AN node and more detailed actions between the UE and AN node are not relevant for the purposes of showing the feasibility of the attack. The AN node could be e.g., a false base station or a false WLAN AP.

3. The UE sends SUCI to the AN node.

4. The AN node sends the SUCI to the compromised NSWOF.

B1.	TLS connection is established. This step is the same as the corresponding step in clause 3 above.

B2. The NSWOF requests an access token from the NRF. This step is the same as the corresponding step in clause 3 above.

B3. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s). This step is the same as the corresponding step in clause 3 above. The check will be successful since the NSWOF is allowed to request this service (i.e., Nausf_UE_AU).

B4.	 If the authorization is successful, the NRF sends access token to the NF Service Consumer in the Nnrf_AccessToken_Get response. This step is the same as the corresponding step in clause 3 above.

C1. TLS connection is established and the NF Service Consumer and NF Service Producer shall authenticate each other following clause 13.3. This step is the same as the corresponding step in clause 3 above.

5. The NSWOF sends the message Nausf_UEAuthentication_Authenticate Request with SUCI, Serving network name with a valid PLMN-ID instead of Access Network Identity including "5G:NSWO" and leaves out the NSWO indicator.

C2.	The NF Service Producer shall verify the token as in the corresponding step in clause 3 above. Token verification will be successful since it is exactly the token with the same scope that would be used for when a NSWOF would be requesting NSWO authentication.

6. The AUSF considers this to be a primary authentication and sends a Nudm_UEAuthentication_Get Request to the UDM, including SUCI and the Serving network name with a valid PLMN-ID.

NOTE: The AUSF does not send an Access Network Identity nor an NSWO indicator since the AUSF assumes this is primary authentication.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM considers this to be a primary authentication and invokes SIDF. SIDF de-conceals SUCI to gain SUPI before UDM can process the request. Based on the SUPI, the UDM/ARPF selects the EAP-AKA´ authentication method and generates an authentication vector using the Serving network name with a valid PLMN-ID as the KDF input parameter. The UDM includes the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´) and SUPI to AUSF in a Nudm_UEAuthentication_Get Response message.

NOTE: The UDM might also select 5G AKA. This could make the attack unsuccessful if the compromised NSWOF does not support 5G AKA. Nevertheless, the attack could proceed if the UDM selects EAP-AKA’.

8. The AUSF stores XRES for future verification. The AUSF sends the EAP-Request/AKA'-Challenge message to the NSWOF in a Nausf_UEAuthentication_Authenticate Response message assuming it to be an AMF.

9. The NSWOF sends the EAP-Request/AKA'-Challenge message to the AN node.

10. The AN Node forwards the EAP-Request/AKA'-Challenge message to the UE.

11. At receipt of the RAND and AUTN in the EAP-Request/AKA'-Challenge message, the UE behaves as in primary authentication according to clause 6.1.3.1 of TS 33.501.

12. The UE sends the EAP-Response/AKA'-Challenge message to the AN node.

13. The AN node forwards the EAP-Response/AKA'-Challenge message to the NSWOF.

14. The NSWOF sends the Nausf_UEAuthentication_Authenticate Request with EAP-Response/AKA'-Challenge message to AUSF.

15. The AUSF verifies if the received response RES matches the stored and expected response XRES. If the AUSF has successfully verified, it continues as in primary authentication according to clause 6.1.3.1 of TS 33.501. Especially, the AUSF derives Kausf and Kseaf.

16. The AUSF sends Nausf_UEAuthentication_Authenticate Response message with EAP-Success, Kseaf and SUPI to NSWOF.

17. The NSWOF sends the EAP-success and a key to AN node. The EAP-Success message is forwarded from AN node to the UE.

18. Upon receiving the EAP-Success message, the UE behaves as in primary authentication according to clause 6.1.3.1 of TS 33.501 and establishes a secure connection with the AN node.

This demonstrates how a compromised NSWOF can start primary authentication with the AUSF for a victim UE. Furthermore, following the same principle, a compromised NSWOF could even initiate any operation within the Nausf_UEAU service with the AUSF (e.g., Nausf_UEAuthentication_deregister).

## 4.2	Analysis of attack

Can this attack happen?

As shown above, the re-use of the same service operation and especially using identical authorization tokens for both primary authentication and NSWO authentication enables a compromised NSWOF to launch primary authentication with the AUSF.

What are the consequences of the attack?

A compromised NSWOF could use the attack for further attacks towards victim UEs. It is likely that such attacks would need futher steps, e.g., a false base station co-operating with the compromised NSWOF to lure the UE to send its SUCI in a context which the UE believes to be a registration to a real 5G network.

Should it be mitigated?

Nevertheless, this threat in SBA services introduces an unnecessary increased risk of attacks and should be mitigated.  This can be also considered as an unfortunate design which circumvents the use of the OAuth authorization framework.

# 5	Discussion

Mitigating this possible threat can be achieved by using specific authorization tokens for primary authentication and NSWO authentication.

Instead of mitigating the threat on stage 2 it is proposed to mitigate this on stage 3 by using resource URI level separation hence the separated "scope" claims for the two cases. SA3 should send an LS to CT4 and ask CT4 separate the two procedures on resource URI level and apply corresponding resource level granularity in OAuth authorization.

Figure below shows the resource URI structure on the AUSF API from TS 29.509. A new resource URI could be defined for NSWO authentication on the same level as the current /ue-authentications resource URI.

Figure 6.1.3.1-1 from TS 29.509: Resource URI structure of the AUSF API

In some discussions it has been proposed that, in order to identify whether a requesting NF is AMF or NSWOF, the AUSF could check the NF Type on the TLS certificate level or on the HTTP headers. However, these are not reliable ways of checking for the following reasons. NF type is not mandatory in the TLS certificate, and even when present, it is not visible to the AUSF when an SCP is present. NF Type is required to be present in the HTTP headers, but it could also be modified by a compromised NSWOF.

In addition to those drawbacks, such solutions would mean additional implementation logic in the AUSF outside of the OAuth authorization logic, and would therefore mean deviating from the OAuth authorization framework based on use of authorization tokens.

It is therefore understood that enabling the use of the OAuth authorization framework to avoid this vulnerability is the right way forward.

# 6	Detailed proposal

It is proposed to endorse that the described attack can happen and it should be mitigated.

It is proposed to mitigate the threat on stage 3 by using resource URI level separation for the two cases. SA3 should send an LS to CT4 and ask CT4 to separate the primary authentication and NSWO authentication procedures on resource URI level and apply corresponding resource level granularity in OAuth authorization.