3GPP TSG-SA3 Meeting #107-e 	S3-221037

# e-meeting, 16 - 20 May 2022											            		    Revision of S3-22xxxx

# Source:	Xiaomi

# Title:	Discussion on Security for ProSe U2N Relay Communication over User Plane

# Document for:	Endorsement

Agenda Item:	4.7

1	Decision/action requested

This contribution discusses the identified issue in current security procedure for ProSe UE-to-Relay Communication over user plane and propose some changes to the current procedure.

[1]	TS 33.503 v0.3.0: “Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)”

[2]	TS 33.303 v17.0.0: “Proximity-based Services (ProSe); Security aspects”

3	Rationale

According to TS 33.503 [1] clause 6.3.3.2, in the current security procedure for ProSe UE-to-Network relay communication over user plane, the Remote UE sends a PRUK Request message to its 5G PKMF in step #1a for requesting a PRUK, which is then provided to the Remote UE by its 5G PKMF in step #1b. However, it is not clarified how the PRUK is made available. On step #4c, when the 5G PKMF of the Remote UE receives a PRUK Request from the 5G PKMF of the Relay UE, PRUK generation is detailed in four different procedure options, which all use the Ks(_ext)_NAF dereived via GBA Push as the PRUK. Such description of PRUK availability in different steps of the procedure raises the following questions:

- Is there any key generation mechanism for PRUK in step #1b of TS 33.503 [1] clause 6.3.3.2?

  - If yes, how is the PRUK generated in step #1b?

  - If no, how is the PRUK made available in step #1b?

- If we go for Q1) where the PRUK needs to be generated in step #1b, is the mechanism of PRUK generation in step #1b and step #4c exactly the same?

  - If the same, why does the 5G PKMF of the Remote UE not communicate with the UDM/BSF/HSS of the Remote UE in step #1b?

  - If not the same, why and how the PRUK generation is different in step #1b and step #4c?

- If we go for Q2) where the PRUK is already available in the PKMF of Remote UE in step #1b, was the PRUK made available via GBA Push in step #4c during a previous PC5 establishement procedure?

  - If yes, both the Remote UE and its 5G PKMF should have the available PRUK which was synchronously generated at both sides.  Then why does the Remote UE have to send a PRUK request in step #1a?

  - If no, why and how does the 5G PKMF of the Remote UE have an available PRUK while the Remote UE does not have one?

With the above questions, it is deemed that 1b) is not possible given that one key cannot have different generation schemes. Both 2a) and 2b) imply the desynchronization of PRUK at the Remote UE side and the PKMF side, which actually needs to be addressed by a round of key generation. Hence it is observed that 1a) is the issue to be addressed.

Observation 1: The PRUK needs to be generated in step #1b of TS 33.503 [1] clause 6.3.3.2 in the same way as in step #4c, while step #4c may not always be needed. Alternatively, the steps #1a~#1b may not be needed.

In addition, although the procedure in TS 33.503 [1] clause 6.3.3.2 largely reuses the procedure in TS 33.303 [2] clause 6.7.3.2, the definitions of PRUK in both specifications are somehow different:

- The PRUK in TS 33.303 [2] refers to ProSe Relay User Key used by the Remote UE and generated by PKMF of the Relay UE;

- The PRUK in TS 33.503 [1] refers to ProSe Remote User Key used by the Remote UE and generated by its own PKMF.

As per TS 33.303 [2], the Remote UE receives the PRUK from the PKMF of the Relay UE and one PRUK is generated for a specific Remote UE by a particular PKMF. It means that when the Remote UE is connecting with different Relay UEs served by different PKMFs, the Remote UE gets different PRUKs. For LTE U2N relay scenario, it could be deployed simply in a way that one particular PKMF supports one specific type of ProSe relay service (e.g. one PKMF deployed for earthquake warning, another PKMF deployed for fire alarm), hence the PRUKs from the different PKMFs could help the Remote UE to differentiate the various types of ProSe service using PC5 link.

As per TS 33.503 [1], the Remote UE receives the PRUK from its own PKMF and one PRUK is generated for a specific Remote UE. It means that the PRUK in the Remote UE is the root key for PC5 links with all Relay UEs and for all ProSe relay services, i.e. the PRUK is relay service agnostic and can be reused by the Remote UE regardless which Relay UE it is connecting with. In Rel-17 ProSe, only UE-to-Network relay scenario is supported, meaning the Remote UE only connects with one type of Relay UE (U2N Relay UE). In Rel-18 ProSe, UE-to-UE relay scenario will also be supported. Some other Rel-18 features (e.g. Ranging) may also need to reuse ProSe mechanism (e.g. PRUK). Consequently, the Remote UE may possibly be connecting with differnt types of Relay UEs (U2N Relay UE, U2U Relay UE, etc.) simultaneously for different types of services (ProSe U2N, ProSe U2U, Ranging) in new Releases. In such case, the Remote UE using one PRUK over PC5 links with all different Relay UE is suboptimal in term of security isolation between different types of services requesting different security levels. For example, one PRUK generated as a root key for one type of service requiring stricter security protection may be assigned with a shorter validation time than another PRUK generated as a root key for another typer of service requiring looser security proection.

Observation 2: Using one PRUK over PC5 links with all different Relay UE is suboptimal in term of security isolation between different types of services requesting different security levels.

To address the issues in the above observations, two proposals are introduced as follows.

4	Detailed proposal

To address Observation 1, the two proposals as below are requested to be endorsed.

Proposal 1: There should be key generation for PRUK by the PKMF of Remote UE after receiving key request in step #1a in the same way as in step #4c, while triggering of step #4c depends on the availability of the PRUK in the Remote UE and its PKMF during PC5 establishment procedure.

Proposal 2: Alternatively, steps #1a~#1b can be removed.

To address Observation 2, the proposal as below is requested to be endorsed.

Proposal 3: The PRUK should be derived in a proper way to achieve security isolation between different types of service requesting different security levels and in a scalable manner for accommodating more types of services using PC5 link.

Based on the above proposals, it is proposed to approve the corresponding pCRs (S3-221038, S3-221039).