3GPP TSG-SA3 Meeting #107-e 	S3-221069

e-meeting, 16 - 20 May 2022	(revision of S3-yyxxxx)

Source:	Lenovo

Title:	New Study to enable 5G registration via trusted non-3GPP access after NSWO Authentication (FS_5GRTN3)

Document for:	Approval

Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Study to enable 5G registration via trusted non-3GPP access after NSWO Authentication

## Acronym: FS_5GRTN3

## Unique identifier:	TBA

## Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: N/A

# 3	Justification

Non-seamless WLAN offload (NSWO) in Rel-17 is based on the principle that the UE connects to a WLAN network and authenticates with the procedure specified in TS 33.501, i.e. the UE performs EAP-AKA’ authentication with the AUSF via an NSWO NF acting as a AAA proxy. After the successful authentication, the UE is not registered in the 3GPP network at this point in time and has no NAS or user plane connectivity to the 5GC. If the UE wants to perform a 5G registration via this WLAN at a later time, assuming the WLAN broadcasts 5GC connectivity, then the UE must first disconnect from the WLAN and then reconnect to the WLAN using the 5G registration via trusted non-3GPP access procedure (23.502, clause 4.12a.2.2).

SA2 agreed in their last SA2#150e meeting the relevant text for a subsequent trusted Non-3GPP access registration for TS 23.501, clause “5.42	Support of Non-seamless WLAN offload”:

“A UE cannot first connect to a WLAN access network using 5GS credentials and without performing 5GS registration, and then later perform on this WLAN access network 5GS registration using the trusted non-3GPP access procedure without having first to release the WLAN and then establish a new WLAN association per the Registration procedure for trusted non-3GPP access defined in TS 23.202 [2] clause 4.12a.2.2.”

The issue is that disconnecting from the WLAN and then reconnecting to the same WLAN in order to perform 5G registration is not a good behaviour for the UE already has a key for the L2 WLAN connection, which could be re-used. A suitable solution can be analysed and adapted by SA3 so that:

- If a UE connects to a WLAN network using the NSWO authentication; and

- if the UE wants later to perform trusted Non-3GPP access 5G registration via this WLAN;

- then the UE can perform 5G registration via this WLAN, without having to disconnect and then re-connect to the WLAN again.

# 4	Objective

The objective of this study is to:

- Study the security procedure enhancements to allow a UE, connected to a WLAN with NSWO authentication, to perform a trusted non-3GPP access 5G registration via the same WLAN, without having to disconnect and then re-connect to the WLAN again.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33.xxx | Study to enable 5G registration via trusted non-3GPP access after NSWO Authentication | TSG#98 | TSG#99 | Andreas Kunz, akunz@lenovo.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

# 6	Work item Rapporteur(s)

Andreas Kunz, akunz@lenovo.com

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Potential interactions with SA2 for the architectural aspects

# 9	Supporting Individual Members

Supporting IM name

Lenovo