3GPP TSG-SA3 Meeting #107-e 	S3-221073

e-meeting, 16 - 20 May 2022

Title:	[Draft] LS on security protection of RRCResumeRequest message

Response to:	N/A

Release:	Rel-18

Work Item:	FS_5GFBS

Source:	SA3

To:	RAN, RAN2, RAN3

Cc:	N/A

Contact person:	Ivy Guo

Ivy_guo@Apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NONE

# 1	Overall description

SA3 agreed on the solution#17 to protect RRCResumeRequest message in TR 33.809 for R18.

In this solution, when the UE initiates the RRC Resume procedure, the UE should use the whole RRCResumeRequest message, except the ResumeMAC-I/shortResumeMAC-I, as an additional input parameter to the VarResumeMac-Input part in order to calculate ResumeMAC-I/shortResumeMAC-I. The UE should send the calculated ResumeMAC-I/shortResumeMAC-I in the RRCResumeRequest message. The UE and the network negotiate/learn each other's capability/support of using the newer version of ResumeMAC-I/shortResumeMAC-I as below:

-	UE's capability is part of an RRC message (i.e., AS SMComplete).

-	gNB/ng-eNB should indicate its capability of supporting the new version of ResumeMAC-I in  SI message (i.e., SIB1, refer to a closely related feature called useFullResumeID in SIB1).

- 	gNB/ng-eNB should also indicate its capability of supporting the new version of ResumeMAC-I in RRCRelease message with SuspendConfig, which is sent after AS SMC thus protected.

If UE receives both source and target gNB/ng-eNB’s capability of supporting new ResumeMAC-I, UE will include the new new ResumeMAC-I in the ResumeRequest.

The ResumeMac-I verification is done by the source gNB and not the target gNB. Target RAN node would potentially need to provide e.g the entire or partial RRC Resume Request Message to the last serving NG-RAN node in the XnAP RETRIEVE UE CONTEXT REQUEST message.

In summary, SA3 sees the security benefit in this solution, if both the UE and the source/target gNB/ng-eNB support this solution, this solution helps to detect the tempering of the RRCResumeRequest message. RAN2 and RAN3 already confirmed the feasibility of this solution from technical point of view in R2-2109121, R2-2201864, R3-221183, therefore, SA3 agrees to endorse this solution from R18.

# 2	Actions

To RAN, RAN2, RAN3:

ACTION: 	SA3 kindly asks RAN/RAN2/RAN3 to take the above information into consideration and make changes in the corresponding specifications.

# 3	Dates of next TSG SA WG 3 meetings

SA3#107-e-Bis	27 June – 01 July 2022	Electronic meeting (TBC)