3GPP TSG-SA3 Meeting #107-e 	S3-221079

# e-meeting, 16- 20 May 2022

# Source:	Apple

# Title:	Introducing Authentication Proxy to AKMA architecture

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is proposed to add a new key issue in AKMA TR33.737.

# 2	References

Null

# 3	Rationale

Null

# 4	Detailed proposal

****START OF CHANGES ***

## 5.X	Key Issue #X: Introduing Authentication Proxy to AKMA

### 5.X.1	Key issue details

In GBA, An Authentication Proxy (AP) is an HTTP proxy which takes the role of a NAF for the UE. It handles the TLS security relation with the UE and relieves the application server (AS) of this task. Based on GBA the AP can assure the ASs that the request is coming from an authorized subscriber of the MNO. The usefulness of an Authentication Proxy may be to reduce the consumption of authentication vectors and/or to minimize SQN synchronization failures. Also, the AP relieves the AS of security tasks.

AP-AS interfaces: The HTTP protocol is run over the AP-AS reference point. Confidentiality and integrity protection can be provided for the reference point between the AP and the AS using NDS/IP mechanisms as specified in TS 33.210 [13].

Ua reference point: The Ua reference point is standardised in specification TS 33.220 [3] and in clauses 4 and 5 of this specification.

### 5.X.2	Security threats

Introducing AP in AKMA is beneficial where different application servers (or Application Functions in AKMA) reside in the same trust domain or in the same edge node. With the AP, these application servers can rely on the AP to execute AKMA procedures, which is more cost efficient than the case where each application servers execute AKMA procedures separately.

### 5.X.3	Potential security requirements

AKMA architecture shall support Authentication Proxy to perform the AF functionality reside between UE and AS(es).

****END OF CHANGES ***