3GPP TSG-SA3 Meeting #107-e 	S3-221090

# e-meeting, 16 – 20 May 2022                                              			Revision of S3-22xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | draft-CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New threat for AUSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 23.122, if the UE detects that the VPLMN altered or removed the steering of roaming information then the UE shall consider the current VPLMN as the lowest priority PLMN and perform PLMN selection. However, the SoR can fail even if the vPLMN have not tampered the SoR information in case the Kausf in UE and AUSF are not same. The aforementioned issue has been resolved in Rel-17 with the restriction on AUSF that it should replace the old Kausf only after successful authentication. Thus, it is proposed to add the new threat of Kausf mismatch in Rel-18 . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new threat for Kausf sychronization in AUSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threat evaluation is missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************.

F.2.2	Threats related to authentication procedures

F.2.2.X	Kausf handling in AUSF

- Threat name: mismatch of Kausf

- Threat Category: Denial of Service

- Threat Description: If the Kausf in the UE and AUSF are not the same, the subsequent SoR procedures will fail. Hence, UE will consider the current serving PLMN as the lowest priority PLMN. As a result, the service of the UE will be interrupted and UE will perform unnecessary PLMN selection.

- Threatened Asset: Sufficient Processing Capacity

*************** End of 1st Change ****************