3GPP TSG-SA3 Meeting #107-e 	S3-221120

# e-meeting, 16 - 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | 0003 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on selected EDGE authentication method indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA3#106e, authentication and authorization method between EEC and ECS/EES was agreed for normative. It was decided that authentication between EEC and ECS/EES, TLS authentication methods shall be used and details of TLS authentication methods (e.g., client certificate, AKMA, GBA) are out of scope of the current document. However, there remains an ambiguity and the specification lack the information on indication of the selected authentication method to the EEC. Failing which EEC may not be able to intiate the security procedure for authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that: - The selected TLS authentication method shall be indicated to the EEC as part of ECS configuration information. Details of providing ECS configuration information to the EEC are specified in TS 23.558 [5]. - The selected TLS authentication method for EES shall be indicated to the EEC by the ECS. The ECS shall include the selected TLS authentication method for the EES in the service response message to the EEC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification regarding indication of the selected authentication method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of changes ****

## 6.2	Authentication and Authorization between EEC and ECS

The ECS shall be configured with the information of authorization methods (token-based authorization or local authorization) used by EESes.

For authentication between EEC and ECS, TLS authentication methods shall be used. Details of TLS authentication methods (e.g., client certificate, AKMA [11], GBA [12]) are out of scope of the current document. If the EEC sends the GPSI to the ECS, then the ECS shall also authenticate the GPSI. The details of how to authenticate the GPSI is out of scope of the current document. The selected TLS authentication method shall be indicated to the EEC as part of ECS configuration information. Details of providing ECS configuration information to the EEC are specified in TS 23.558 [5].

After successful authentication, the ECS shall authorize the EEC by its local authorization policy.

After successful authentication and authorization, the ECS decides whether access tokens are required for the candidate EESes using the configuration information and issues separate EES access tokens to be used for each candidate EESes that use token-based authorization. The claims of the EES service tokens shall include the ECS FQDN (issuer), EEC ID (subject), expected EES service name(s) (Scope), EES FQDN (audience), expiration time (expiration). The ECS sends the service response back to the EEC, which may include EES access token(s).

## 6.3	Authentication and Authorization between EEC and EES

For authentication between EEC and EES, TLS authentication methods shall be used. Details of TLS authentication methods (e.g., client certificate, AKMA [11], GBA [12]) are out of scope of the current document. If the EEC sends the GPSI to the EES, then the EES shall also authenticate the GPSI. The selected TLS authentication method for EES shall be indicated to the EEC by the ECS. The ECS shall include the selected TLS authentication method for the EES in the service response message to the EEC. The details of how to authenticate the GPSI is out of scope of the current document.

For authorization of EEC by the EES, the EEC shall send the access token, if received from the ECS, to the EES and the EES shall authorize the EEC by using the token. Otherwise, the ECS shall authorize the EEC by its local authorization policy.

After successful authentication and authorization, the EES processes the request and sends the service response back to the EEC.

**** End of changes ****