- 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222529-r1

- e-meeting, 10th – 14th October, 2022

- Source:	Huawei, Hisilicon, InterDigital, Ericsson

- Title:	New KI, solution and conclusion on Authorization between EESes

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new KI, solution and conclusion on Authorization between EESes.

# 2	References

NA

# 3	Rationale

This pCR proposes to add a new KI, solution and conclusion on Authorization between EESes.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.739.

***BEGIN OF First CHANGE***

### 5.2.X		Key issue #2.X: New KI on authorization between EESes

#### 5.2.X.1 	Key issue details

According toTR 23.558 [11], the EDGE-9 reference point enables interactions between the Edge Enabler Servers (EES).

EDGE-9 supports:

a)	Discovery of T-EAS information to support Application Context Relocation (ACR);

b)	EEC context relocation procedures; and

c)	Transparent transfer of the application context during Edge Enabler layer Managed ACR.

In the situations such as UE mobility, overload control, or maintenance, different EESs can be more suitable for serving the ACs in the UE. Such mobility transitions result in replacing the source the EES (S-EES) with a target EES (T-EES). Replacing the S-EES with the T-EES requires a procedure named Application Context Relocation (ACR).

TS 33.558 [4] clause 5.1.2 states that "confidentiality, integrity, and replay protection shall be supported on the EDGE-1-4 and EDGE 6-9 interfaces". In addition, for the interfaces EDGE-3/6/9, "the EAS, EES and ECS shall support TLS and HTTPS". However, how these EESes authorize each other was not clearly defined.

Therefore, it is proposed to study authorization between two EESes.

#### 5.2.X.2	Threats

Without authorization, a malicious EES may be able to register with the ECS, further expose the wrong information, such as the T-EAS or the EEC context, to other EESes.

#### 5.2.X.3	Potential security requirements

Authorization between EESes on EDGE-9 should be supported.

***Next CHANGE***

## 6.Y		Solution #Y: Using local policy on authorization between EESes

### 6.Y.1	Solution overview

This solution addresses KI#2.X on the authorization between EESes.

### 6.Y.2	Solution details

For the EDGE-9 interface between EESes, it is proposed to base authorization on local policies described in TS 33.501 [7] clause 13.3.0.

### 6.Y.3	Solution evaluation

This solution addresses the requirements of KI#2.X on the authorization between EESes.

Local policy could be reused here for the authorization between EESes. No further impact is identified on the EES.

***Next CHANGE***

## 7.X	Conclusions for Key Issue #2.X

Solution #Y reuses the static authorization mechanism to solve key issue #2.X is endorsed for normative work. Since local policy is already specified in TS 33.501 [7], only minor clarification may be needed to refer to it in TS 33.558 [7].

*** END OF second CHANGE ***