- 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222534-r1

- e-meeting, 10th – 14th October, 2022

- Source:	Huawei, Hisilicon

- Title:	Resolving ENs in Sol#12

- Document for:	Approval

Agenda Item:	5.24

# 1	Decision/action requested

This contribution proposes to resolve ENs in Sol#12.

# 2	References

NA

# 3	Rationale

This pCR proposes to resolve ENs in Sol#12, since the solution proposed for the involved NFs authorization applies to the authorization performed by the NFp, which means that the notification target reselection of the NFc is irrepespective of this solution. If the authorization is allowed by the NFp, then the corresponding notification will be sent to the potential NFc according. Hence, the EN can be removed.

Futhermore, evaluation is added.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.875.

***BEGIN OF First CHANGE***

## 6.12	Solution #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios

### 6.12.1	Introduction

This solution addresses key issue #3: Service access authorization in the “Subscribe-Notify” scenarios by including “notification URI” in the token request.

This solution proposes to include “notification URI” and CCA of the NF which owns the “notification URI” at the access token request. The NRF verifies whether the notification URI is authorized to receive the service/notification that is requested in the access token request.

Editor’s Note: It is ffs how the solution works in case of notification target reselection as described in clause 6.3.1.0 of TS 23.501.

### 6.12.2 	Solution details

During access token request process, NF Service Consumer include "notification URI" in Nnrf_Access Token_Get Request when operation semantics of the requested service type is "Subscribe/Notify". NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. In addition, when "notification URI"(s) are included in the access token request, NRF may verify whether the "notification URI"(s) locate its location(s) with the same address(es) which are indicated by its own FQDN or IP address as registered in NRF. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

When the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims which may include “notification URI” as requested by NF Service Consumer.

Figure 6.12.2.-1: Access token request for "Subscribe-Notify" NF Service illustration 1

A NF Service Consumer (NF_A) may request to subscribe to NF Service offered by a NF Service Producer(NF_B) on behalf of NF_C according to the Service Request from NF_C (e.g., Figure 4.15.3.2.2-1: Nudm_EventExposure_Subscribe, Unsubscribe and Notify operation in TS23.502).

The Figure 6.12.2-2 describe the solution to verify the service request from NF_A including Notification URI of NF_C.

Figure 6.12.2-2: Access token request for "Subscribe-Notify" NF Service illustration 2

Step 0.	The NF_C sends a Service Request to the NF_A for notification service on an event with an access token including notification URI and CCA of the NF_C.

Step 1.	The NF_A decides to subscribe a service of the NF_B associated to the Service Request received at step 0 and sends an Access Token Request to the NRF for a Service Request toward the NF_B including notification URI and CCA of the NF_C.

Step 2.	The NRF verifies whether the NF_A is authorized and whether the NF_C identified by the appended CCA is a valid Network Function and authorized to receive the requested NF Service from the NF_B. NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF_C of the CCA of the NF_C, or those in the NF profile of the NF_C.

Step 3.	Upon successful verification, the NRF publishes an access token for a Service Request of the NF_A toward the NF_B. The access token includes notification URI of the NF_C in the claims as requested in step 2.

Step 4.	The NF_A sends a Service Request for a notification service toward NF_B including the access token received at step 3 and CCA of the NF_A.

Step 5.	The NF_B verifies the access token and CCA of the NF_A. During the verification, the NF_B can check whether notification URI in the Service Request are included in the access token. If the verification is successful, the NF_B may respond the NF_A and provide the NF_C with the notification service as requested by the NF_A.

6.12.3 	Evaluation

The solution fulfils the potential security requirements from Key Issue #3 on the notification URI granularity.

The solution extends the existing token-based authorization method by including the notification URI of the NF_C in the token claims.

The solution has impact on the access token generation by adding the notification URI of the NF_C into the access token, and token verification.

Editor’s Note: Further evaluation is FFS.

*** END OF second CHANGE ***