3GPP TSG-SA3 Meeting #108e-AdHoc   	draft_S3-222595-r1

- e-meeting, 10 - 14 October 2022

- Source:	InterDigital

- Title:	New Solution for E2E Authentication with Layer-3 UE-to-UE Relay

Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add a new solution addressing Key Issue #1, #2, #3.

# 2	References

[1]	3GPP TR 33.740 V0.2.0

# 4	Proposal

It is proposed to approve the following new solution for inclusion in TR 33.740 [1]. Note that all text is new.

***	BEGIN OF CHANGES	***

## 6.X	Solution #X: E2E Authentication with Layer-3 UE-to-UE Relay

### 6.X.1	Introduction

This solution is for the 5G ProSe Layer-3 UE-to-UE Relay case. It addresses Key Issue #2: Security of UE-to-UE Relay, Key Issue #3: Authorization in the UE-to-UE Relay Scenario and 2nd requirement of Key Issue #1: Security for UE-to-UE Relay discovery (protection of privacy sensitive information of source and target UE).

TR 23.700-33[2] describes several solutions for Layer-3 based UE-to-UE Relay which are all based on IP routing functionality at the UE-to-UE Relay. As part of the PC5 unicast link establishment procedure, the ProSe 5G UE-to-UE Relay allocates an IP address/prefix to the UE or is informed of the UE's IP address/prefix. The Relay stores the association of the UE's Application layer ID (also called User Info) and UE's IP address/prefix (e.g. into its DNS entries). When a source UE needs to communicate with a target UE via the ProSe 5G UE-to-UE Relay, it sends a request (e.g., DNS query) to the ProSe 5G UE-to-UE Relay, over the unicast link, to obtain the target UE's IP address/prefix (based on Target User Info). The Relay returns the IP address/prefix of the target UE. The source UE sends IP data to the target UE via the PC5 unicast link to UE-to-UE Relay. The UE-to-UE Relay acts as an IP router and forwards the packets to the corresponding PC5 unicast link towards the target UE.

When using the IP based routing, the UE and UE-to-UE Relay may wish to allow E2E communication only with a peer UE that is authenticated and authorized by the UE for both privacy reasons and to prevent unauthorized usage of UE-to-UE Relay resources (e.g., due to unauthorized IP traffic). To enable this, a UE informs the UE-to-UE Relay during the PC5 link establishment procedure whether E2E authentication/authorization with peer UEs is required before allowing communications via the UE-to-UE Relay. The Relay enforces the E2E authentication/authorization between peer UEs accordingly.

### 6.X.2	Solution details

This solution describes how E2E authentication and authorization is enforced prior to allowing user data traffic. Key_Est_Info used in the procedure below is a generic authentication container as defined in TS 33.536 [9]. This container is transparent to the PC5 interface and used to transport different data required for key establishment depending on the authentication method used by the application.

Figure 6.X.2-1: End-to-end Authentication via Layer-3 UE-to-UE Relay

- PC5 unicast links are established between UE1 and the Relay, UE2 and the Relay. During PC5 link establishment, UE1 and/or UE2 informs the Relay that E2E authentication and authorization is required prior to E2E communication, i.e. DNS resolution/E2E IP communication is not allowed before E2E authentication and authorization is successfully run. The Relay stores the E2E authentication and authorization requirement information along with UE1 and/or UE2 user info.

- UE1 sends DNS query to Relay including target UE2 user info, UE1 user info, and Key_Est_Info.

Editor's Notes: whether the E2E authentication and key establishment is triggered by DNS message or other message is to be determined in coordination with SA2.

- Relay determines that E2E authentication and authorization is required based on stored UE1 and UE2 user info and triggers E2E authentication and authorization.

The following steps 4 and 5 may be run multiple times depending on the authentication method used.

- (a) Relay sends a Relayed Auth and Key Establishment Request message to UE2 including UE1 user info, UE2 user info, and Key_Est_Info. (b) UE2 sends a Relayed Auth and Key Establishment Response message to Relay including UE1 user info, UE2 user info, and Key_Est_Info. The final response indicates to the relay the authentication is complete with the corresponding result.

- (a) Relay sends a Relayed Auth and Key Establishment Request message to UE1 including UE2 user info, UE1 user info, and Key_Est_Info. (b) UE1 sends a Relayed Auth and Key Establishment Response message to Relay including UE1 user info, UE2 user info, and Key_Est_Info. The final response indicates to the relay the authentication is complete with the corresponding result.

- Once a final successful authentication result is received by the Relay from UE1 and UE2, Relay replies to the DNS Query message from UE1 by sending a DNS Response message including UE2’s IP address. If the E2E authentication and authorization fails, the Relay sends a DNS Response indicating failure.

### 6.X.3	Evaluation

TBD

***	END OF CHANGES	***