3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222620-r1

- e-meeting, 10 - 14 October 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution for automated certificate management for network slicing

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

It is requested to approve this proposal

# 2	References

# 3	Rationale

In the SA3 meeting #107-e, key issue #9 on Automated Certificate Management for Network Slicing was approved. This solution addresses the security requirements of this key issue, with focus on deployments where different 3rd party slices co-exist and interoperate.

# 4	Detailed proposal

************ START OF CHANGES ************

- 6.y	Solution #Y: Automated Certificate Management for Network Slices

- 6.y.1	Introduction

The solution addresses key issue #9. The solution presents a proposal to enable the automated certificate management for network slices, taking into consideration that certificates might belong to different domains, e.g., in deployments where different 3rd party slices may co-exist and interoperate.

Different domains may require different configurations with respect to certificate management, moreover when those domains belong to 3rd parties. To allow for 3rd party slice tenants / slice service providers (e.g., verticals) to configure the usage of their own CA for the network slices they own, interfaces need to be provided in order to manage the required configurations.

This solution proposes a new function referred to as Network Slice Certificate Orchestration Function (NSCOF) for providing those interfaces between different domains, including authorized 3rd parties, who can own one or more network slices. NSCOF function may enable automated certificate management procedures by orchestrating the communication in the interfaces between the different CAs and sub-CAs of the referred slicing domains, as well as between network slice orchestrator and CAs, in the following scenarios (among others):

- Slice service provider specific root CA for all network slices owned and administrated by that slice service provider. This specific root CA will be used in the certificate management procedures for all network functions of all network slices owned by the slice service provider. Interactions with operator 5G Core shared functions may require cross certification schemas between operator CA and slice service provider CA, or alternatively an initial (manual) out-of-band exchange of public root CA certificates.

- Network slice specific root CA, used by all network functions for that specific network slice, owned and administrated by the slice service provider. The slice service provider may have other network slices using the operator CA.

- Slice service provider specific sub-CA, signed by the operator root CA (or intermediate CA), used for all network slices owned by that slice service provider. The administration of the slice service provider specific sub-CA can be done by the operator, the slice service provider (3rd party) or shared among both entities.

- Network slice specific sub-CA, signed by the operator root CA, used for all network functions of that particular slice. The administration of the slice service provider specific sub-CA can be done by the operator, the slice service provider (3rd party) or shared among both entities.

- 6.y.2	Solution details

The figure 6.y.2-1illustrate an example of orchestration tasks provided by the NSCOF function to enable automated certificate management for Slices, when 3rd parties (administrators and CAs) are involved.

The pre-requisites for this solution proposal are:

- Secure mutual TLS connection is established between the 3rd party certificate administrator and NSCOF function.

- NSCOF has a pre-established trust relationship with Operator’s RA/CA

- Operator’s RA/CA has a pre-established trust relationship with 3rd Party CA

Figure 6.y.2-1: Example of certificate management procedure orchestrated by NSCOF

1) 3rd Party Certificate Admin provides slice-specific root-CA OR sub-CA configurations to NSCOF.

For each NF allocated to applicable slice:

2a) (Optional) In some implementations, after having verified a successful deployment of NFs associated with specific NSSAIs, the Network Slice Orchestrator may request NSCOF for certificates for every of those NFs belonging to specific slices.

2b) (Optional) In some implementations, if NSCOF can identify the NFs associated with the NSSAIs from the information provided by 3rd party certificate administrator, NSCOF may send the Certificate Signing Request to Operator’s CA on behalf of the NFs.

3) NSCOF provides the operator RA/CA with the information related to the (sub-) CA, and if applicable 3rd party CA, configurations according to the NSSAI to which each NF is associated.

4) Operator’s RA/CA executes the certificate signing procedures with the corresponding (sub-) CAs (in the diagram a 3rd party CA is represented as example) as per received configurations.

5) Operator’s RA/CA provides the signed NF certificate to NSCOF

6a) (Optional) In implementations where Step 2a) is used as trigger for NF certificate signing, a response is sent to Network Slice Orchestrator, which takes care of provisioning the signed certificate to the respective NF.

6b) (Optional) In some implementations, NSCOF may provision the signed certificate to the respective NFs.

NOTE:	In some implementations, NSCOF can be an integral part of Network Slice Orchestrator.

- 6.y.3	Evaluation

TBD

************ END OF CHANGES ************