- 3GPP TSG-SA3 Meeting #108Adhoc-e	draft S3-222639-r4

- e-meeting, 10th – 14th October, 2022

- Source:	ZTE

- Title:	New solution about the roaming AKMA architecture of the AF in Data Network

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

This contribution proposes proposes a new solution about the roaming AKMA architecture of the AF in Data Network.

# 2	References

[1] 	3GPP TR33.737 V0.2.0

# 3	Rationale

This new solution proposes the roaming AKMA architecture of the AF in Data Network.

# 4	Detailed proposal

********************Start of 1st Change******************

## 6.X	Solution #X: Roaming AKMA architecture of the AF in Data Network (internet)

### 6.X.1	Introduction

This solution addresses KI#1 (Case 3). This solution is proposed for the cases of function that enables LI deployment when AF located in Data Network(internet).

6.X.2	Solution details

Figure 6.X.1-2 shows the architecture that UE is roaming in a VPLMN and accesses the application deployed in Data Network(internet). The UE accesses the third party deployment application. If the data between the UE and application had not been encrypted, the VPLMN can detect the unencrypted data flow through UPF in the VPLMN. If the data between the UE and application had been encrypted by using an encrpted Ua* protocal, the UPF in the VPLMN has to obtain related the KAF from the AAnF in the HPLMN base on the agreements between  the HPLMN and the VPLMN.. In this case, the function that enables LI could be UPF in the VPLMN..

Editor’s Note: how to obtain Kaf (or LI context) from HPLMN and store it in the VPLMN is FFS.

Figure 6.X.1-2: The architecture that UE is roaming in a VPLMN and accesses the application deployed Data Network (*function that enables LI)

### 6.X.3	Solution Evaluation

TBD

********************End of 1st Change******************