3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222728 merged in draft_S3-222676-r7

- e-meeting, 10 - 14 October 2022

- Source:	US National Security Agency, NIST, CISA ECD, Lenovo, CableLabs, InterDigital, AT&T, Johns Hopkins University APL, CIS

- Title:	ZT Tenet 7

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

SA3 is asked to approve this addition to TR 33.894

# 2	References

[1]	3GPP TR 33.894, ‘Study on applicability of the Zero Trust Security principles in mobile networks

(Release 18)’.

[2]	NIST Special Publication 800-207, ‘Zero Trust Architecture’, August 2020.

# 3	Rationale

This submission contains a gap analysis comparing the NIST zero trust tenets to current 3GPP security mechanisms for the 5G Core. For this analysis, some thought was put into how the tenets specifically apply to a 5G Core since the NIST Special Publication 800-207 Zero Trust Architecture utilizes a user-to-enterprise example for applying the zero trust tenets. Further it is worth noting that, according to [2], ‘The tenets attempt to be technology agnostic.’.

# 4	Detailed proposal

***	BEGINNING OF 1st CHANGE   ***

5.Y	Tenet #7: Data collection to improve security posture

5.Y.1	Description

According to tenet 7 titled, ‘The enterprise collects as much information as possible about the current state of assets, network infrastructure and communications and uses it to improve its security posture’ in [2] describes that, “An enterprise should collect data about asset security posture, network traffic and access requests, process that data, and use any insight gained to improve policy creation and enforcement. This data can also be used to provide context for access requests from subjects (see Section 3.3.1).”.

Security posture data collected from the network should be used for periodic review and improvement of an applied Zero Trust Architecture’s policies.  These improvements include the creation of new policies and enforcement of existing policies.

5.Y.2	Relevant security mechanisms

There are currently no standard procedures for data collection to improve overall core network security posture.

5.Y.3	Evaluation

Tenet 7 is an overall directive to network operators to:

- facilitate data collection related to security posture, control plane network traffic (i.e., message exchanges between NFs) and access requests,

- processing of data (based on operator specific implementation), and

- use any insight gained to improve policy creation and enforcement (based on operator policies) in the 5GC.

Based on this observation, the operator network can consider standard mechanisms to support data collection to enable improvements in security posture.

***	END OF 1st CHANGE   ***