3GPP TSG-SA3 Meeting #108Adhoc-e 	S3-222711

- e-meeting, 10th – 14th October 2022                                                               Revision of S3-22xxxx

- Source:	Nokia, Nokia Shanghai Bell, Lenovo, Apple, CableLabs

- Title:	New KI on TNAP mobility

- Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is requested to approve the new KI

# 2	References

[1]	3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

# 3	Rationale

Please refer to the Discussion paper S3-XXX and updated SID S3- XXX proposal.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.x	Key issue #X: Security aspect of TNAP mobility

### 5.x.1	Key issue details

Mobility between two TNAPs within the same trusted Non-3GPP Access Network Gateway Function(TNGF) is not supported in 3GPP currently. For example, when UE moves between two nearby or overlapping TNAP1 and TNAP2, the connectivity will break. Therefore, UE services will be interrupted. The UE needs to reconnect, go through another authentication procedure to continue the service even though the two non-3GPP access connect to the same 5GC.

There could be some potential security solutions where UE switches the TNAP1 to TNAP2 without breaking the connectivity. However, the security aspects of optimizations of inter-TNAP mobility were never studied in SA3.

### 5.x.2	Threats

TBD.

### 5.x.3	Potential security requirements

UE shall be able to switch from one TNAP to another TNAP within the same TNGF without performing a full authentication.

While switching from one TNAP to another TNAP within the same TNGF, the interface between UE and the new TNAP shall be confidentiality, integrity, and replay protected.

**** END OF CHANGE ****