- 3GPP TSG-SA3 Meeting #108Adhoc-e	draft_S3-222723-r1

- e-meeting, October 10-14, 2022	revision of S3-222723

- Source:	CATT

- Title:	pCR to TR33.740 Centralized discovery key management and U2U relay authorization

Agenda Item:	5.3

# 1	Decision/action requested

Approve the pCR below

# 2	References

# 3		Rationale

A proposal for discovery key management and U2U relay authorization.

# 4	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.Y	Solution #Y: Centralized discovery key management and U2U relay authorization

### 6.Y.1	Introduction

This solution addresses Key Issue #1: Security for UE-to-UE Relay discovery and Key issue #3: Authorization in the UE-to-UE Relay Scenario. For Key Issue #1, this solution specifically addresses the security requirement: The 5G System shall provide a means to securely provision the security materials for UE-to-UE Relay discovery.

This solution assumes that the discovery security materials used for direct discovery and U2U relay discovery are different. Therefore, U2U relay authorization can be executed during the U2U relay discovery key provision process. Otherwise this solution does not address the Key Issue #3.

This solution needs to extend the existing specification as follows:

-	Permit multiple roles in the restricted discovery request. (Currently, there is only one role (Announce, Monitor, Query or Response) in the restricted discovery request.)

-	Permit to return more than two sets security materials in the restricted discovery response. (Currently, the restricted discovery response can return code-sending-security-parameter, code-receiving-security-parameter or both.

This solution assumes that only one ProSe Key Management Function (PKMF) is used to generate and provision discovery security materials for direct discovery and U2U relay discovery. The PKMF in this solution is also called 5G U2U Relay PKMF to distinguish it from other kinds of PKMFs. A 5G U2U Relay PKMF can provision various discovery security materials based on a 5G ProSe UE’s roles in U2U relay discovery procedures.

### 6.Y.2	Solution details

Figure 6.Y.2-1: Centralized discovery key management and U2U relay authorization procedure

NOTE:	This solution only covers the parameters related to security material retrieval.

0.	The 5G ProSe UE (Source UE/Target UE/Relay UE) gets the 5G U2U Relay PKMF address from the PCF/5G DDNMF of its HPLMN, which is responsible for provisioning discovery security materials related to direct discovery and U2U relay discovery.

Editor’s Note: Discovery request with PKMF instead of DDNMF or PCF is FFS (to be aligned with SA2 if needed).

1.	The 5G ProSe UE establishes a secure connection with the 5G U2U Relay PKMF and sends a U2U Relay Discovery Request to the 5G U2U Relay PKMF. The request includes User Info ID, Security capability, and UE's roles in RSC-specified relay service. The request should include RSC if discovery security materials for U2U relay discovery are required.

Security for the interface between the 5G ProSe UE and the 5G U2U Relay PKMF relies on Ua security if GBA specified in TS 33.220 [i1] is used (see clause 5.2.3.4) or Ua* security if AKMA specified in TS 33.535 [i2] is used (see clause 5.2.5.4).

Editor’s Note: Role representation is FFS and should be aligned with SA2.

2.	5G U2U Relay PKMF checks whether the 5G ProSe UE is authorized to play these roles in RSC-specified relay service.

3.	The 5G U2U Relay PKMF provides a set of discovery security materials corresponding to the authorized roles to the 5G ProSe UE through U2U Relay Discovery Response. The discovery security materials related to U2U relay discovery are associated with the RSC. The 5G U2U Relay PKMF may include relay discovery security policies specifying how the provided security material is used in the U2U relay discovery procedure in the response. For example, a security policy can specify whether end-to-end protection needs to be enabled in relay discovery messages.

### 6.Y.3	Evaluation

TBD

#### *** NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-33: "Study on System enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

[3]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[5]	3GPP TS 22.115: " Service aspects; Charging and billing".

[6]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[7]	3GPP TR 33.870: "Study of privacy of identifiers over radio access".

[8]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[9]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[i1]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[i2]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

#### *** END OF CHANGES ***