3GPP TSG-SA3 Meeting #108e Ad hoc	S3-222762r2

- e-meeting, 10 – 14 October 2022

- Source:	Qualcomm Incorporated

- Title:	An update on the evaluation of solution #4

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This contribution proposes an update on the evaluation of solution #4.

# 2	References

[1]	TR 33.809 v0.19.0

# 3	Rationale

This contribution proposes to add a text in the evaluation of solution #4 to address the EN. This contribution is based on the draft_S3-221371-r2 provided by Qualcomm in 107e-AdHoc meeting.

# 4	Detailed proposal

It is proposed to include the below pCR in the TR [1].

***** START OF CHANGES *****

## 6.4	Solution #4: Enriched measurement reports

### 6.4.1	Introduction

This solution addresses the first security requirement in the following key issue:

-	Key issue #3: network detection of false base stations (first requirement).

The solution provides a mechanism for enhancing the detection of false base stations by enriching the measurement reports from the UE. The solution is applicable to UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

### 6.4.2	Solution details

#### 6.4.2.1	Enrichment of measurement report

The UE measurement reports specified in 3GPP TR 38.331 [2] already contain several information relevant for the detection of false base stations, e.g., identifier and received-signal strength information of the cell. Additionally, 3GPP TS 38.331 provides support for reporting of Cell Group Info (CGI_info) which contains information broadcasted in MIB and SIB1.

In addition to the existing information, the system should support the following new information about camped and neighbouring cells to be included in the measurement report:

-	mib_info = hash of the MIB, which helps in detection of DoS attempts, e.g., cellBarred=barred;

-	sib_info = list of {SIB number, hash of the SIB}, which helps in detection of DoS, fraud, and subscription identification attempts, e.g., ims-EmergencySupport=false, tampered SI-SchedulingInfo, and useFullResumeID=true;

NOTE 1:	The sib_info could contain at least SIB1 which the UE currently obtains to generate CGI_info.

NOTE 2:	The mib_info is computed without including the SFN in MIB. It should contain its corresponding recorded time.

NOTE 3:	The sib_info could contain its corresponding recorded time.

NOTE 4:	The network can set the requesting frequency for CGI info, similar to the ANR frequency configuration. That is implementation specific.

It is suggested to reuse the algorithm of SHA256 to calculate the hashes of MIB/SIBs.

In addition to the information above which can be transmitted by UEs in CONNECTED mode, it is also possible to extend the logged measurements, currently discussed in TS 38.331 for Rel-16, to include the following information that can help in detecting false base stations:

-	reject_info = information about REJECTs that the UE had received earlier, which helps in detection of DoS attempts, e.g., presence of rogue REJECTs;

-	signal_info = information about signal as below, which helps in detection of DoS attempts, e.g., presence of erratic radio signals:

-	just power: signal is not associated with any normal pilots or reference signals.

-	power with just pilots: signal is associated with normal pilots or reference signals, but those signals do not provide any readable system information.

-	power with pilots and system info: signal is associated with normal pilots or reference signals and those signals provide system information, but the system information is wrong (e.g., inconsistent information, not possible to access the network according to the information).

The UE should send the above information to the network only after a successful AS security mode command procedure.

The network should verify and compare the above information with what is expected, e.g., comparing hashes of MIB/SIBs for reported cells with those of genuine cells using the same hash algorithm used in the UE.

To verify the hashes of MIB/SIBs reported by the UE in the MR, the gNB should store the hashes of all broadcasted MIB/SIBs of all cells that belong to the gNB and records the changed time when the MIB/SIBs are modified.

#### 6.4.2.2	Verification of the MIB/SIBs Hashes

When the serving cell receives a MR/logged MR with hashes of neighbouring cell MIB/SIBs, the serving cell can report the MR/logged MR to OAM, then the OAM communicates with the corresponding neighbouring cells to do the MIB/SIBs hashes verification; the serving cells also can communicate with the OAM to get the corresponding neighbouring cell's MIB/SIBs hashes, and verify the reported MIB/SIBs hashes in local. The communication between OAM and cells are implementation specific.

If verification fails, e.g., the hashes do not match, the network may consider it as a factor to detect the presence of the false base station.

During handovers, the serving cell may use the above information, i.e., the MIB/SIBs hashes verification result, to decide whether or not to attempt handover of the UE to the reported neighbouring cell.

### 6.4.3	Evaluation

This solution addresses FBS network detection of Key issue #3. It enhances FBS detection by enriching the measurement reports from UEs.

A) Potential impact to RAN radio interface and resource:

1. Regarding hash value reporting:

- Reporting hashes of system information is currently not supported. The logged MDT, mobility history information and CGI reporting procedures could be modified but that requires additional work.

- Regarding MIB/SIBs reading, the existing procedures already require UEs to read MIB and SIBs:

  - UEs in RRC_IDLE/INACTIVE state UEs read MIB/SIB1/other interested SIBs of camped cell. Calculating the hash of these MIB/SIBs is not expected to have a negative impact on power consumption. However, calculating the hash of additional SIBs (i.e. SIBs currently not acquired by the UE) will cause extra power consumption and is not preferred.

  - UE in RRC_CONNECTED state UEs, when configured with CGI reporting, read MIB and SIB1 of neighbour cells. The existing CGI reporting creates interruptions whenever the UE needs to tune in to another cell. Too frequent request of CGI reporting in CONNECTED will not only cause extra power consumption but will also create frequent interruptions. Those impacts become much worse if something else than MIB/SIB1 needs to be acquired. Thus, the scope of CGI reporting (i.e. acquired MIB/SIB1) is preferred not to be expanded and the procedure should not be triggered often.

2. Regarding reporting reject_info and signal_info

- The reporting RSRP/RSRQ/RSSI/beam level information of SSB or CSI-RS is supported since Rel-15. And reporting connection establishment failure and radio link failure is supported in Rel-16. If other information is required to be reported, additional work will be required.

B) Potential impact to RAN network interfaces:

- Performing an enriched measurement report does not have any direct impact to RAN network interfaces.

- There might be impacts to RAN network interface depending on the triggering mechanism, e.g. if triggered by CN or RAN nodes, NGAP and XnAP might be impacted. However, such impacts are expected to be straightforward and the overhead introduced can be implementation dependent (out of scope of 3GPP). No impact is foreseen in case the enriched measurements are triggered by the OAM.

If false base stations are mobile and portable, in particular, if the false base station attacks are discontinued in the concerned area at the time of detection, the network based false base detection may be limited to post-mortem diagnostics. In these cases, precise detection may require the network to collect information from many different UEs.

Reporting hash of SIBs can detect SIB modification or fake SIB injection attacks that would typically result in DoS. This solution does not address  DoS launched against UEs by other means (e.g., jamming),. Furthermore, mismatch of hash values is not sufficient for identifying the root cause of the issue, e.g., whether it is due to an attack or bit error in reading SIBs.

For SIB modification or fake SIB injection being successful against the UEs in the connected mode, if the attacker broadcasts those SIBs with higher power than those transmitted from the legitimate base stations, such attacks can be detected with the existing power measurement.

It is also possible that a malicious UE may report faked measurements, which may lead the network to falsely classify a legitimate base station as a false one, if this problem is not addressed by the network.

.

***** END OF CHANGES *****