3GPP TSG-SA3 Meeting #108e-AdHoc	draft_S3-222773-r3

- e-meeting, 10 - 14 October 2022 						 	  			merger of S3-222773, S3-222651, S3-222890

- Source:	Ericsson, Intel, Nokia, Nokia Shanghai Bell, ZTE, Xiaomi

- Title:	New Key Issue "Authentication for UE access to hosting network"

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

Approve the pCR to TR 33.858 below.

# 2	References

[1]	3GPP TR 33.858: "Study on security aspects of enhanced support of Non-Public Networks phase 2".

# 3	Rationale

This pCR is a resubmission of draft_S3-221982-r1, discussed at SA3#108-e in August 2022.

# 4	Detailed proposal

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[y]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

*** NEXT CHANGE ***

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in TR 23.700-08 [x] apply:

Local service, Localized service: Service, which is localized (i.e. provided at specific/limited area) and/or can be bounded in time. The service can be realized via applications (e.g. live or on-demand audio/video stream, electric game, IMS, etc), or connectivity (e.g. UE to UE, UE to Data Network, etc.).

Hosting network: A network providing access to Local/Localized services.

*** NEXT CHANGE ***

## 5.X	Key issue #X: Authentication for UE access to hosting network

### 5.X.1	Key issue details

The terms "localized service" and "hosting network" are explained in clause 3.1 of this document.

TR 23.700-08 [x] studies "Key Issue #3: Enabling NPN as hosting network for providing access to localized services" and "Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services". Key Issue #3 in TR 23.700-08 [x] states "NOTE 2: Collaboration with SA WG5 and SA WG3 may be needed." Key Issue #4 in TR 23.700-08 [x] states "NOTE 2: Security aspects (such as authentication of the UE […]) are addressed in SA WG3. The authentication architecture is addressed in both SA WG2 and SA WG3."

The intention of this key issue is to study authentication of  UE access to a hosting network, if existing security mechanisms can be reused or new security mechanisms are needed.

This is also related to service requirements for localized services in TS 22.261 [y]:

-	Clause 6.41.2.4: "The 5G system shall be able to authenticate and authorize the UE of a user authenticated to a hosting network to access the hosting network and its localized services on request of a service provider."

-	Clause 6.41.2.2: "The 5G system shall support secure mechanisms to allow a home network to coordinate with a hosting network for a subscriber to temporarily access the hosting network (e.g., based on temporary credentials) at a given time (start time and duration) and location."

It seems reasonable to assume that "secure mechanisms" includes UE-network mutual authentication. The UE-network authentication related aspects are in scope of this key issue.

### 5.X.2	Threats

If the UE is not authenticated towards the network, it is possible to impersonate the UE.

If the network is not authenticated towards the UE, it is possible to impersonate the network.

5.X.3	Potential security requirements

The UE and the hosting network shall support mutual authentication between the UE and the network.

*** END CHANGES ***