3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222819-r2

- e-meeting, 10 - 14 October 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Central authorization function for user consent

- Document for:	Approval, Information, Discussion

Agenda Item:	5.22

# 1	Decision/action requested

Add a key issue and solution on handling of authorization of user consent retrieval.

# 2	References

[1]	3GPP TR33.896

# 3	Rationale

Add a key issue and solution on handling of authorization of user consent retrieval.

# 4	Detailed proposal

***** START OF CHANGES

## 5.X	Key Issue #X: Unified framework for user consent related data retrieval, notification, and revocation

### 5.X.1	Key issue details

User consent is stored in the UDM/UDR. All NFs/AFs must retrieve the consent flag from the UDM and accordingly collect the data from a UE or UE related data from other NFs. However, not all NFs/AFs do contact UDM before collecting data.

For example, AF does not interact with UDM before collecting the data related to a UE. Even if AF interacts with UDM for the user consent, then AF collects data via AMF, therefore AMF also needs to coordinate with UDM before collecting data from a UE or UE related data from other NFs. This means, that the same consent checking is necessary at multiple places/NFs.

The SBA related framework provides for authorizing services. But there is no authorization in the 5GC which allows or disallows NFs/AFs to collect UE/user-related data. Furthermore, keeping track of revocation and which NF has received which UE-related user consent details seems to become tedious. Rel-17 TS 33.501 clause V.2 states that user consent revocation service is not provided by UDM.

This key issue looks into the benefits of a unified framework such as a central authorization function or a service for coordinating and keeping track of user consent retrieval, notification, and revocation.

### 5.X.2	Security threats

When user consent needs to get revoked, all NFs/AFs that have asked for user data beforehand need to revoke them. Otherwise, there is the danger that some NFs/AFs are not tracked and thus not informed about the revocation demand. This can result in accessing user-related data by NFs/AFs even after revocation by the user.

### 5.X.3	Potential security requirements

In addition to the requirements listed in TS 33.501 clauses V.1 and V.4:

The 5GS shall ensure that only authorized NFs/AFs can access user consent parameters.

The 5GS shall ensure that all NFs/AFs that collected user consent related data are notified.

***** NEXT CHANGE

>>> Solution deleted

***** END OF CHANGES