3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222823-r3

- e-meeting, 10 - 14 October 2022

- Source:	Ericsson

- Title:	A solution for UE authentication method negotiation

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve the pCR to TR 33.739.

# 2	References

[1]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2"

# 3	Rationale

To consider the case that there is more than one authentication method for the authentication of EEC/UE, the TR 33.739 [1] includes a key issue for authentication method negotiation. This contribution proposes a solution that re-uses existing negotiation mechanism for AKMA/GBA.

# 4	Detailed proposal

Approve the following changes to TR 33.739 [1].

*** Start of Change ***

## 6.X	Solution #X: Using existing AKMA/GBA negotiation mechanism

### 6.X.1	Solution overview

This solution addresses key issue #2.2 that focuses on authentication method negotiation for the case that there is more than one authentication method for the authentication of EEC/UE. This contribution proposes a solution that re-uses existing negotiation mechanism for AKMA/GBA.

### 6.X.2	Solution details

It is assumed that the ECS/EES is preconfigured with the information indicating which feature (AKMA or GBA) is supported by the HPLMN. The steps of the negotiation procedure are described below.

Editor’s Note: Preconfiguration of HPLMN authentication method support in the ECS/EES is FFS.

- The UE/EEC and ECS/EES establish TLS connection using TLS server certificate.

- The UE/EEC sends all possible PSK hints (“3GPP-AKMA”, “3GPP-bootstrapping”, “3GPP-bootstrapping-uicc”, “3GPP-bootstrapping-digest”) and the HPLMN identifier to the server (ECS/EES).

- The server indicates the PSK identity hints considering the support of the HPLMN i.e., “3GPP-AKMA” or “3GPP-bootstrapping”, “3GPP-bootstrapping-uicc”, “3GPP-bootstrapping-digest” in the HTTP response.

4.   Remaining steps in clause 5.3 in TS 33.222 [10] with the enhancements specified in Annex B.1.2 of TS 33.535 [8] are executed.

Editor’s Note: Using of HPLMN identifier is FFS.

Editor’s Note: It is FFS to exclude shared key-based mutual authentication between UE/EEC and ECS/EES.

Editor’s Note: It is FFS whether ECS/EES needs to check if it can obtain AKMA/GBA keys from network during the authentication mechanism selection procedure.

6.X.3	Solution evaluation

*** End of Change ***