3GPP TSG-SA3 Meeting #108e-AdHoc	draft_S3-222829-r1

- e-meeting, 10 - 14 October 2022

- Source:	Ericsson

- Title:	A new solution of building initial trust for NF certificate enrolment

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Approve the pCR to TR 33.876.

# 2	References

[1]	3GPP TR 33.876: "Study on automated certificate management in Service-Based Architecture (SBA)"

# 3	Rationale

Key Issue #2 in TR 33.876 [1] is about security protection of NF certificate enrolment.

This contribution proposes a new solution by introducing certificate management (CM) proxy functionality as a bridge and facilitate the initial trust establishment between NF and CA/RA without having impact on CA/RA.

# 4	Detailed proposal

Approve the following changes to TR 33.876 [1].

#### *** Start of Change 1 ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[4]	RFC 7515: "JSON Web Signature".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS) ".

[6]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3"

[X1]	3GPP TR 33.848: "Study on Security Impacts of Virtualisation"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

*** End of Change 1 ***

#### *** Start of Change 2 ***

## 6.X	Solution #X: Build initial trust for NF certificate enrolment

### 6.X.1	Introduction

This solution addresses Key Issue 2: Security protection of NF certificate enrolment.

As stated in the key issue details, before issuing a certificate, operator CA/RA needs to establish an initial trust with the requestor NF instance, ensuring that the requestor NF instance is the correct one and is entitled to request a certificate. This solution introduces certificate management (CM) proxy functionality to verify the NF identies, and facilitates the initial trust establishment between NF and CA/RA. Considering that the new functionality also acts as a bridge between CA domain and SBA domain, it can be implemented in the same entity called as Certificate Management Network Entity (CMNE) that is introduced in the Solution #7: A solution addressing the relation between certificate lifecycle management and NF lifecycle management.

To enhance the trustworthiness of the virtualized 3GPP NF, it is proposed to have optional procedures where CM proxy acting as Relying Party can verify VNF based on attestation result when attestation is in use. The details of attestation solution and procedure is further investigated in the FS_SIV study [X1] and out of scope of this solution.

It is also assumed that the CM proxy trusts the OAM and CA/RA, and there are many ways to establish the trust between them (for example SSH or TLS), which is out of scope of this solution.

### 6.X.2	Solution Details

The following description provides a high-level overview about a possible solution by introducing the certificate management (CM) proxy as a bridge and facilitate the initial trust establishment between NF and CA/RA without having impact on CA/RA. The description shows the steps by which an entity in the NF receives an end-entity certificate. The individual steps are only indicative.

NOTE: How to use attestation and trusted execution is out of scope of this solution.

Figure 6.X.2-1: High-level procedure to build initial trust for NF certificate enrolment

- NF has been successfully instantiated (with day-0 configuration).

The day-0 configuration can include the initial credential to be used by NF to establish a secure communication channel initiated from CM proxy.

For example, if TLS is used as a secure communication channel between NF and CM proxy, the NF can be preconfigured with a TLS server certificate which is signed by a root certificate that is stored in the CM proxy.

- (Optional when attestation is in use) To enhance the trustworthiness of the virtualized 3GPP NF, the VNF is attested. It is assumed that the attestation results are maintained by the Attestation Verifier for subsequent access [X1].

- CM proxy is provisioned through OAM procedure with necessary information.

The necessary information includes initial credential to be used by CM proxy to setup secure communication channel with NF. For example, if TLS is used as a secure communication channel between NF and CM proxy, the CM proxy can verify the TLS server certificate of the NF using a preconfigured root certificate in the CM proxy.

The necessary information also includes NF identities (e.g., FQDNs to be presented in the certificate).

- (Optional when attestation is in use) CM proxy acting as Relying Party can verify VNF based on attestation result and decide whether the NF is eligible for certification or not.

- CM proxy registers NF end-entity to CA/RA.

The identities of NF end-entity (got from previous steps) are registered to CA/RA.

- CA/RA provides authentication credentials to the CM proxy.

Depends on the agreement with the CA/RA, for example an initial authentication key (IAK) is agreed to be used in the enrollment protocol to get a certificate for the NF end-entity, the CA/RA sends IAK as the authentication credentials to the CM proxy.

- CM proxy provides the NF about enrolment information and authentication materials.

The enrolment information includes enrolment protocol, CA/RA details and registered NF identities in step 5.

The authentication materials sending from CM proxy need be protected. For example, if TLS is used as a transport layer protection between NF and CM proxy.

- NF generates key pair and prepares the CSR (Certificate Signing Request) message.

- NF sends the certificate enrolment request to the CA/RA. Authentication credential received in step 7 is used to to authenticate the origin of CSR from the end entity to the CA/RA.

The NF can authenticate the CA/RA based on out-of-band means. For example, CA/RA's root certificate is pre-configured as trust anchor in the NF, or it is installed during step 7.

- CA/RA returns the issued certificate to the NF.

CA/RA validates the certificate enrolment request based on local policies using the identities received in step 5.

### 6.X.3	Evaluation

TBD

*** End of Change 2 ***