3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222834-r1

- e-meeting, 10 - 14 October 2022

- Source:	Ericsson

- Title:	A solution for authentication of UE and GPSI verification by EES/ECS

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve the pCR to TR 33.739.

# 2	References

[1]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications"

[2]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2"

# 3	Rationale

The methods for authentication of EEC by the ECS/EES has been left as out of scope for Rel-17 in TS 33.558 [1]. It is required to define the authentication methods for Rel-18.

According to key issue #2.1 in TR 33.739 [2], EEC/UE authentication is required considering both the roaming and non-roaming cases. EEC authentication means verification of the EEC ID sent from the EEC to the ECS/EES while UE authentication means verification of the GPSI of the UE that hosts the EEC communicating with the ECS/EES. Since UE authentication can be enough and there may be no need to authenticate the EEC, this contribution proposes a solution for UE authentication by the ECS/EES.

# 4	Detailed proposal

Approve the following changes to TR 33.739 [2].

*** Start of Change ***

## 6.X	Solution #X: A solution for authentication of UE and GPSI verification by EES/ECS

### 6.X.1	Solution overview

This solution addresses the key issue #2.1 where it is stated that UE authentication is required considering both the roaming and non-roaming cases. UE authentication means verification of the GPSI of the UE that hosts the EEC communicating with the ECS/EES.

The solution proposes usage of AKMA for UE authentication by the ECS/EES. In addition to the AKMA usage for UE authentication, this solution uses AF Specific UE ID retrieval API as an optional solution to authenticate the UE, i.e., to verify the GPSI.

### 6.X.2	Solution details

The procedure flow of the solution is depicted in Figure 6.X.2-1 and the steps are explained below.

Figure 6.X.2-1: Authentication of EEC and UE by ECS/EES

Step 0. UE and HPLMN run primary authentication and derive KAKMA. UE learns the A-KID and KAKMA.

Step 1. KAF is derived as defined in AKMA procedure and revealed to the EEC.

Step 2. The EEC sends a session establishment request to the ECS/EES, including the parameter A-KID.

Step 3. The EES/ECS request KAF from the network. In this request, ECS/EES sends A-KID and ECS-ID/EES-ID. The ECS/EES can also send an indication for receiving the GPSI of the UE if the ECS/EES prefers to use AKMA for GPSI authentication. Otherwise, the ECS/EES can invoke the AF specific UE ID API for GPSI retrieval (steps 3.a and 3.b).

Editor’s Note: It is FFS whether steps 3.a and 3.b are necessary.

Step 4. The network executes the authentication and authorization for the ECS/EES as defined in the AKMA procedure and if the result is successful then derives the KAF such that KAF = KDF (KAKMA, ECS-ID/EES-ID).

Step 5. The network sends KAF, expire time for the key and and optionally GPSI to the ECS/EES.

Step 6. The EEC and the ECS/EES use KAF in Ua* protocol.

If the EEC sends GPSI to the ECS/EES, then the ECS/EES verifies the received GPSI using the GPSI received from the network via AKMA or via UE ID API.

### 6.X.3	Solution evaluation

*** End of Change ***