3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222862-r1

- e-meeting, 10 - 14 October 2022														    Revision of S3-22xxxx

- Source:	Xiaomi

- Title:	33.893: Solution on Token-based Authorization of the Role of the UE during Discovery

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed new solution to be added in TR 33.893.

# 2	References

[1]	TR 33.893	Study on Security Aspects of Ranging Based Services and Sidelink Positioning

[2]	S3-222861	33.893: Solution on Subscription-based Authorization of the Role of the UE during Discovery

# 3	Rationale

Based on the solution in S3-222861 [2], this pCR proposes an enhanced solution to address one of the requirements in Key Issue #2 in TR 33.893 [1] “The 5G Ranging/SL positioning system shall be able to support the authorization of the UE as a target UE/reference UE/assistant UE/Located UE in the Ranging/Sidelink Positioning service”.

# 4	Detailed proposal

*************** Start of the Change ****************

## 6.Y	Solution #Y: Token-based authorization of the role of the UE during Discovery

### 6.Y.1	Introduction

This solution addresses Key Issue #2 on Authorization for Ranging/SL Positioning service. Specifically, it addresses the third requirement in KI#2: “The 5G Ranging/SL positioning system shall be able to support the authorization of the UE as a target UE/reference UE/assistant UE/Located UE in the Ranging/Sidelink Positioning service”.

This solution is an enhancement of Solution #X. Authorization in Solution #X is performed during discovery interaction with the network by linking it with discovery security material generation and provisioning. But during discovery interaction between UEs over PC5, a misbehaving UE can still send a claimed role different from its authorized role to cheat the receiving UE, which is not able to verify whether or not the claimed role is the authorized role of the sending UE.

This solution introduces an authorization token (Auth-Token) which is generated and provisioned to the UE based on Solution #X. When broadcasting the discovery message, the sending UE includes the Auth-Token for its claimed role, which will be used by the receiving UE to authorize the claimed role by verifying the received Auth-Token.

### 6.Y.2	Solution details

During the discovery interaction with the network, an authorization token (Auth-Token) is generated for the UE to be used for the requested Ranging/SL positioning service if the role of the UE is authorized by the network. The Auth-Token could be generated by the authorization entity, i.e. the Ranging Server or the UDM. Or the Auth-Token could also be generated by the entity generating discovery security material, i.e. the 5GDDNMF/PKMF, based on the required information provided by the authorization entity. The Auth-Token is provisioned together with the discovery security material to the UE by the 5GDDNMF/PKMF.

The UE then sends the Auth-Token in the broadcast discovery message. Then the receiving UE verifies the received Auth-Token in addition to the verification of the received message. In this way, a misbehaving UE claiming an unauthorized role is not able to cheat the receiving UE, which will respond to the sending UE only after successfully authorizing the role based on the Auth-Token.

Provisioning of the Auth-Token during discovery procedure

Option A:	Auth-Token generated by the Ranging Server of the UDM

Steps #1~#5 refer to an Announcing UE (A-UE):

Steps #1~#2 are the same steps as in Solution #X.

3.	The Ranging Server or the UDM of A-UE determines whether A-UE is allowed to play the role(s) corresponding to its capabilities (e.g. whether A-UE is allowed to act as a Target/Server UE), and then generates an Auth-Token for A-UE based on the authorization related information in the subscription or the service agreement.

4.	The Ranging Server or the UDM returns to the 5G DDNMF/PKMF of A-UE an Authorization Response which contains the matched role(s) of A-UE and the generated Auth-Token.

5.	If the authorization with the Ranging Server or the UDM of A-UE is successful, the 5G DDNMF/PKMF of A-UE sends the Role(s) of A-UE and A-UE’s Auth-Token to the UE in the Discovery Response.

Figure 6.Y.2-a:	Auth-Token generated by the Ranging Server of the UDM

Steps #6~#15 refer to a Monitoring UE (M-UE):

Steps #6~#7 are the same steps as in Solution #X.

8.	The Ranging Server or the UDM of M-UE determines whether M-UE is allowed to play the role(s) corresponding to its capabilities (e.g. whether M-UE is allowed to act as a SL Reference/Located UE), and then generates an Auth-Token for M-UE based on the authorization related information in the subscription or the service agreement.

9.	The Ranging Server or the UDM returns to the 5G DDNMF/PKMF of M-UE an Authorization Response which contains the matched role(s) of M-UE and the generated Auth-Token.

Steps #10~#14 are the same steps as in Solution #X.

15.	The 5G DDNMF/PKMF of M-UE sends the Role(s) of M-UE and the M-UE’s Auth-Token received in step #9 to the UE in the Discovery Response.

Option B:	Auth-Token generated by the 5G DDNMF or PKMF

Steps #1~#6 refer to an Announcing UE (A-UE):

Steps #1~#2 are the same steps as steps #1~#2 in Option A.

3.	The Ranging Server or the UDM of A-UE determines whether A-UE is allowed to play the role(s) corresponding to its capabilities (e.g. whether A-UE is allowed to act as a Target/Server UE).

4.	The Ranging Server or the UDM returns to the 5G DDNMF/PKMF of A-UE an Authorization Response which contains the matched role(s) of the A-UE and the authorization related information.

5.	The 5G DDNMF/PKMF of A-UE generates an Auth-Token for A-UE based on the received authorization related information.

Step #6 is the same step as step #5 in Option A.

Figure 6.Y.2-b:	Auth-Token generated by the 5G DDNMF or PKMF

Steps #7~#17 refer to a Monitoring UE (M-UE):

Steps #7~#8 are the same steps as steps #6~#7 in Option A.

9.	The Ranging Server or the UDM of M-UE determines whether M-UE is allowed to play the role(s) corresponding to its capabilities (e.g. whether M-UE is allowed to act as a Reference/Located UE).

10.	The Ranging Server or the UDM of M-UE returns to the 5G DDNMF/PKMF an Authorization Response which contains the matched role(s) of the M-UE and the authorization related information.

Steps #11~#13 are the same steps as steps #10~#12 in Option A.

14.	The Ranging Server or the UDM returns an Authorization Response indicating whether the authorization is successful. The response may also include more authorization information on the association between A-UE and M-UE in the requested service.

15.	If the authorization with the Ranging Server is successful, the 5G DDNMF/PKMF of A-UE responds to the 5G DDNMF/PKMF of M-UE with a Monitor Response message including the authorization information on the association between A-UE and M-UE in the requested service.

16.	The 5G DDNMF/PKMF of M-UE generates an Auth-Token for M-UE based on the authorization related information received in steps #10 and #15.

Step #17 is the same step as the step #15 in Option A.

The use of the Auth-Token during discovery procedure

The Use of Auth-Token in Discovery Message over PC5 – Model A

1.	A-UE starts announcing. A-UE forms the announcement message and protects it with the discovery security material. The announcement message also contains the claimed role of A-UE and its Auth-Token provisioned by the network.

2.	M-UE listens for and receives an announcement message containing the Auth-Token of A-UE and verifies the message with the discovery security material.

3.	M-UE determines whether the claimed role pf A-UE in the announcement message is the role it monitors for. M-UE then further authorizes the claimed role of A-UE using the Auth-Token received from A-UE.

Figure 6.Y.2-c:	The Use of Auth-Token in Discovery Message over PC5 – Model A

The Use of Auth-Token in Discovery Message over PC5 – Model B

1.	The discoverer (R-UE) forms the discovery solicitation message and protects it with the discovery security material. The solicitation message also contains the claimed role of R-UE and its Auth-Token provisioned by the network.

2.	The discoveree (E-UE) listens for a discovery solicitation message and verifies the message with the discovery security material, after which the discoveree determines whether the claimed role of R-UE in the solicitation message is the role it monitors for. E-UE then further authorizes the claimed role of R-UE using the Auth-Token received from R-UE.

3.	The discoveree (E-UE) returns a discovery response message to the discoverer, which contains the claimed role of E-UE and its Auth-Token provisioned by the network.

4.	The discoverer (R-UE) verifies the discovery response message with the discovery security material, after which R-UE determines whether the role of E-UE in the response message is the role it solicits. R-UE then further authorizes the claimed role of E-UE using the Auth-Token received from the E-UE.

Figure 6.Y.2-d:	The Use of Auth-Token in Discovery Message over PC5 – Model B

Editor’s Note: The details of Auth-Token is FFS.

Editor’s Note: Whether 5G DDNMF/PKMF/Application Server are involved in Ranging/SL positioning discovery is FFS.

*************** End of the Change ****************