3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222891-r1

- e-meeting, 10 - 14 October 2022

- Source:	Xiaomi

- Title:	KI1, New Sol on Authentication mechanism for trusted non-3GPP Access in NPN scenarios

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the new solution for KI #1 of 3GPP TR 33.858 v0.1.0.

# 2	References

[1]	3GPP TR 33.858: "Study on security aspects of enhanced support of Non-Public Networks (NPN) phase 2"

# 3	Rationale

This pCR proposes a solution to address secure non-3GPP access issue of key issue #1 in 3GPP TR 33.858 [1].

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.A	Solution #A: Authentication mechanism for trusted non-3GPP Access in SNPN scenarios

### 6.A.1	Introduction

This solution addresses key issue #1.

Specifically, in SNPN scenarios, the UE may register for onboarding, therefore the UE needs to send new registration type to TNAN. Moreover, TNGF may need to leverage IDi to identify KTNGF, which can authenticate the identity of the UE. And IDi can be set as the SUCI/onboarding SUCI. However, in some cases, the UE may send anonymous value SUCI to TNGF in the registration request, making TNGF not able to locally link the identity of the UE with the corresponding KTNGF. Without the mapping between UE identity and KTNGF, TNGF cannot authenticate the identity of the UE.

To access to SNPN, on the one hand, the UE may provide the new identity (e.g. onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to TNAN. On the other hand, the UE may use anonymous value SUCI during the registration procedure, which results in the consequence that TNGF cannot locate KTNGF to authenticate the identity of UE.

This solution enables the UE to access SNPN via trusted non-3GPP access network.

### 6.A.2	Solution details

This solution reuses the authentication mechanism in clause 7.2.1 of TS 33.501 [X] with the following modifications.

- The UE may send SUCI/onboarding SUCI to the TNAP/TNGF. If the construction of SUCI as described in clause 6.12 of TS 33.501 [X] cannot be used and if the employed EAP method supports SUPI privacy, then the UE can send an anonymous value SUCI to TNAP/TNGF. If the UE indents to access SNPN, AN parameters should also include SNPN identifier, which consists of PLMN ID and NID.

- To support the onboarding procedure. Registration Type "SNPN Onboarding" indicates that the UE wants to perform SNPN Onboarding Registration (i.e. allowing the UE to access an ON-SNPN for the purpose of provisioning the UE with SO-SNPN credentials) should be included in the registration request. The AN parameters sent from the UE to TNAP/TNGF shall include Onboarding indication if the UE is accessing SNPN for Onboarding.

- The authentication mechanisms could be 5G AKA, EAP-AKA’, and any other key-generating EAP authentication method as described in clause I.2 of TS 33.501 [X].

- For the IKE_AUTH exchange part in step 13a of clause 7.2.1 of TS 33.501 [X], names in the ID payloads should correspond to the keys used to generate the AUTH payload. In case  the UE utilizes the anonymous value SUCI in step 5, the UE shall initiate an IKE_AUTH exchange and shall include the SUCI/onboarding SUCI in ID payloads rather than anonymous value SUCI. To help TNGF identify KTNGF, the AMF should send the newly generated KTNGF and the corresponding SUCI/onboarding SUCI to the TNGF. The AMF may obtain the SUCI/onboarding SUCI from the AUSF.

### NOTE: The UE can only provide SUCI/Onboarding SUCI in case it is configured with SUPI privacy parameters.

Editor’s Note: The usage of Onboarding SUCI in step 13 for a non 3gpp access is FFS.

6.A.3		System impact

TBD

### 6.A.4	Evaluation

TBD

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[X]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

*************** End of 2nd Change ****************