3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222892-r2

- e-meeting, 10 - 14 October 2022

- Source:	Xiaomi

- Title:	KI1, New Sol on Authentication mechanism for untrusted non-3GPP Access in NPN scenarios

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the new solution for KI #1 of 3GPP TR 33.858 v0.1.0.

# 2	References

[1]	3GPP TR 33.858: "Study on security aspects of enhanced support of Non-Public Networks (NPN) phase 2"

# 3	Rationale

This pCR proposes a solution to address secure non-3GPP access issue of key issue #1 in TR 33.858 [1].

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.A	Solution #A: Authentication mechanism for untrusted non-3GPP Access in SNPN scenarios

### 6.A.1	Introduction

This solution addresses key issue #1.

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

To access to SNPN via no-3GPP access, on the one hand, the UE may need to handle the new identity (e.g., onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to N3IWF. On the other hand, the UE may use anonymous value SUCI during the registration procedure, which will result in N3IWF failing to locate KN3IWF to authenticate the identity of UE.

This solution is proposed to address the aforementioned problem and enables the UE to access SNPN via an untrusted non-3GPP access network.

### 6.A.2	Solution details

This solution reuses the untrusted non-3GPP access authentication procedure in PLMN scenarios in clause 7.2.1 of TS 33.501 [x] with the following modifications:

- In SNPN scenarios, if the construction of SUCI as described in clause 6.12 of TS 33.501 [X] cannot be used and if the employed EAP method supports SUPI privacy, then the UE can send an anonymous value SUCI to N3IWF based on configuration. And SNPN identifier, which consists of PLMN ID and NID, should also be included in AN parameters, which are sent to the N3IWF. Moreover, to fulfill the onboarding requirements, the UE may also send onboarding SUCI to the N3IWF.

- The AMF can choose 5G AKA, EAP-AKA', or any other key-generating EAP authentication method to authenticate UE as described in clause 6.1.3 or clause I.2.2 of TS 33.501 [X].

- If EAP-AKA' or key-generating EAP authentication method is used for authentication as described in clause 6.1.3.1 and clause I.2.2 of TS 33.501 [X], the AUSF shall include the EAP-Success in step 7.

Editor’s Note: The need for including SUPI privacy case is FFS.

### 6.A.3		System impact

TBD

### 6.A.4	Evaluation

TBD

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[X]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

*************** End of 2nd Change ****************