3GPP TSG-SA3 Meeting #108e-AdHoc	S3-222906

- e-meeting, 10 - 14 October 2022

- Source:	NTT DOCOMO

- Title:	pCR to TR 33.884 new solution on UE authentication

- Document for:	Approval

Agenda Item:	5.11 study on SNAAPP security

# 1	Decision/action requested

It is proposed to approve the pCR below

# 2	References

[1]	3GPP TR 33.884 v0.1.0 Study on security of application enablement aspects for subscriber-aware northbound API access

# 3	Rationale

User / resource owner / UE authentication can be based on many different mechanisms, such as GBA, AKMA, username/password, certificates, and many more. In order to limit the impact of authentication mechanism selection to as few parties as possible, it is proposed to base authentication on openID connect. This allows the relying party to always deal with the same kind of identity token, regardless of the chosen authentication mechanism.

The authentication function can always be located in the home network.

This solution is primarily for UEs that are able to interact with the user through a browser.

# 4	Detailed proposal

It is proposed to approve the following pCR.

++++++++++++++++++++++++++++++++ Start of Changes ++++++++++++++++++++++++++++

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[xx]	openID.net: " OpenID Connect Core 1.0 incorporating errata set 1". Available at: https://openid.net/specs/openid-connect-core-1_0.html

+++++++++++++++++++++++++++++++++ Next Change ++++++++++++++++++++++++++++++

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANF	authentication function

AZF	authorization function

+++++++++++++++++++++++++++++++++ Next Change ++++++++++++++++++++++++++++++

## 6.Y	Solution #Y: Authentication using OpenID Connect

### 6.Y.1	Introduction

Authentication of the UE can be performed by including an OpenID Connect identity token. In OpenID Connect, the authorization server is usually also the OpenID provider. This solution separates these roles into the authorization function (AZF) and the authentication function (ANF). The authentication function (ANF) can then always be placed in the home network.

### 6.Y.2	Solution details

Figure 6.Y.2-1: OpenID Connect for SNAAPP AFs

The flow follows the authorization code flow of OpenID Connect [xx]. All communication is over TLS secured connections.

1.	Prior to any interaction, AZF should be registered with ANF, giving information that will be provided to the UE when requesting authentication.

2.	The UE accesses the AZF without identity token

3.	The AZF shall redirect the UE to the ANF_URI, with the type set to code (to indicate authorization code flow), scope set to openid (to indicate that this is an openID request), client_ID set to its own ID, redirect_URI set to the URL to be used in step 8, and may set state to some state that can be used by AZF to reduce its internal state.

4.	The UE shall access the ANF at the redirected to ANF_URI, including the parameters of step 3.

5.	The ANF shall authenticate the UE. This can be done in the usual way the home network authenticates the user, which may be GBA, AKMA, or using a proprietary method such as username/password.

6.	The ANF should request authorization to release the required personal information to the AZF from the user. In this setp, the ANF presents to the user the information that was made available to the ANF about the AZF in the enrollment.

7.	The ANF shall redirect the UE back to the AZF at the AZF_URI, including as code a single use token, which shall be specific to the AF, and as state the state that was set in step 3.

8.-9.	The AZF shall request the identity token from the ANF at its token endpoint. The ANF shall verify that the identity token is indeed being requested by the correct AZF, and, if correct, return the identity token to the AZF.

10.	The AZF shall verify the validity of the identity token according to RFC 6749, especially Sections 4.1.2 and 10.12.

Editor's Note: which identifiers are to be used is FFS.

### 6.Y.3	Evaluation

The ANF can always be placed in the home network, thus providing the user with a consistent appearance. The solution is not required if the ANF and AZF can be collocated in the home network of the UE.

Editor's Note: further evaluation is FFS

++++++++++++++++++++++++++++++++++ End of Changes +++++++++++++++++++++++++++++++++