3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222918r2

- e-meeting, 10 - 14 October 2022

- Source:	Ericsson

- Title:	Updates to the architectural assumptions clause

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is proposed to approve the additions to the proposed additions to the  TR 33.737.

# 2	References

[1]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[2]	3GPP TR 33.737: "Study on Authentication and Key Management for Applications (AKMA) phase 2".

[3]	3GPP S3-221581: "Discussion about the roaming architecture"

# 3	Rationale

The discussion paper [3] was endorsed in SA3#107Adhoc-e. The proposals are copied below for convenience.

It is proposed to agree the following in order for SA3 to converge to solutions:

1)	When a UE is roaming and trying to access accessing an internal HPLMN AF, the UE uses home-routed PDU session and the access is handled by the internal AF model in TS 33.535 [1], clause 6.2.

2)	When a UE is roaming and trying to access an external AF to either HPLMN or VPLMN, the access is handled by the external AF model in TS 33.535 [1], clause 6.3, in other words the external AF contacts the HPLMN AAnF via the HPLMN NEF.

3)	When a UE is roaming and trying to access an internal AF to the VPLMN, the access is handled by the external AF model in TS 33.535 [1], clause 6.3, in other words the VPLMN AF contacts the HPLMN AAnF via the HPLMN NEF.

These architectural assumptions are proposed to be recorded in the clause 4 of the TR 33.737 [2].

# 4	Detailed proposal

*** 1st CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.535: " Authentication and key management for applications based on 3GPP credential in 5G （AKMA）".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) ".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[5]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security ".

[X1]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

*** 2nd CHANGE ***

# 4	Architectural assumptions

## 4.1	General

The present document is based on the AKMA architecture and procedures specified in TS 33.535 [2]. This clause captures the architectural assumptions for the non-roaming and roaming models. These architectural models do not include the AKMA functions which enable LI in the VPLMN as these functions are the objects of the study.

## 4.2	AKMA Non-Roaming network model

The following figure shows the fundamental non-roaming network model of AKMA, as well as the interfaces between them. Details of the AKMA requirements and procedures are documented in TS 33.535 [2].

Figure 4.1-1: Fundamental Non-Roaming Network Model for AKMA

## 4.2	AKMA Roaming network model

When the UE is roaming there are a few cases to consider for the AF, whether the AF is internal to the HPLMN, internal to VPLMN or external in a DN e.g. hosted on the Internet.

Figure 4.2-1 shows the fundamental roaming network model for the case that the UE is roaming in the VPLMN and accesses an AF internal to the HPLMN. This network model is a simplified version of the Roaming 5G System architecture - home routed scenario from TS 23.501 [X1] since the UE accesses an AF internal to the HPLMN. It is therefore assumed that the UE is configured to setup a home routed PDU session for the Ua* protocol between itself and the AF. The HPLMN AF accesses the AAnF following the internal AF model from TS 33.535 [2].

Figure 4.2-1: Fundamental Roaming Network Model for AKMA when the AF is internal to the HPLMN

Figure 4.2-2 shows the fundamental roaming network model for the case that the UE is roaming in the VPLMN and accesses an AF internal to the VPLMN or external in the DN. This network model is a combination and simplification of the Roaming 5G System architecture - local breakout and home-routed scenarios from TS 23.501 [X1]. In the case that the UE accesses an internal AF to the VPLMN a local-breakout scenario is used between the UE and AF. In the case that a UE accesses an external AF which is in the DN, it does not matter if the UE uses the local breakout or the home-routed scenario. A VPLMN AF and an external AF accesses the AAnF via the HPLMN NEF following the external AF model from TS 33.535 [2]. In other words, the N32 interface is not used in the case of the AF internal to the VPLMN.

Figure 4.2-2: Fundamental Roaming Network Model for AKMA when the AF is internal to the VPLMN or AF is external

*** END OF CHANGES ***