3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222926-r2

- e-meeting, 10 - 14 October 2022

- Source:	Thales

- Title:	AKMA roaming with AF outside VPLMN

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

This contribution proposes a solution addressing Key Issue #1 of TR 33.737.

# 2	References

[1]	3GPP TR 33.737	v0.2.0

# 3	Rationale

This contribution proposes to address the Key Issue #1 "Support for AKMA roaming scenario" of TR 33.737 [1], especially the scenario when the UE is roaming in VPLMN and trying to access AF located outside VPLMN, i.e. this AF could be an internal HPLMN AF or external AF in Data Network.

The solution proposes to introduce an AAnF in the VPLMN, named vAAnF, to enable the VPLMN to decrypt user traffic encrypted between the roaming UE and AF outside the VPLMN thanks to AKMA-related credentials. The vAAnF receives from the HPLMN the AKMA Application Key KAF and associated parameters enabling the VPLMN to decrypt the user content exchanged between the roaming UE and AF located outside the VPLM. The solution also ensures that the Application Key KAF provided by the HPLMN is correct.

# 4	Detailed proposal

It is proposed that SA3 review and approve the following pseudo-CR to 3GPP TR 33.737 [1].

*** START of CHANGE ***

## 6.X	Solution #X: AKMA roaming with AF outside VPLMN

### 6.X.1	Introduction

This solution addresses Key Issue #1 "Support for AKMA roaming scenario", especially the scenario when the UE is roaming in VPLMN and trying to access AF located outside VPLMN, i.e. this AF could be an internal HPLMN AF or external AF in Data Network.

The solution proposes to introduce an AAnF in the VPLMN, named vAAnF, to enable the VPLMN to decrypt user traffic encrypted between the roaming UE and AF thanks to AKMA-related credentials. The vAAnF receives from the HPLMN AKMA Application key KAF and associated parameters enabling the VPLMN to decrypt the user content exchanged between the roaming UE and AF located outside the VPLM. The solution also ensures that the AKMA Application Key KAF provided by the HPLMN to VPLMN is correct.

### 6.X.2	Solution details

The solution addresses the scenario where AF is an internal HPLMN AF.

Figure 6.X.2-1: Sending of AKMA credentials to VPLMN

Pre-requisite: there is establishment of the key KAKMA between the UE and hAAnF.

Steps 1, 2, 3 take place as specified in TS 33.535 [2].

4. The hAAnF determine whether the UE is roaming. If the UE is roaming then the hAAnF provides the AKMA Application key KAF and associated parameters A-KID, AF_ID, SUPI, and KAF expiration time to the vAAnF.

Editor’s Note: the scenario needs to be studied where provided encryption keys are not related to KAF

Editor’s Note: how to determine if UE is roaming and how hAAnF find current vAAnF are FFS.

5. The vAAnF stores the key KAF and associated parameters received from the hAAnF and sends back response to the hAAnF.

Steps 6 and 7 take place as specified in TS 33.535 [2].

8. The UE sends a message to the AMF in order to check that the vAAnF stores the correct AKMA Application Key KAF to address LI requirement.This message contains the current GUTI value encrypted by the UE with KAF, and associated A-KID and AF-ID. The GUTI value encrypted with KAF is named GUTIe.

NOTE 1:	GUTIe could also be Message Authentication Code.

Editor’s Note: Whether a new NAS message needs to be defined is FFS.

9. The AMF sends to the vAANF: GUTIe, A-KID, AF-ID received from the UE in step 8, and GUTI value in clear text.

10. The vAANF retrieves the AKMA Application Key KAF thanks to A-KID, AF-ID and decrypts the GUTIe with KAF. Then, the vAAnF verifies that the decrypted GUTI is the same than the GUTI in clear text sent by the AMF.

11. The vAANF sends to the AMF the result of the KAF check.

12. The AMF forwards the result of the KAF check to the UE.

13. The UE can use KAF if the KAF check was successful.

This solution also covers the scenario where AF is an external AF in Data Network. The same steps apply.

### 6.X.3	Evaluation

TBD

*** END of CHANGE ***