3GPP TSG-SA3 Meeting #108-e 	S3-221818r1

- e-meeting, 22 - 26 August 2022

- Source:	Qualcomm Incorporated

- Title:	Proposed text for gNB-DU part of draft CR to TR 33.926

- Document for:	Approval

Agenda Item:	4.6

# 1	Decision/action requested

This contribution proposes gNB-DU related text for the draft CR to TR 33.926 for split gNBs.

# 2	References

[1]	S3-221814 Proposed skeleton for draft CR to TR 33.926 for split gNB

# 3	Rationale

This contribution contains proposed changes for the gNB-DU in the draft CR for TR 33.926. The justification for the changes is covered in S3-221813.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft CR to TR 33.926 for gNB-DU.

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.916: "Security Assurance Methodology for 3GPP network products classes".

[3]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[4]	3GPP TR 33.821: "Rationale and track of security decisions in Long Term Evolution (LTE) RAN/3GPP System Architecture Evolution (SAE)".

[5]	3GPP TS 33.116: "Security Assurance Specification for MME network product class".

[6]	3GPP TS 33.511: "5G Security Assurance Specification (SCAS); NR Node B (gNB)"

[7]	3GPP TS 38.300 v15: "NR; NR and NR-RAN Overall Description; Stage 2".

[8]	3GPP TS 23.501 v15: "System Architecture for 5G System; Stage 2".

[9]	3GPP TS 38.323 v15: "NR; Packet Data Convergence Protocol (PDCP) specification".

[10]	3GPP TS 38.322 v15:	"NR; Radio Link Control (RLC) protocol specification".

[11]	3GPP TS 33.250: "Security assurance specification for the PGW network product class".

[12]	3GPP TS 33.516: "5G Security Assurance Specification (SCAS) for the AUSF network product class".

[13]	3GPP TS 33.517: "5G Security Assurance Specification (SCAS) for the Security Edge Protection Proxy (SEPP) network product class".

[14]	3GPP TS 33.501 Release 15: "Security architecture and procedures for 5G system".

[15]	3GPP TS 33.518: "5G Security Assurance Specification (SCAS) for the Network Repository Function (NRF) network product class".

[16]	3GPP TS 33.519: "5G Security Assurance Specification (SCAS) for the Network Exposure Function (NEF) network product class".

[17]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[18]	3GPP TS 33.513: "5G Security Assurance Specification (SCAS); User Plane Function (UPF)".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN);Overall description;Stage 2."

[20]	3GPP TS 33.216: "Security Assurance Specification (SCAS) for the evolved Node B (eNB) network product class."

[21]	3GPP TS 33.514: "5G Security Assurance Specification (SCAS) for the Unified Data Management (UDM) network product class".

[22]	3GPP TS 33.512: "5G Security Assurance Specification (SCAS); Access and Mobility management Function (AMF)".

[23]	3GPP TS 33.521: "Security Assurance Specification (SCAS) for the Network Data Analytics Function (NWDAF) network product class".

[24]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services".

[25]	3GPP TS 33.226: "Security assurance for IP Multimedia Subsystem (IMS)".

[26]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

[27]	3GPP TS 33.522: "5G Security Assurance Specification (SCAS); Service Communication Proxy (SCP)".

[28]	3GPP TS 23.501: "System Architecture for 5G System; Stage 2" (Release 16).

[aa]	3GPP TS 38.300: "NR; NR and NR-RAN Overall Description; Stage 2" (Release 17).

[bb]	3GPP TS 38.401: "NG-RAN; Architecture description" (Release 17).

[cc]	3GPP TS 23.501: "System Architecture for 5G System; Stage 2" (Release 17).

[dd]	3GPP TS 33.742: "5G Security Assurance Specification (SCAS); Split gNB product classes".

**** NEXT CHANGE ****

- Annex Z: 
Aspects specific to the network product class gNB-DU

- Z.1	Network product class description for the gNB-DU

- Z.1.1	Introduction

The present document captures the network product class descriptions, threats and critical assets that have been identified in the course of the work on 3GPP security assurance specifications. The main body of the present document contains generic aspects that are believed to apply to more than one network product class, while Annexes cover the aspects specific to one network product class.

- Z.1.2	Minimum set of functions defining the gNB-DU network product class

As part of the gNB-DU network product, it is expected that the gNB-DU contains gNB-DU application, a set of running processes (typically more than one) executing the software package for the gNB-DU functions and OAM functions that are specific to the gNB-DU network product model. Functionalities specific to the gNB-DU network product introduce additional threats and/or critical assets as described below. Related security requirements and test cases have been captured in TS 33.742 [dd].

NOTE:	For the purposes of the present document, this common set is defined to be the list of gNB-DU functions contained in 3GPP TS 38.300 [aa],  3GPP TS 38.401 [bb] and TS 23.501 [cc].

- Z.2	Assets and threats specific to the gNB-DU

- Z.2.1	Critical assets

In addition to the critical assets of a GNP described in clause 5.2 of the present document, the critical assets specific to the gNB-DU to be protected are:

-	gNB-DU Application;

-	Mobility Management data: e.g. QoS and so on, etc;

-	User plane data;

-	The interfaces of gNB-DU whose data needs to be protected and which are within SCAS scope;

- 	F1 interface;

-	Console interface, for local access: local interface on gNB-DU; and

-	OAM interface, for remote access: interface between gNB-DU and OAM system; and

NOTE 1: 	The detailed interfaces of the gNB-DU class are described in clause 4, Network Product Class Description of the present document.

-	gNB-DU Software: binary code or executable code.

NOTE 2: 	gNB-DU files may be any file owned by a user (root user as well as non-root uses), including User account data and credentials, Log data, configuration data, OS files, gNB-DU application, Mobility Management data or gNB-DU Software.

- Z.2.2	Threats related to Control plane and User plane in the network

### Z.2.2.1	Control plane data confidentiality protection

- Threat name: gNB-DU control plane data confidentiality protection.

- Threat Category: Information Disclosure.

- Threat Description: If the gNB-DU does not provide confidentiality protection for control plane packets on the F1-C reference point, then the control plane packets sent over F1-C reference point can be compromised by attackers. This means the UE information exchanged can be accessed by the attackers leading to huge security breach. This threat scenario assumes that the F1-C reference points is not within the security environment.

- Threatened Asset: Mobility Management data.

Z.2.2.2	Control plane data integrity protection

- Threat name: Control plane data integrity protection.

- Threat Category: Tampering data, Denial of Service.

- Threat Description: If the gNB-DU does not provide integrity protection for control plane packets on F1-C reference point, the control plane packets sent over this reference point can be modified without detection. The intruder manipulations on control plane packets can lead to denial of service to legitimate users. This threat scenario assumes that the F1-C reference point is not within the security environment.

- Threatened Asset: Sufficient Processing Capacity, Mobility Management data.

Editor’s note: User plane threats are FFS.

**** END OF CHANGES ****