3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230090

# Electronic meeting, 16 - 20 January 2023

# Source:	Huawei, HiSilicon

# Title:	New solution to prevent S-NSSAI information leakage

# Document for:	Approval

Agenda Item:	5.12 FS_eNS_Ph3

# 1	Decision/action requested

Approve to include the PCR for the new solution in TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes a new solution to address the S-NSSAI leakage issue.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 5.Y	Solution #Y: Prevention of S-NSSAI information leakage

### 5.Y.1	Introduction

Two options are proposed: one being restriction of SoR-AF deployment while the other is to provide addtioanl protection to S-NSSAI sent to SoR-AF.

### 5.Y.2	Solution details

#### 5.Y.2.1	Option 1

SoR-AF is not allowed to be deployed outside the 3GPP operator domain. In this way, no other changes to the specification are needed (other than this requirement itself).

#### 5.Y.2.1	Option 2

If an SoR AF is allowed to be deployed outside the 3GPP operator domain, further protection from S-NSSAI leakage is needed when sending to the SoR AF.  In this case, it is proposed to store a mapping at the UDM, i.e. translate S-NSSAI to a slice identifier, i.e. external network slice identifier (ENSI). The ENSI instead of S-NSSAI is sent to SoR AF. Similarly, the UDM translates ENSI to S-NSSAI when receiving from the SoR AF.

The rest of the SoR procedures can be kept unchanged.

NOTE: the mapping is needed only for SoR AF deployed outside the 3GPP operator domain.

### 5.Y.3	Evaluation

Both options can address the key issue #X to prevent S-NSSAI information leakage.

The option 1 has minimum impact to the specifications (e.g. a new NOTE may be needed) but with a deployment restriction.

The option 2 requires an S-NSSAI mapping available at UDM.

***	END OF CHANGES	***